Bahir Dar University

Bahir Dar Institute of Technology

Faculty of Chemical and Food Engineering

Model Exit Exam Questions

Department: Chemical Engineering

Exam Year: 2026

| ### Course Name: Reaction Engineering I & II | ### Course Code: |
| --- | --- |

- What is the chemical reaction rate?

- Chemical reaction rate is a chemical property

- Chemical reaction rate is a physical property

- Chemical reaction rate is an intensive property

- Chemical reaction rate is an extensive property

- A catalyst

- Initiates a reaction.

- lowers the activation energy of reacting molecules.

- is capable of reacting with any one of the reactants.

- can not be recovered chemically unchanged at the end of a chemical reaction.

- 6 gm of carbon is burnt with an amount of air containing 18 gm oxygen. The product contains 16.5 gms CO2 and 2.8 gms CO besides other constituents. What is the degree of conversion on the basis of the disappearance of the limiting reactant?

- 85%

- 95%

- 75%

- 50%

- In which of the following reaction, the rate of reaction is a function of rate constant alone?

- Zero order reaction

- First-order reaction

- Second order reaction

- Third-order reaction

- The rate constant of a first-order reaction depends on the _____________

- Temperature

- Pressure

- Time

- Concentration

- The reaction between oxygen and organic material is a/an ______ reaction.

- Endothermic

- Exothermic

- Photochemical

- Biochemical

- Which of the following is not a chemical step in a fluid-solid catalytic reaction?

- Surface chemical reaction

- Adsorption

- Desorption

- None of these

- With increase in initial concentration, the fractional conversion of a first-order reaction in given time

- Remains constant

- Decreases

- Increases

- Unpredictable

- The order of a chemical reaction is

- Always equal to the total stoichiometric number of reactants

- An experimentally determined quantity

- Never fractional

- None of these

- Catalyst deactivation may be caused by

- Fouling

- Poisoning

- Sintering

- All

- The equilibrium constant of a catalytic chemical reaction __________ due to the presence of a catalyst.

- Remain unchanged

- Decrease

- Increase

- Can't be predicted

- A possible mechanism for the reaction, 2A + B  C + D, is:

| (1) |  | A + A A2 | fast, equilibrium |
| --- | --- | --- | --- |
| (2) |  | 2A + A A3 | slow |
| (3) |  | A3 + B A + C + D | fast |

According to the mechanism, the rate law will be:

- Rate = kCA2

- Rate = kCACB

- Rate = kCA3CB

- Rate = kCA3

- Arrhenius equation represents graphically the variation between the ____ and temperature.

- activation energy

- rate of reaction

- rate constant

- frequency factor

- For the reaction 2H2S(g) + O2(g)  2S(s) + 2H2O(l), which one of the following statements is absolutely true?

- The reaction is first order with respect to H2S and second order with respect to O2.

- The reaction is 3rd order overall.

- The rate law is: rate = kCH2SCO2.

- The rate law cannot be determined from the information given.

- When the concentration of reactant molecules increases, the rate of reaction increases. As the reactant concentration increases,

- The average kinetic energy of molecules increases.

- The frequency of molecular collisions increases.

- The rate constant increases.

- The activation energy increases.

- The half-life for a first-order reaction is 32 s. What was the original concentration if, after 2.0 minutes, the reactant concentration is 0.062 M?

- 0.84 M

- 0.09 M

- 0.10 M

- 0.07 M

- Which one of the following is false about Fluid-Fluid reaction?

A) Gas film resistance controls for highly soluble gases

B) If the resistance is in the main body, it needs a large volume of liquid

C) Bubble contactor is preferable for sparingly soluble gases

D) Packed columns are preferred when liquid film resistance controls

- The batch reactor has the disadvantage(s) of

- High labour and handling cost

- Poorer quality control of the product

- Considerable shutdown time to empty, clean out and refill

- All of the above

- Rate of a chemical reaction is not influenced by the

- Temperature

- Number of molecules of reactants taking part in a reaction

- Reactant concentration

- Catalyst

- N number of plug flow reactors in series with a total volume (V) gives the same

conversion as single

- PFR of volume V

- PFR of volume V/N

- CSTR of volume V

- PFR of volume N*V

- A first-order reaction requires two unequal-sized mixed-flow reactors, which of the

following gives higher yield?

- Smaller reactor followed by a larger one

- Large reactor followed by a smaller one

- Either of the arrangements A or B will give the same yield

- Data insufficient, can't be predicted\

- If the time required to change the concentration of reactant to half its original value is

independent of the initial concentration, the order of reaction is

- 0

- 1

- 2

- 3

- For a homogeneous reaction of nth order, the dimension of the rate constant is given by

- (concentration)1−n /time

- 1/(time)n

- (concentration)n−1 /time

- concentration /time

- Reaction rate equation for the reaction, below . If S is present in large excess, what is the order of this reaction?

- zero

- one

- two

- three

- For the irreversible elementary reactions in parallel with

the rate of disappearance of 'X' is equal to

- CX(K1+K2)

- CX(K1 + K2)/2

- CX*K1/2

- CX *K2/2

- Radioactive decay follows __________ order kinetics.

- zero

- first

- second

- third

- For a vapor phase catalytic reaction (A + B      P) which follows the Ridel mechanism and the reaction step is rate controlling, the rate of reaction is given by (reaction rate is irreversible, product also absorbs).

- In ideal mixed flow reactor the

- contents are well stirred

- Composition throughout the reactor remains same

- Exit stream has the same composition as the fluid within the reactor

- All of the above

- The rate of a gas phase reaction is given by KCACB. If the volume of the reaction vessel is reduced to l/4th of its initial volume, then the reaction rate compared to the original rate will be __________ times.

- 4

- 16

- 8

- 2

- Suppose doubling the concentration of a reactant increases the rate of reaction by a factor of 4, and tripling the concentration of the reactant increases the reaction rate by a factor of 9, the order of reaction is;

- 0

- 1

- 2

- 3

- Consider an isothermal PFR with a zero order reaction, A              R Suppose the inlet flow rate of A doubles. How does the rate change?

- Doubles

- Doesn’t change

- Decreases to half its value

- Triples

- If the conversion of a first-order liquid phase reaction occurring in a CSTR is 75%, molar feed rate is 5 mol/min, the rate of the reaction is 5 mol/litre.min then the volume of the reactor (in litre) is?

- 0.33

- 0.4

- 0.75

- 0.5

- If the residence time is 5 s, first order rate constant, k = 0.25 sec-1 and the number of tanks, N is 5, then the conversion is?

- 87.45%

- 33%

- 75%

- 67.2%

- Which of the following is NOT a part of reactor design?

- Flow rates of the reactant and product streams

- Method of operation

- Type of the reactor

- Size of the reactor

- The sequence in which three CSTR's of volumes 5, 10 and 15 m3 will be connected in series to obtain the maximum production in a second order irreversible reaction is

- 15, 10, 5

- 5, 10, 15

- 10, 5, 15

- 10, 15, 5

- The present CSTR is replaced by another CSTR of double the volume. A first-order liquid phase reaction occurs in both reactors. If the conversion achieved in the old reactor is 75%, what is the conversion achieved in the new reactor?

- 0.857

- 0.675

- 0.765

- 0.99

Course Name: Thermal unit operation                                    Course Code: ChEg3104

- In a double pipe counter current heat exchanger, the temperature profiles shown in the figure were observed. During operation, due to fouling inside the pipe, the heat transfer rate reduces to half of the original value. Assuming that the flow and the physical properties of the fluids, do not change, what is the LMTD (℃) in the new situation?

(A)	     10

(B) 20

(C)    40

(D) Indeterminate

- A heat exchanger is designed to heat liquid water from 150 ℃ to 190 ℃ inside the tube using steam condensing at 230 ℃ on the outer surface of the tube. For a constant flow rate, the effect of fouling of the heat transfer surface is to:

- Increase the temperature rise of the water

- Decrease the temperature rise of the water.

- Increase the heat exchanger effectiveness.

- Make no change in heat exchanger effectiveness.

- Two streams of fluids of unit constant specific heat and unit mass flow rate exchange thermal energy in an adiabatic heat exchanger. The inlet temperature of hot and cold fluid streams is 300 ℃ and 30 ℃ respectively. The effectiveness of the heat exchanger if the outlet temperature of both the fluid are equal is

(A)	     0.5

(B) 0.6

(C)    0.8

(D) 0.7

- The analysis of heat exchangers can be greatly simplify by making an assumptions, which are closely approximated in practice. Which one

- The outer surface of the heat exchanger is perfectly insulated

- The flow of non-condensable is considerable

- Axial heat conduction along the tube is assumed

- None steady-flow

- An oil with a flow rate of 1000 kg/h is to be cooled using water in a double-pipe counter-flow heat exchanger from a temperature of 70°C to 40°C. Water enters the exchanger at 25°C and leaves at 40°C. The specific heats of oil and water are 2 kJ kg-1 K-1 and 4.2 kJ kg-1 K-1, respectively. The overall heat transfer coefficient is 0.2 kW m-2 K-1. The minimum heat exchanger area (in m2) required for this operation is

(A)	     3.95

(B) 3.75

(C)    3.85

(D) 3.65

- Of the following tube layouts for exchangers, the one that will be accessible for external cleaning and cause a lower pressure drop when a fluid flow.

- Square pitch

- Triangular pitch

- Rotated square pitch

- Triangular pitch with cleaning lanes

- In a certain heat exchanger, cold fluid enters at 40℃ and rate of the flow is 1650 kg/hr. The hot fluid is at a temperature of 130℃ and cold fluid leaves the heat exchanger at 90℃. Specific heat of the cold fluid is 3.5 kJ/kg-K. If heat transfer area is 0.707 m2, then the overall heat transfer coefficient is

(A)	     1840 W/m2-K

(B) 184 W/m2-K

(C)    18.4 W/m2-K

(D) 2250 W/m2-K

- For a given heat duty, the size of the heat exchanger is a function of the following parameters except:

- Fluid flow rate

- Conductance ratio of the process fluids

- Thermal effectiveness

- Fouling factor

- Suppose you have a Double Pipe Heat Exchanger, with the inner tube of diameter 20mm (neglect thickness) and outer tube of diameter 30mm.  You have also two fluids A & B, you desire to have their flow rates as 15m3/s and 21m3/s respectively. Which fluid pass arrangement would be best suited for the heat exchanger?

- Parallel flow with A in inner pipe and B in the annulus

- Counter flow with B in inner pipe and A in the annulus

- Counter flow with A in inner pipe and B in the annulus

- Parallel flow with B in inner pipe and A in the annulus

Question 62 to 63: The following data were obtained from a heat exchange:

Tube side film coefficient (for water) = 180 Btu/h.ft2.℉

Shell side film coefficient (for steam) = 2010 Btu/h.ft2.℉

Fouling factor outside tube                     = 0.0005 h.ft2.℉/Btu

Fouling factor inside tube                     = 0.001 h.ft2.℉/Btu

Tube ID = 0.902’’      OD = 1’’ Thickness= 0.049’’ km=63 Btu (h.ft2.℉/ft.)

- Based on the outside surface, the overall heat transfer coefficient Uo [Btu/h.ft2.℉] will be:

- 361

- (B) 860

- (C)    600

- (D) 454

- If the tubes are cleaned on both sides to remove the scale, the clean over heat transfer coefficient Uo [Btu/h.ft2.℉] will be:

- 680

- 860

- 730

- (D) 800

- Two fluids, A and B exchange heat in a counter flow heat exchanger. Fluid A enters at 420℃ and has mass flow rate of 2 kg/s. fluid B enters at 20℃ and also has mass flow rate of 2 kg/s; effectiveness of heat exchanger is 75%. The exit temperature (℃) of fluid B will be:  (Specific heat of fluid A is 1 kJ/kg-K and that of fluid B is 4 kJ/kg-K).

(A)	     120

(B) 95

(C)    75

(D) 100

- When you want to have extremely small pressure drop for the shell side and also desire to provide support to the tubes, which one the following setup should you use?

- Segmental Baffles

- Doughnut Baffles

- Orifice Baffles

- Support Plates

- A decision must be made as to which fluid will flow through the tubes and which will flow through the shell. As a professional engineer, which fluid allocation you recommended

- The more seriously fouling fluid flows through the tube

- The corrosive fluid must flow on the shell side

- The stream with the lower heat transfer coefficient flows through the tube

- The more seriously fouling fluid flows on shell side.

- All of the following are true about the shell-side film coefficients except

- When the tube bundle employs baffles directing the shell-side fluid across the tubes from top to bottom or side to side, the heat-transfer coefficient is higher than for undisturbed flow along the axes of the tubes.

- The coefficients for triangular pitch are roughly 25 per cent greater than for square pitch.

- Increasing the baffle spacing cause increasing the film coefficients

- Decreasing the baffle spacing will cause the increment of the film coefficients

- Which one of the following dirt factor should we assign to the HE for two moderately clean streams?

- UD = 20, UC = 30

- UD = 40, UC = 50

- UD = 50, UC = 60

- UD = 100, UC = 110

- The effectiveness of a heat exchanger depends on

- The flow arrangement

- Number of transfer unit

- Capacity rate ratio

- All

- Which of the following phase of fluid need much higher heating surface area.

- Gas–Liquid phase

- Liquid –Liquid phase

- Gas–Gas phase

- Solid- liquid phase

- One of the following is not the requirement of a heat exchanger

- There should be two streams

- There should be a temperature difference between the two streams

- There should be no mixing

- There should be mixing

- Why are baffles used in shell-and-tube heat exchangers?

- To promote a better heat-transfer coefficient on the shell side and to support the tubes.

- To increase the heating surface of the heat exchanger.

- To promote a better heat-transfer coefficient on the tube side.

- To support the housing of the exchanger.

Fig 1: Heating, cooling and isothermal streamline flow of viscous fluid through the heat changer.

- Referring fig1, which velocity profile is matched correctly?

- 3                       Cooling

- 2                       Heating

- 1                        Isothermal

- 2                        Isothermal

- Of the following four feeding operation of multiple effect evaporator, which feed operation is suited for cold feed and to keep the product viscosity as low enough

- Forward feed

- Backward feed

- Mixed feed

- Parallel feed

- One of the following is not correct about the heat transfer in double pipe heat exchanger.

- Heat is transferred from the hot fluid to the wall by  convection

- Heat is transferred  through the wall by conduction

- Heat is transferred from the wall to the cold fluid by  convection

- Heat is transferred from the hot fluid to the cold fluid by  radiation

- Of the following, one is true about single effect evaporator

- The condensate product and the solution in the evaporator have the same composition and temperature.

- Increasing the evaporation rate by using a series of evaporators between the steam supply and condenser.

- Applicable for large capacity operations

- The latent heat, can be recovered and reused (effective steam utilization)

- A heat exchanger is used to cool hot water from 80°C other to 60°C by transferring heat to other stream of cold water enters the heat exchanger at 20°C and leave at 40°C. In which condition this heat exchanger operate?

(A)	 Parallel flow condition

(B) Countercurrent flow condition

(C) Cross flow condition

(D) Mixed flow condition

- Why we need evaporation

- Reduces deteriorative chemical reactions

- Recovery of solvent

- Reduce transportation and storage costs

- All

- Which one of the following cannot be determined by knowing fouling factors?

- When to clean the equipment

- When the equipment will stop to work

- Its value is zero for a new heat exchanger

- Its value is infinity for an old heat exchanger

- For countercurrent heat exchanger, inside and outside heat transfer coefficients hi and ho have been computed as 700 and 250 Btu/h.ft2.℉ respectively. By experience, it is known that a dirt resistance, Rdi=0.001 will deposit annually on the inside of the tube while Rdo = 0.0015 will deposit on the outside of the tube. The tube wall thickness is very small. The metal wall resistance can be neglected. The overall heat transfer coefficient [Btu/h.ft2.℉] on the basis which the heat transfer surface should be calculated so that the exchanger need be cleaned only once a year is nearer to

- 100

- (B) 124

- (C)    172

- (D) 150

- Which of the following methods can determine the fouling factor of a Heat Exchangers?

- A dirty Heat Exchanger

- A clean Heat Exchanger

- A dirty and a clean Heat Exchanger

- A dirty and a fully choked Heat Exchanger

- Which one of the following is not a type of feeding in an evaporator?

- Forward feed

- Backward feed

- Convoluted feed

- Mixed feed

- In the counter flow heat exchange, for the hot fluid the specific heat is 3 kJ/kg K, mass flow rate is 6 kg/s, inlet temperature is 150 °C and outlet temperature is 100 °C. For the cold fluid, the specific heat is 3 kJ/kg K, mass flow rate is 8 kg/s, inlet temperature is 20 °C. Neglecting the heat transfer to surrounding, the outlet temperature of the cold fluid in °C will be:

(A)	     7.5

(B)   57.5

(C) 46.6

(D) 60

Question 32 to 33: A double pipe countercurrents heat exchanger (Outside surface area = 200ft2) has reached its design capacity of 490,000 Btu/h and is to be shut down for cleaning. The data exchanger are as follows: 
                                                                                               Shell side                  tube sides

Temperature of hot fluid, ℉   in                  350

Temperature of hot fluid, ℉   out               450

Temperature of cold fluid, ℉   in                                                              300

Temperature of cold fluid, ℉   out                                                            310

Heat transfer coefficient, Btu/h.ft2.℉              38.4                                      300

Tubes 1’’ OD 18 BWG

- The dirty overall coefficient of heat transfer [Btu/h.ft2.℉] is nearer to

- 50

- 40

- 28

- 35

- If the metal wall resistance is negligible and the tube inside dirt factor is 0.003 h.ft2.℉/Btu

- 0.003

- 0.00265

- 0.0025

- 0.00285

- Of the following, one is not the component of evaporator

- Heat exchanger

- Vacuum

- Vapour separator

- Distillation unit

- The selection of type of evaporator best suited for a particular service is

- Characteristics of the feed and product.

- The pressure and temperature of the process

- Scale deposition and materials of construction

- All

- Forced circulation evaporators are usually more costly than natural circulation evaporators. However, the natural circulation evaporators are not suitable except.

- Highly viscous solutions due to low heat transfer coefficient

- Solution containing suspended particles

- For heat sensitive materials

- Low viscous solutions due to high heat transfer coefficient

- When single effect evaporator is used:

- When a cheap supply of steam is available

- When the required capacity is large

- When sufficient supply of steam is available

- Except alternative A

- Why do you use more than one type of feeding in an evaporator?

- To increase the feed efficiency

- To increase the efficiency of the steam used

- To decrease the steam consumption per unit mass of feed

- To increase the steam consumption per unit mass of feed

Question 91 to 92: A single effect evaporator is to concentrate 30,000 Ib/h of caustic solution from 10 to 50% concentration. The evaporator is to be supplied with 30 psia steam. The feed at 100℉. It operates at a vacuum 26’’ Hg. Some additional data are given below

Overall coefficient of heat transfer = 450 Btu/h.ft2.℉

Boiling point of caustic solution = 198℉

Specific heat of steam, Cp = 0.46 Btu/Ib.℉

- Cooling water required (gpm) is nearer to

(A)	     1200

(B)   1100

(C) 1500

(D) 1300

- Steam economy [Ib evaporated per Ib of steam] is nearer to

(A)	     1.0

(B)   0.5

(C) 0.837

(D) 0.75

- Which is the major means of heat transfer in a Double Pipe heat exchanger?

- Convection

- Conduction

- Radiation

- Combined Convection and Conduction

- In an operation where you want to heat a stream of liquid by Steam, you have the option to use extended fins. Then which of the following is best suited?

- Steam on the annular side with the fins on the cold liquid side

- Steam on the annular side with the fins on the steam side

- Steam on the inner side with the fins on the cold liquid side

- Steam on the inner side with the fins on the steam side

- When gas is used as a fluid in a double pipe heat exchanger, which one of the following is not true?

- The gas side has a low heat transfer coefficient

- Extended fins are used on the gas side to increase the Heat Transfer coefficient

- The gas side heat transfer coefficient is the highest

- Fins increase necessary heat transfer area

- Which one of the following is not a suitable application of evaporators?

- Refrigeration

- Cooling

- Heating

- Crystallization

- What is the driving force for evaporation to take place?

- Difference in partial pressure

- Difference in pressure

- Difference in Concentration

- Difference in temperature

- Which one of the following do we prefer to use for single effect over multiple effect evaporator?

- Low pressure operations

- High pressure operations

- Corrosive materials use

- High temperature operations

- What is the maximum baffle spacing for a Shell and Tube Heat Exchanger?

- ID of Shell

- ID of Tube

- ID of Shell/5

- ID of Shell/4

- Which one of the following is not an assumption in multiple effect evaporator’s calculations?

- There is no sub cooling of the condensate from different steam chests

- Heat transfer surface does not undergo fouling

- The entering steam is at their boiling point

- Pressure is same in every effect

- Which type of feeding is supposed to give maximum economy to a particular consumption?

- Forward feed

- Backward feed

- Parallel feed

- Mixed feed

- What is the reason for using decreasing pressure on every subsequent effects in a multiple effect evaporator?

- To be able to reuse the vapour from previous effect

- To decrease the vapour pressure

- To increase the boiling point

- To decrease the boiling point

- One of the following is not correct about correction factors for multi-pass arrangements

- Depends on geometry of the heat exchanger and the inlet and outlet temperatures of hot and cold fluid streams

- It is always less than unity for shell and tube heat exchanger

- It is the ratio of energy actually transferred to the maximum possible theoretical energy transfer.

- When a phase change is involved, as in condensation or boiling, the correction factor is always unity.

- What is the value of correction factor from the below F-curve chart where R=0.6, and P=0.45

(A)	     0.65

(B)   0.88

(C) 0.94

(D) 0.95

- Which of the following formula is correct for equivalent diameter for a triangular pitch?

- The increment of all of the following parameter leads to the decrement of the mass velocity of the shell side fluid except

- Clearance

- Tube pitch

- Baffle spacing

- Internal diameter of the shell

- Which of the following has the maximum heat transfer rate for double pipe heat exchanger?

- Counter flow

- Parallel flow

- Cross flow

- Split flow

- What should be done to increase the heat transfer of the tube side

- Decreasing the baffle space

- Increase number of tubes

- Increase baffle cut

- Increase tube outside diameter

- Of the following one is not suited to reduce the pressure drop of the tube side fluid

- Decrease number of tube passes

- Increase tube diameter

- Decrease tube length and increase shell diameter and number of tubes

- Use double or triple segmental baffles

- Suppose the length of a bundle is divided by six baffles, how many times all the fluid travels across the bundle.

- 6

- 7

- 8

- 9

- When designing a heat exchanger caloric temperature used for evaluating the physical properties of the fluid. However, the designer may be evaluating the physical properties of the fluid using arithmetic mean temperature in place of caloric temperature,  if all of the following conditions fulfilled except.

- Neither of the fluid is very viscous at the cold terminal (not  more than 1.0 centipoise)

- The temperature ranges ((T1-T2) or  (t2-t1)) do not exceed 50-100℉

- The temperature difference  (for counter current flow (T1- t2) and (T2-t1)) is less than 50℉

- Neither of the fluid is very viscous at the hot terminal (not  more than 1.0 centipoise)

- In a shell and tube heat exchanger, the baffle plates are used to:

  - Increase heat transfer by disturbing fluid flow

  - Reduce the pressure drop across the heat exchanger

C.  Prevent tube rupture

D. Increase the fluid velocity inside the shell

- The effectiveness of a heat exchanger is defined as:

A. The ratio of heat absorbed by the cold fluid to the heat transferred to the hot fluid

B. The ratio of heat exchanged to the maximum possible heat exchange

C. The ratio of the inlet temperature difference to the outlet temperature difference

D. The ratio of the heat transfer area to the tube length

- In the design of a heat exchanger, the choice of heat transfer area mainly depends on:

A. The temperature difference between the fluids

B. The specific heat of the fluids

C. The flow arrangement and heat duty

D. The pressure drop across the heat exchanger

- For an optimal heat exchanger design, the temperature approach is:

A.  The temperature difference between the hot and cold fluids at the outlet

B. The temperature difference between the hot and cold fluids at the inlet

C. The minimum allowable temperature difference between the hot and cold fluids at any point

D.  The average temperature difference between the hot and cold fluids

- The LMTD (Log Mean Temperature Difference) method is used for heat exchanger design when:

- The flow arrangement is parallel flow only

- The flow arrangement is countercurrent flow only

- The temperature difference between the hot and cold fluids is constant

- The flow arrangement involves both parallel and countercurrent flow

- In a heat exchanger, fouling is generally caused by:

- High fluid velocities

- Build-up of scale, sediment, or biological growth

- Low temperature differences

- High heat transfer rates

- In thermal unit operations, heat and mass transfer processes must obey thermodynamic laws. Which statement is MOST correct?

  - Heat transfer can occur without a temperature difference if mass transfer is present

  - First law governs the rate of heat transfer, while the second law governs direction

  - The second law determines the magnitude of heat transfer

  - Mass transfer eliminates entropy generation

- For heating a high-viscosity, fouling-prone liquid with steam in a chemical plant, the MOST appropriate heat exchanger is:

  - Plate heat exchanger

  - Double-pipe heat exchanger

  - Shell-and-tube heat exchanger

  - Air-cooled heat exchanger

- Which statement correctly compares the LMTD method and the ε–NTU method?

  - LMTD method is preferred when outlet temperatures are unknown

  - ε–NTU method requires known outlet temperatures

  - ε–NTU method is useful when outlet temperatures are unknown

  - Both methods give different heat duties for the same exchanger

- For the same inlet conditions and heat transfer area, which flow arrangement yields the highest effectiveness?

  - Parallel flow

  - Counterflow

  - Crossflow unmixed

  - Single-pass shell-and-tube

- In a double-pipe heat exchanger operating in counterflow, increasing the inner tube diameter while keeping mass flow rate constant will MOST likely:

  - Increase the heat transfer coefficient and pressure drop

  - Decrease the heat transfer coefficient and pressure drop

  - Increase the heat transfer coefficient but decrease the pressure drop

  - Decrease the heat transfer coefficient but increase the pressure drop

- Which design feature MOST directly increases the shell-side heat transfer coefficient in a shell-and-tube heat exchanger?

  - Increasing tube wall thickness

  - Increasing baffle spacing

  - Decreasing baffle spacing

  - Increasing shell diameter

- Film-wise condensation on a clean horizontal tube compared to dropwise condensation results in:

  - Higher heat transfer coefficient

  - Lower heat transfer coefficient

  - Same heat transfer coefficient

  - No pressure drops

- Which fouling effect MOST directly leads to increased operating cost in heat exchangers?

  - Increase in overall heat transfer coefficient

  - Reduction in pressure drop

  - Increase in thermal resistance and pumping power

  - Increase in fluid velocity

- The overall heat transfer coefficient U in a heat exchanger is MOST strongly affected by:

  - Only the wall thermal conductivity

  - Individual film resistances and fouling resistances

  - Fluid inlet temperatures only

  - Heat exchanger length

- In a thermal unit operation involving simultaneous heat and mass transfer (e.g., evaporation), which statement is MOST correct?

  - Heat transfer rate is independent of mass transfer

  - First law determines feasibility, second law determines energy balance

  - Heat transfer provides latent heat, while mass transfer governs phase change

  - Entropy generation is zero due to phase equilibrium

- A shell-and-tube heat exchanger is being designed where outlet temperatures are unknown, fouling is expected, and pressure drop is constrained. Which combination of design choices is MOST appropriate?

  - LMTD method with parallel flow

  - ε–NTU method with counterflow

  - LMTD method with crossflow

  - ε–NTU method with parallel flow

- In both double-pipe and shell-and-tube heat exchangers, decreasing hydraulic diameter on the fluid side generally results in:

  - Lower heat transfer coefficient and lower pressure drop

  - Higher heat transfer coefficient and higher pressure drop

  - Lower heat transfer coefficient and higher pressure drop

  - No effect on either heat transfer or pressure drop

- You are investigating the thermal conductivity of several materials: stainless steel, brass, aluminum, copper, mild steel, and fiberglass for use in a thermal management application. Your goal is to select the most suitable material for thermal insulation in a high-temperature system. Consider the following constraints:
A.Thermal conductivity should be minimized.
B. Material should withstand moderate mechanical loads.
C. Cost-effectiveness and availability are secondary but relevant.

- Which material would be the most appropriate choice?

  - Aluminum — low density, moderate conductivity

  - Stainless steel — moderate conductivity, corrosion resistant

  - Brass — moderate-high conductivity, corrosion resistant

  - Fiberglass — very low thermal conductivity, moderate mechanical strength

- In a double-pipe heat exchanger operating in counterflow, increasing the inner tube diameter while keeping mass flow rate constant will MOST likely:

  - Increase the heat transfer coefficient and pressure drop

  - Decrease heat transfer coefficient and pressure drop

  - Increase heat transfer coefficient but decrease pressure drop

  - Decrease heat transfer coefficient but increase pressure drop

- A thermally optimized but hydraulically poor design will MOST likely:

  - Minimize operating cost

  - Increase pumping energy consumption

  - Reduce pressure drop

  - Improve sustainability

- A heat exchanger initially operates with an overall heat transfer coefficient of 600 W/m²K, which drops to 350 W/m²K after several months. Which explanation BEST accounts for this change?

  - Reduction in the temperature difference between fluids

  - Increased fouling resistance on heat transfer surfaces

  - Increased fluid velocity inside the tubes

  - Reduced turbulence in the shell side

- An engineer increases the flow velocity in heat exchanger tubes to improve heat transfer. Which trade-off must be considered MOST carefully?

  - Increase in heat transfer area

  - Increase in pressure drop and pumping power

  - Decrease in Reynolds number

  - Decrease in overall heat transfer coefficient

- Which statement BEST explains why counterflow heat exchangers are more thermally efficient than parallel flow exchangers?

  - Counterflow reduces fouling formation

  - Counterflow maintains a higher average temperature driving force

  - Counterflow eliminates pressure drop

  - Counterflow reduces the heat exchanger size to zero

- A heat exchanger experiences increasing pressure drop and decreasing heat transfer performance. Which root cause is MOST probable?

  - Increased coolant flow rate

  - Formation of fouling deposits inside tubes

  - Reduced turbulence in the shell side

  - Increased heat transfer area

- The value of the fouling factor depends upon the:

  - Characteristic of process fluid.

  - Velocity of process fluid containing suspended solids.

  - Suspended solids in the fluid.

  - All

- A single-effect evaporator is replaced with a triple-effect evaporator. Which change is MOST likely?

  - Increased steam consumption

  - Increased steam economy

  - Reduced evaporation rate

  - Increased boiling point depression

- Dropwise condensation is rarely sustained in industrial condensers primarily because:

  - Condensation rate becomes too low

  - Surface contamination promotes film formation

  - The heat transfer coefficient becomes unstable

  - Vapor pressure decreases

- A chemical plant implements heat integration through a heat exchanger network. What is the primary objective?

  - Increase reactor temperature

  - Reduce external heating and cooling utilities

  - Increase pressure drop in pipelines

  - Reduce process flow rates

- The pinch point in heat integration represents:

  - Maximum pressure drops in heat exchangers

  - Minimum allowable temperature difference between hot and cold streams

  - Maximum steam consumption

  - Highest heat transfer coefficient

- Heat transfer in a heat exchanger improves significantly when the flow changes from laminar to turbulent because:

  - Viscosity decreases

  - Thermal conductivity increases

  - Mixing enhances heat transport

  - Pressure drop becomes negligible

- Reducing baffle spacing in a shell-and-tube heat exchanger MOST likely:

  - Decreases the shell-side heat transfer coefficient

  - Increases turbulence and pressure drop

  - Eliminates shell-side flow

  - Reduces heat transfer area

- In heat exchangers, the dominant thermal resistance is often associated with:

  - Tube wall material

  - Fluid boundary layers

  - Heat exchanger shell thickness

  - External insulation

- Effectiveness of a heat exchanger approaches unity when:

  - NTU is very large

  - Temperature difference is zero

  - Pressure drop is negligible

  - Flow velocity is extremely low

- Increasing tube length in a heat exchanger MOST likely results in:

  - Reduced heat transfer area

  - Increased pressure drops

  - Reduced turbulence

  - Lower heat transfer coefficient

- Recovering heat from hot process streams primarily improves plant performance by:

  - Increasing product purity

  - Reducing external energy requirements

  - Increasing fluid viscosity

  - Decreasing Reynolds number

- Poor insulation in thermal systems primarily results in:

  - Reduced pressure drops

  - Increased heat loss to the environment

  - Increased condensation rate

  - Increased boiling temperature

- Increasing cooling water flow in a condenser MOST likely:

  - Reduces heat transfer rate

  - Increases condensation rate

  - Eliminates vapor condensation

  - Reduces pressure drop

- Which strategy MOST effectively improves energy efficiency in chemical plants?

  - Increasing reactor pressure

  - Implementing waste heat recovery systems

  - Increasing cooling water usage

  - Increasing heat exchanger size indefinitely

- In fluid transport systems, pumping power is MOST sensitive to:

  - Pressure drops

  - Fluid density

  - Temperature difference

  - Heat transfer coefficient

- Thermal stress in heat exchangers is MOST likely caused by:

  - Uniform temperature distribution

  - Large temperature gradients

  - Low heat transfer coefficients

  - Low operating pressure

- The key goal of heat exchanger network design is to:

  - Maximize pressure drop

  - Minimize energy consumption

  - Increase reactor temperature

  - Eliminate heat exchangers

- Forced circulation evaporators are preferred when:

  - Solutions are highly viscous or prone to fouling

  - Solutions have low boiling point

  - Pressure is very low

  - Vapor condensation rate is high

- An evaporator is operated under vacuum conditions instead of atmospheric pressure. What is the PRIMARY advantage of this operation?

- Increased boiling temperature of the solution

- Reduced boiling temperature of the solution

- Increased steam consumption

- Increased pressure drops

- In a heat exchanger where one fluid has a very low heat transfer coefficient, the overall heat transfer performance is MOST likely limited by:

  - The higher heat transfer coefficient fluid

  - The lower heat transfer coefficient fluid

  - The metal wall conductivity

  - The external insulation

- If the cooling water entering a condenser becomes warmer than usual, which outcome is MOST likely?

  - Increased condensation rate

  - Reduced condensation efficiency

  - Increased vapor pressure inside the condenser

  - Both B and C

- During process heat integration, a hot stream is cooled below the pinch temperature. Which consequence is MOST likely?

  - Improved energy efficiency

  - Violation of pinch design rules and increased energy consumption

  - Reduced heat exchanger area

  - Elimination of heat recovery opportunities

- In a drying operation, the drying rate suddenly drops after some time despite constant air temperature and velocity. Which stage of drying is occurring?

  - Constant rate drying period

  - Falling rate drying period

  - Initial warming period

  - Saturation drying period

- During distillation of a binary mixture, increasing the reflux ratio will primarily result in:

  - Lower separation efficiency

  - Higher number of theoretical stages required

  - Better separation but higher energy consumption

  - Decreased condenser duty

- A counter-current heat exchanger generally performs better than a parallel-flow heat exchanger because:

  - Temperature difference remains larger along the exchanger length

  - The heat transfer area is smaller

  - Fluid velocity is higher

  - Thermal conductivity increases

- In an evaporation process, if the boiling point of the solution increases during concentration, the phenomenon is known as:

  - Boiling point depression

  - Boiling point elevation

  - Latent heat reduction

  - Vapor pressure suppression

- A hot process stream leaving a reactor at 250°C is currently cooled using cooling water. Which modification would MOST effectively improve energy efficiency?

  - Increasing cooling water flow rate

  - Installing a heat exchanger to preheat the feed stream

  - Lowering reactor operating temperature

  - Increasing insulation thickness on the reactor

- Which of the following condensation modes provides the highest heat transfer coefficient?

  - Film condensation

  - Dropwise condensation

  - Laminar condensation

  - Natural convection condensation

- Which maintenance strategy is MOST effective in minimizing unexpected heat exchanger failure?

  - Reactive maintenance after breakdown

  - Periodic predictive monitoring of performance parameters

  - Reducing operating temperature

  - Eliminating preventive cleaning

- Operating thermal equipment at very high temperatures requires careful material selection primarily because:

  - High temperatures increase the heat transfer coefficient

  - Thermal expansion and material degradation may occur

  - Fluid viscosity increases significantly

  - Heat transfer area decreases

- In high-pressure thermal equipment, pressure relief valves are installed mainly to:

  - Improve heat transfer efficiency

  - Prevent catastrophic failure due to overpressure

  - Increase turbulence in fluid flow

  - Reduce pumping power requirements

- An energy audit reveals unusually high steam consumption. Which of the following issues is MOST likely responsible?

  - Excessive heat exchanger fouling

  - High thermal conductivity of materials

  - Increased heat transfer coefficient

  - High turbulence in process streams

- Reducing operating costs in thermal systems is most effectively achieved by:

  - Increasing heat exchanger size indefinitely

  - Balancing capital investment with operating energy costs

  - Operating at the highest possible temperatures

  - Minimizing heat transfer area

- Which engineering approach MOST effectively reduces carbon emissions in thermal systems?

  - Increasing fossil fuel usage

  - Improving heat recovery and energy efficiency

  - Increasing pressure drop in heat exchangers

  - Increasing steam generation

- A plant experiences decreasing heat exchanger efficiency, increasing pressure drop, and rising energy costs. Which of the following is the MOST probable root cause?

  - Fouling and scaling inside heat exchanger tubes

  - Excessive insulation on pipes

  - Reduced heat transfer area

  - Increased coolant temperature

- The shell-side pressure drops of a heat exchanger doubles during operation. Which situation MOST likely causes this behavior?

  - Increased baffle spacing

  - Decreased fluid density

  - Fouling deposits narrowing flow passages

  - Improved turbulence due to higher Reynolds number

- Multiple-effect evaporators improve energy efficiency primarily because they:

  - Increase the boiling point of the solution

  - Reuse vapor from one effect as heating steam in another effect

  - Increase heat transfer area

  - Reduce the operating pressure of the evaporator

- Increasing fluid velocity in a heat exchanger generally results in:

  - Lower heat transfer coefficient and lower pressure drop

  - Higher heat transfer coefficient but higher pressure drops

  - Lower heat transfer coefficient and higher pressure drop

  - No change in heat transfer performance

- Scaling occurs in evaporators when:

  - Volatile components condense on heat transfer surfaces

  - Dissolved salts precipitate and deposit on heated surfaces

  - Vapor pressure decreases due to vacuum conditions

  - Turbulence increases in the heating zone

- When gas is used as a fluid in a double pipe heat exchanger, which one of the following is not true?

- Fins increase necessary heat transfer area

- The gas side heat transfer coefficient is the highest

- Extended fins are used on the gas side to increase the Heat Transfer coefficient

- The gas side has a low heat transfer coefficient

- Which of the following phase of fluid need much higher heating surface area.

- Gas–Liquid phase

- Liquid –Liquid phase

- Gas–Gas phase

- Solid- liquid phase

- Which one of the following do we prefer to use for a single-effect over a multiple-effect evaporator?

- Low-pressure operations

- High-pressure operations

- Corrosive materials use

- High temperature operations

- One of the following is not correct about the heat transfer in double pipe heat exchanger.

    - Heat is transferred from the hot fluid to the wall by convection

    - Heat is transferred from the wall to the cold fluid by convection

    - Heat is transferred from the hot fluid to the cold fluid by radiation

    - Heat is transferred through the wall by conduction

- Which of the following is not a typical type of heat exchanger?

  - Shell and tube heat exchanger

  - Plate heat exchanger

  - Air-cooled heat exchanger

  - Flash drum heat exchanger

- The effect of increasing the flow rate of the fluid on the heat transfer rate in a heat exchanger is:

  - Increases the heat transfer rate and decreases the temperature difference

  - Decreases the heat transfer rate and increases the temperature difference

  - Increases the heat transfer rate and increases the temperature difference

  - Decreases the heat transfer rate and decreases the temperature difference

- In a heat exchanger, the "U" value (overall heat transfer coefficient) depends on:

  - The thermal conductivity of the fluid

  - The fouling factor

  - The heat transfer area

  - All of the above

- The primary function of a heat exchanger in a chemical process is to:

  - Transfer mass between two streams

  - Transfer heat between two fluids

  - Remove particulate matter from the fluid

  - Control fluid pressure

- The efficiency of a heat exchanger can be improved by:

- Decreasing the heat transfer surface area

- Increasing the flow resistance

- Reducing thermal conductivity

- Increasing the heat transfer coefficient

- Which process utilizes the concept of latent heat?

  - Drying of solids

  - Heat exchanger operation

  - Boiling of a liquid

  - Thermal insulation

- The Prandtl number in heat transfer is the ratio of:

  - Momentum diffusivity to thermal diffusivity

  - Thermal diffusivity to momentum diffusivity

  - Kinematic viscosity to density

  - Density to specific heat

- The Nusselt number is a dimensionless parameter used to describe:

  - Heat conduction in solids

  - Convective heat transfer

  - Heat loss through radiation

  - The efficiency of a distillation column

- Which of the following materials is commonly used for insulation in thermal systems?

  - Copper

  - Fiberglass

  - Steel

  - Aluminum

- What does the term "latent heat" refer to?

  - Heat required to raise the temperature of a substance

  - Heat involved in a phase change

  - Heat lost due to radiation

  - Heat due to chemical reactions

- In distillation, the driving force for mass transfer is:

  - Concentration gradient

  - Pressure gradient

  - Temperature gradient

  - Thermal conductivity

- A countercurrent heat exchanger has:

  - Both fluids flowing in opposite directions

  - Both fluids flowing in the same direction

  - One fluid stagnant

  - None of the above

- Which equipment is commonly used for thermal evaporation?

  - Rotary kiln

  - Falling-film evaporator

  - Fluidized bed

  - Packed column

- Which of the following is NOT a mode of heat transfer?

  - Conduction

  - Radiation

  - Diffusion

  - Convection

Course Name: Chemical Engineering Apparatus Design

- Which of the following is the mechanism to estimate the physical properties data for designing apparatus?

- by using extrapolation of experimental values

- by using interpolation of experimental values

- by using correlation

- All

- An acceptable rate of attack will depend on:

- the cost of the material

- the duty

- all

- None

- During separation column design, the Design Requirement is not:

- smallest column with the largest pressure drop

- the highest-performance internals.

- acceptable plate spacing

- All

- Which one the following is not true from the given alternatives

  - Plate columns are normally suitable for fouling liquids or laden with solids.

  - Tray Column is well suited for vacuum services

  - Foaming liquids can be handled more readily with Packed column

  - Packed towers are more suitable for corrosive services

  - All

- Which one is not the constraints on tray performance (vapor flow conditions on tray design)

  - Foaming

  - Raining

  - Flooding

  - Dumping

  - None

- The Selection of a suitable material must take into account:

  - the suitability of the material for fabrication (particularly welding)

  - the compatibility of the material with the process environment.

  - the material that gives the lowest cost over the working life of the plant.

  - All

- Which one of the following is not true?

  - The column diameter depends on the vapour flow-rate.

  - The flooding condition fixes the upper limit of vapour velocity.

  - Vapour velocity at the weep point is the maximum design vapour velocity

  - To reduce pressure, drop in the plate, use low weir height

- Which one the following is not true from the given alternatives

  - The tensile strength of metals decreases with increasing temperature.

  - The elastic modulus of metals increases with increasing temperature

  - If equipment is being designed to operate at high temperatures, materials that retain their strength must be selected

  - An increase in temperature usually results in an increased rate of corrosion

- The life of the material in service can not be predicted from experimentally determined

  - corrosion rates

  - penetration rate

  - Erosion Rates

  - Creep Behavior

  - All

  - None

- An acceptable rate of attack will depend on:

  - the cost of the material

  - the duty

  - particularly as regards to safety

  - the economic life of the plant

  - All

- Which of the following is/are the factor that affect the selection of material for Corrosion Resistance?

  - Temperature

  - pH

  - heat transfer rate

  - steam velocity and agitation

  - All

  - None

- ------------------ is a dimensionless designator of pipe size.

  - Nominal pipe size

  - Diameter

  - Thickness

  - None

- Which one of the following is the component of piping System from the given alternatives?

  - Fittings

  - Flanges

  - Valves

  - All

  - None

- -------------------- is not an actual fitting but it is describing how the branch connection is created

  - Unions

  - stub-in

  - Cap

  - Tee

- How to control the rate, the volume, the pressure and the direction of a fluid within a pipe?

  - By using valves

  - By using pipe

  - By using support

  - By using elbow

- ------------- is selected to resist the internal pressure, with an allowance for corrosion.

    - The pipe wall thickness

    - The valve

    - The pipe

    - The reinforcement

- During pipe sizing, Loads will not arise from:

    - Vibration

    - The weight of the pipes, their contents, insulation and any ancillary equipment.

    - The reaction to the fluid pressure drops.

    - Loads imposed by the operation of ancillary equipment, such as relief valves.

    - None

- Economic pipe diameter means

    - the one which the highest capital cost and highest pumping costs of a pipe

    - the one which the lowest capital cost and highest pumping costs of a pipe

    - the one which gives the lowest annual operating cost.

    - All

    - None

- The most dominant factor for optimum pipe diameter to give the minimum cost is

    - mass flow rate

    - temperature

    - pressure

    - All

    - None

- The main purpose of a separation column in chemical engineering is to:

  - Remove solids from a liquid

  - Separate components of a mixture based on differences in volatility

  - Increase the pressure of a fluid

  - Mix two immiscible liquids

- In a distillation column, the separation efficiency can be enhanced by:

  - Decreasing the number of stages

  - Increasing the reflux ratio

  - Reducing the column diameter

  - Increasing the temperature of the reboiler

- The height of a distillation column is determined by:

        - The required number of theoretical stages

        - The column diameter

        - The pressure of the system

        - The type of packing used

- The flooding condition in a distillation column refers to:

        - The condition when the liquid rate exceeds the capacity of the column

        - The condition when the vapor velocity exceeds the design capacity, causing liquid to accumulate

        - The condition when the temperature inside the column exceeds the boiling point

        - The condition when the separation between components is negligible

- Which of the following factors affects the tray efficiency in a distillation column?

  - Vapor flow rate

  - Tray design

  - Liquid flow rate

  - All of the above

- The optimum column diameter for a separation column is primarily influenced by:

  - The desired separation efficiency

  - The vapor and liquid flow rates

  - The type of feed mixture

  - The type of packing material used

- In a separation column, the entrainment refers to:

  - The entrainment of liquid in the vapor phase

  - The separation of solids from the liquid phase

  - The change in the boiling point of a mixture

  - The condensation of vapor in the column

- The plate spacing in a distillation column is typically determined by:

  - The type of packing used

  - The pressure of the system

  - The vapor flow rate and required vapor-liquid contact time

  - The column diameter

- Pipe supports in a piping system are used to:

  - Increase fluid flow

  - Hold the pipes in place and reduce stress on the system

  - Regulate the temperature of the fluid

  - Direct the flow of the fluid

- The allowable stress in a piping system is determined by:

  - The temperature of the fluid being transported

  - The material strength and safety factors

  - The pipe's internal pressure only

  - The size and weight of the pipe

### Course Name: Chemical Engineering Thermodynamics II

- Which of the following is NOT a state function?

- Internal Energy (U)

- Enthalpy (H)

- Work (W)

- Entropy (S)

- The First Law of Thermodynamics is based on the principle of:

- Conservation of mass

- Conservation of energy

- Conservation of momentum

- Conservation of charge

- The equation for the First Law of Thermodynamics for a closed system is:

- ΔU = Q + W

- ΔU = Q - W

- ΔU = W - Q

- ΔU = Q × W

- Which of the following processes is reversible?

- Free expansion of a gas

- Isothermal compression of an ideal gas

- Mixing of two gases

- Dissipation of energy due to friction

- The Second Law of Thermodynamics states that:

- Energy cannot be created or destroyed

- The entropy of an isolated system always increases

- The enthalpy of a system is constant

- The internal energy of a system is always zero

- The Gibbs free energy (G) is defined as:

- G = H - TS

- G = U + PV

- G = H + TS

- G = U - TS

- For an ideal gas, the relationship between pressure (P), volume (V), and temperature (T) is given by:

- PV = nRT

- PT = nRV

- VT = nRP

- PV = RT

- The compressibility factor (Z) for an ideal gas is:

- Z = 0

- Z = 1

- Z = ∞

- Z = PV

- The Maxwell relations are derived from:

- The First Law of Thermodynamics

- The Second Law of Thermodynamics

- The fundamental thermodynamic equations

- The ideal gas law

- The Clausius-Clapeyron equation is used to describe:

- Phase equilibrium between liquid and vapor

- Heat capacity of solids

- Entropy change in an ideal gas

- Enthalpy change in a chemical reaction

- A gas undergoes an isothermal expansion. What happens to its internal energy?

- Increases

- Decreases

- Remains constant

- Becomes zero

- In a Carnot cycle, the efficiency is given by:

- η =1- TCTH

- η = 1 - 1- THTC

- η = TH - Tc

- η = TCTH

TH and TC are Temperature of hot & cold reservoirs respectively

- The heat capacity ratio (γ) for a monatomic ideal gas is:

- γ = 1.67

- γ = 1.40

- γ = 1.33

- γ = 1.00

- The critical point of a substance is defined as the point where:

- Liquid and vapor phases coexist

- The latent heat of vaporization is zero

- The substance cannot exist as a liquid

- The substance cannot exist as a gas

- The third law of thermodynamics states that:

- The entropy of a perfect crystal at absolute zero is zero

- The enthalpy of a system is always positive

- The internal energy of a system is constant

- The entropy of a system always increases

- The Joule-Thomson coefficient (μ) is defined as:

- ∂T∂PH

- ∂P∂TH

- ∂H∂TP

- ∂S∂TP

- The Helmholtz free energy (A) is defined as:

- A = U - TS

- A = H - TS

- A = U + PV

- A = H + PV

- The entropy change for an irreversible process is:

- Always zero

- Always positive

- Always negative

- Can be positive, negative, or zero

- What is the definition of a partial molar property?

- The change in the total property of a mixture when one mole of a component is added at constant temperature and pressure

- The total property of a mixture divided by the number of moles of a component.

- The property of a pure component in a mixture.

- The change in the total property of a mixture when the temperature is increased.

- What does the activity coefficient (γ) represent in non-ideal solutions?

- The deviation of a component's behavior from ideality.

- The ratio of the partial pressure of a component to its vapor pressure.

- The enthalpy change of mixing.

- The entropy change of mixing

- The Gibbs-Duhem equation relates:

- The partial molar properties of components in a mixture.

- The temperature and pressure dependence of chemical potential.

- The enthalpy and entropy of mixing.

- The activity coefficients of components in a mixture.

- The excess property of a solution is defined as:

- The sum of the partial molar properties of the components.

- The property of the pure components in the mixture.

- The change in property due to mixing.

- The difference between the actual property and the property of an ideal solution.

- The Lewis-Randall rule applies to:

- Ideal gas mixtures.

- Ideal solutions.

- Non-ideal solutions.

- Pure components

- The Margules equation is used to model:

- The activity coefficients in non-ideal solutions.

- The enthalpy of mixing in ideal solutions.

- The entropy of mixing in regular solutions.

- The fugacity of components in gas mixtures.

- In a binary phase diagram, the tie line connects:

- Points of the same temperature.

- Points of the same pressure.

- Points of the same composition in equilibrium.

- Points of the same enthalpy.

- The Henry's law constant describes the solubility of a gas in a liquid and is valid for:

- Dilute solutions.

- Concentrated solutions.

- Ideal solutions.

- Non-ideal solutions

- The equilibrium constant K for a chemical reaction is related to the standard Gibbs free energy change ∆Go by:

- ∆Go = −RTlnK

- ∆Go = RTlnK

- ∆Go = −RTlogK

- ∆Go =  RTlogK

- For a reaction at equilibrium, the reaction quotient QQ is:

- Greater than the equilibrium constant K.

- Less than the equilibrium constant K.

- Equal to the equilibrium constant K.

- Independent of the equilibrium constant K.

- Le Chatelier's principle states that if a system at equilibrium is disturbed, the system will:

- Remain unchanged.

- Shift in the direction that increases the disturbance.

- Reach a new equilibrium state unrelated to the disturbance.

- Shift to counteract the disturbance and re-establish equilibrium.

- The equilibrium constant Kp​ for a gas-phase reaction is expressed in terms of:

- Partial pressures of the reactants and products.

- Concentrations of the reactants and products.

- Mole fractions of the reactants and products.

- Fugacities of the reactants and products.

- For an exothermic reaction, an increase in temperature will:

- Shift the equilibrium toward the products.

- Have no effect on the equilibrium position.

- Increase the value of the equilibrium constant K.

- Shift the equilibrium toward the reactants.

- The standard Gibbs free energy change ∆Go for a reaction is zero when:

- The reaction is at equilibrium.

- The reaction is spontaneous.

- The reaction is non-spontaneous.

- The reaction quotient Q is zero.

- For a reaction with ∆Go <0, the equilibrium constant K is:

- Less than 1.

- Equal to 1.

- Greater than 1.

- Zero.

- The equilibrium constant K for a reaction depends on:

- Temperature.

- Pressure.

- Initial concentrations of reactants.

- Volume of the reaction vessel.

- The equilibrium constant KK for a reaction is 1. What does this imply about the reaction?

- The reaction favors the formation of products.

- The reaction favors the formation of reactants.

- The reaction is non-spontaneous.

- The reaction is at equilibrium when the concentrations of reactants and products are equal.

Course Name: Chemical Engineering Thermodynamics I           Course Code: ChEg2084

- What is the change in internal energy (ΔU) for an ideal gas undergoing an isothermal process?

  - ΔU = 0

  - ΔU = q (heat added)

  - ΔU = w (work done)

  - ΔU = q + w

- 9. What is the condition for equilibrium between two phases of a pure substance?

  - Equal temperatures.

  - Equal pressures.

  - Equal chemical potentials.

  - All of the above.

- Which of the following is NOT a state function?

  - Temperature

  - Pressure

  - Heat

  - Enthalpy

- For an ideal gas undergoing a reversible adiabatic process, which of the following is constant?

  - Temperature

  - Pressure

  - Volume

  - TV (γ-1) (where γ = Cp/Cv)

- Which thermodynamic property is a measure of the randomness or disorder of a system?

  - Enthalpy

  - Gibbs Free Energy

  - Entropy

  - Internal Energy

- A Carnot cycle consists of which of the following processes?

  - Two isothermal and two isobaric processes.

  - Two adiabatic and two isobaric processes.

  - Two isothermal and two isochoric processes.

  - Two isothermal and two adiabatic processes.

- Which of the following statements is true regarding the standard enthalpy of formation (ΔHf°) of an element in its standard state?

  - It is always positive.

  - It is always negative.

  - It is always zero.

  - It can be positive, negative, or zero depending on the element.

- What is the physical significance of the fugacity coefficient (φ)?

  - It is a measure of the deviation of a real gas from ideality.

  - It is a measure of the compressibility of a gas.

  - It is a measure of the molar volume of a gas.

  - It is a measure of the thermal conductivity of a gas.

- Which of the following is the correct expression for the change in Gibbs Free Energy (ΔG) at constant temperature and pressure?

  - ΔG = ΔH - TΔS

  - ΔG = ΔU + PΔV

  - ΔG = -RTlnK

  - All of the above

- Which of the following is the correct statement of the Third Law of Thermodynamics?

  - Energy cannot be created or destroyed.

  - The entropy of a perfect crystal at absolute zero is zero.

  - The entropy of the universe is always increasing.

  - Heat cannot spontaneously flow from a cold body to a hot body.

- 11. In a throttling process (e.g., flow through a valve), which thermodynamic property remains constant?

  - Temperature

  - Pressure

  - Enthalpy

  - Entropy

- What is the significance of the critical point on a phase diagram?

  - It is the point where all three phases (solid, liquid, gas) coexist in equilibrium.

  - It is the point beyond which distinct liquid and gas phases do not exist.

  - It is the point where the vapor pressure is equal to atmospheric pressure.

  - It is the point where the solid and liquid phases have the same density.

- Which of the following is the correct definition of chemical reaction equilibrium?

  - The rate of the forward reaction equals the rate of the reverse reaction.

  - The change in Gibbs free energy is at its minimum value.

  - The sum of the chemical potentials of the products equals the sum of the chemical potentials of the reactants, each multiplied by their respective stoichiometric coefficients.

  - All of the above

- Which of the following is NOT a typical assumption in the thermodynamic analysis of a control volume?

  - Steady-state conditions.

  - Uniform flow at inlets and outlets.

  - Changes in kinetic and potential energy are negligible.

  - The control volume is adiabatic.

- Which of the following is the best statement of the Zeroth Law of Thermodynamics?

  - Energy cannot be created or destroyed.

  - The entropy of a perfect crystal at absolute zero is zero.

  - If two systems are each in thermal equilibrium with a third system, then they are in thermal equilibrium with each other.

  - Heat cannot spontaneously flow from a cold body to a hot body.

- The First Law of Thermodynamics is a statement of:

  - The conservation of energy.

  - The direction of spontaneous processes.

  - The impossibility of reaching absolute zero.

  - The definition of entropy.

- The Second Law of Thermodynamics states that:

  - Energy cannot be created or destroyed.

  - The entropy of a perfect crystal at absolute zero is zero.

  - The entropy of the universe always increases for a spontaneous process.

  - Heat cannot spontaneously flow from a hot body to a cold body.

- The Third Law of Thermodynamics provides a reference point for:

  - Internal energy.

  - Enthalpy.

  - Entropy.

  - Gibbs free energy.

- Which of the following processes is theoretically reversible according to the Second Law of Thermodynamics?

  - Heat transfer across a finite temperature difference.

  - Friction.

  - Expansion of a gas into a vacuum.

  - A process that occurs infinitely slowly, with no dissipative forces.

- Which law of thermodynamics is the basis for absolute temperature scales (like Kelvin or Rankine)?

  - Zeroth Law

  - First Law

  - Second Law

  - Third Law

- One of the following is not the characteristic of non-ideal gases

- There are intermolecular attractions

- The gas molecules themselves occupy a finite volume

- The molecules themselves occupy negligible volume

- Liquefy at low pressures

- Which is the more effective way to increase the efficiency of a Carnot engine:

- Increase  T1

- Keeping T2 constant

- Decrease  T2

- Keeping T1 constant

- Of the following  one is considered as open system

- Thermos flask

- A gas enclosed in a cylinder

- Human digestive system

- Refrigerator

- The specific heat capacity at constant pressure is greater than the specific heat capacity at constant volume for an ideal gas with exception of water at 4℃. This may due to

- All the heat supplied at constant pressure raises its internal energy and does work in expanding the gas against the external pressure

- All the heat supplied at constant pressure only raises its internal energy

- All the heat supplied at constant volume raises its internal energy and does work in expanding the gas against the external pressure

- All the heat supplied at constant pressure only does work in expanding the gas against the external pressure

- Identify correctly matched statement about work done in thermodynamic processes.

- Isothermal process;

- Isobaric process;

- Isochoric process;

- Adiabatic process;

- Which of the following statements is true concerning a heat pump?

- A heat pump is a refrigeration cycle operating in reverse

- A heat pump is a device that moves heat from a high temperature to a low temperature

- A heat pump is only theoretical; it cannot function because it is a perpetual motion machine

- A heat pump often has a thermal efficiency (coefficient of performance) in excess of 100%

- A heat pump is a device that delivers power by virtue of a heat input

- Two systems (A and B) are shown in the sketch below. Which of these systems are closed systems?

- System A is the only closed system

- System B is the only closed system

- Both systems A and B are closed systems

- Neither system A or B is a closed system

- A vapor compression refrigeration cycle, operating as a refrigerator, has a coefficient of performance of 2.0. If the same cycle operates as a heat pump, the coefficient of performance of the heat pump would be most nearly,

- 1.0

- 1.5

- 2.5

- 3.0

- The enthalpy of an ideal gas is a function of

- Temperature and pressure

- Temperature and specific volume

- Pressure only

- Temperature only

- A process that is reversible and adiabatic must also be,

- Isobaric

- Isenthalpic

- Isothermal

- Isentropic

- A closed, simple compressible system undergoes a process where the heat input to the system is 20 Btu, the kinetic energy increases by 10 Btu, the potential energy decreases by 12 Btu, and the internal energy of the system decreases by 4 Btu. Utilizing the normal sign convention for heat and work, the work done during this process is most nearly,

- -6 Btu

- 14 Btu

- 18 Btu

- 26 Btu

- Which one is not correct about the energy transfer between system and its surrounding?

- The energy flow as result of temperature difference is heat

- Heat and works can be possessed by or contained within the system

- The energy flow in response to driving force is considered as work

- Heat and work refers to the only energy being transferred

- Which one is not an expression for closed system energy balance?

- ∆U + ∆KE +∆PE = Q – W

- ∆U = - W (For stationary, no vertical displacement, isothermal and / or perfectly insulated system)

- ∆H + ∆KE + ∆PE = Q – W

- ∆U = Q (for stationary, no vertical displacement, no moving boundaries)

- Transfer of energy can be accompanied by

- Flow of heat

- Transport of mass

- Work done

- All

- An air conditioning system is used to keep a home cool during hot summer days. Consider a day where the outside temperature is 95°F and it is desired to keep the house at 72°F. The house is gaining heat at a rate of 48,000 Btu/hr. The minimum amount of power required to run the air conditioner is most nearly,

- 1,990 Btu/hr.

- 2,076 Btu/hr.

- 3,199 Btu/hr.

- 11,621 Btu/hr.

- 15,333 Btu/hr.

- In an isothermal expansion of an ideal gas, the amount of heat that flows into the gas is

- Greater than the amount of work done by the gas.

- Equals the amount of work done by the gas.

- Less than the amount of work done by the gas, but greater than zero.

- Zero.

- Heat does not spontaneously flow from a colder body to a hotter one. Which of the following thermodynamics law states this?

- Zeroth law of thermodynamics

- First law of thermodynamics

- Second law of thermodynamics

- Third law of thermodynamics

Choose the correct answer based on the following question from 258 to 263

| State | P(bar(abs)) | T(℃) | X(quality) | Phase | h(kJ/Kg) | s(kJ/Kg.K) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 600 | N/A | Superheated vapor | 3,625.86 | 6.9045 |
| 2 | 0.1 | 45.81 | 0.9 | Saturated liquid-vapor mixture | 2,344.7 | 7.39884 |
| 3 | 0.1 | 45.81 | 0 | Saturated liquid | 191.81 | 0.6492 |
| 4 | 100 | 55 | N/A | Compressed liquid | 259.55 | 0.8260 |
| 5 | 1 | 25 | N/A | Compressed liquid | 104.83 | 0.3672 |
| 6 | 1 | 40 | N/A | Compressed liquid | 167.53 | 0.5724 |

The figure below shows a simple vapor power plant operating at steady state with water as the working fluid. Data at key locations are given in the table. Stray heat transfer and kinetic and potential energy effects may be neglected. The mass flow rate through the system is 12 kg/s and the mass flow rate of the cooling water is 412.03 kg/s, the isentropic enthalpy of the turbine (h2S) and the isentropic enthalpy of pump (h4S) are 2,187.023 kJ/Kg and 202.44 kJ/Kg respectively.

Fig. 1: Vapor Power plant

- The rate of the heat transfer to the steam generator (MW) is nearer to

- 40.4

- 41.5

- 43.0

- 42.5

- The power consumed by the pump ( in KW) is nearer to

- 813

- 812

- 815

- 820

- The thermal efficiency of this power cycle [%] is nearer to

- 36

- 37

- 38

- 34

- The net power developed by the system [in KW] is nearer to

- 13.6

- 14.6

- 15

- 20

- The isentropic efficiency of the turbine  [in %] is nearer to

- 89

- 84

- 87

- 78

- The isentropic efficiency of the pump  [in %] is nearer to

- 15.7

- 14

- 20

- 25

- The amount of energy absorbed or released during a phase-change process referred as

- Heat capacity

- Sensible heat

- Latent heat

- Heat of vaporization

- One of the following is not considered as pure substance

- Mixture of oil and water

- Air

- Water

- Carbone dioxide

- Identify correctly matched statement about temperature measuring units

- One-degree Celsius increment = one-degree Rankine increment

- One-degree Fahrenheit increment = one-degree Rankine decrement

- One-degree Kelvin increment = one-degree Celsius increment

- One-degree Fahrenheit increment = one-degree Celsius increment

- An ideal gas is taken around the cycle shown in this PV diagram, from a to b to c and back to a. Process b → c is isothermal. For this complete cycle,

- Q > 0, W > 0, and ∆U = 0.

- Q > 0, W > 0, and ∆U > 0.

- Q = 0, W > 0, and ∆U < 0.

- Q = 0, W < 0, and ∆U > 0.

- Q > 0, W = 0, and ∆U > 0.

- Of the following  one is  not the characteristic of superheated vapor

- Higher temperature at a given pressure

- Higher  internal energies at a given pressure or temperature

- Lower  pressure at a given temperature

- Lower enthalpies at a given temperature or pressure

- An ideal gas begins in a thermodynamic state a. When the temperature of the gas is raised from T1 to a higher temperature T2 at a constant volume, a positive amount of heat Q12 flows into the gas. If the same gas begins in state a and has its temperature raised from T1 to T2 at a constant pressure, the amount of heat that flows into the gas is

- Greater than Q12.

- Equal to Q12.

- Less than Q12, but greater than zero.

- Negative (heat flows out of the system).

- Which one of the following definitions best describes the concept of work?

- The flow of energy from one object or substance to another due to a difference in temperature

- The flow of energy from one body to another through uniform molecular motion

- The force associated with molecular motion

- The random motion of molecules in a gas at low pressure

- All are correct about compressibility factor except

- It is a measure of deviation from ideal-gas behavior

- It is unity for ideal gases

- It is greater than or less than unity for real gases

- It is a measure of deviation from real –gas behavior

- Of the following  one is  not the characteristic of compressed liquid

- Higher pressure at a given temperature

- Higher  internal energies at a given pressure or temperature

- Lower  temperature at a given pressure

- Lower enthalpies at a given temperature or pressure

- All are correct about entropy except:

- The entropy of a pure crystalline substance at absolute zero temperature is zero

- It is a measure of molecular disorder, or molecular randomness

- The entropy of a pure crystalline substance at absolute zero temperature is greater than zero

- There is no entropy transfer associated with energy transfer as work.

Question 276 to 277: When a system is taken from state a to state b in the accompanying figure along path a-c-b, 100 J of heat flows into the system and the system does 40 J of work.

- How much heat flows into the system along path a-e-b if the work done by the system is 20 J?

- 80 J

- 60 J

- 85 J

- 75 J

- The system returns from b to a along path b-d-a. If the work done on the system is 30 J, how much heat the system absorb or liberate heat?

- 90 J

- -90 J

- 85 J

- -85 J

- One of the following is true about the Clausius Inequality Statement

- A system executes a cyclic process the integral around the cycle of (δQ/T) is less than or equal to zero.

- The efficiency of an irreversible heat engine is always less than the efficiency of a reversible one operating between the same two reservoirs.

- It is impossible to construct a device which operating in a cycle produces no other effect than extract heat from a high temperature reservoir and convert the heat completely into mechanical work

- No cyclic process can transfer heat from a cold reservoir to a hotter region with no input of mechanical work.

- Entropy, which is a measure of the disorder of a system, is:

- Independent of pressure

- Independent of temperature

- Zero at absolute zero temperature for a perfect crystalline substance

- All (a), (b) & (c

- Two Carnot refrigerators operate in series between two reservoirs maintained at 20 ℃ and 200 ℃, respectively. The energy output by the first refrigerator is used as the heat energy input to the second refrigerator. If the COPs of the two refrigerators are the same, what should the intermediate temperature be?

- 99.3 ℃

- 85 ℃

- 120.5℃

- 90.75 ℃

- Air undergoes a three-process cycle with a P = const. process, a T = const. process, and a V = const. process. Select the correct statement for a piston-cylinder arrangement.

- W = 0 for the P = const. process

- Q = 0 for the V = const. process

- Q = 0 for the T = const. process

- W = 0 for the V = const. process

- A heat pump is to provide 2000 kJ/hr. to a house maintained at 20 ℃. If it is −20 ℃ outside, what is the minimum power requirement?

- 385 kJ/hr.

- 316 kJ/hr.

- 273 kJ/hr.

- 184 kJ/hr.

- Which of the following can be assumed to be reversible?

- A paddle wheel.

- A burst membrane.

- A resistance heater.

- A piston compressing gas in a race engine.

- An inventor claims that a thermal engine, operating between ocean layers at 27 ℃ and 10℃, produces 10 kW of power while discharging 9900 kJ/min. This engine is:

- impossible

- reversible

- possible

- probable

- A Carnot cycle is a cycle of special interest because:

- It establishes a lower limit on cycle efficiency.

- It operates between two constant-temperature thermal reservoirs.

- It provides the maximum efficiency for any cycle.

- When it is carefully constructed in a laboratory, it provides an upper limit on cycle efficiency.

- A Carnot engine, operating between reservoirs at 20℃ and 200℃, produces 10 kW of power. The rejected heat is nearest:

- 26.3 kJ/s

- 20.2 kJ/s

- 16.3 kJ/s

- 12.0kJ/s

- A househusband is cooking beef stew for his family in a pan that is (I) uncovered, (II) covered with a light lid, and (III) covered with a heavy lid. For which case will the cooking time be the shortest?

- I

- II

- III

- I & III

- A gas flow steadily through a rotary compressor. The gas enters the compressor at a temperature of 16℃, a pressure of 100 kPa, and an enthalpy of 39l.2 kJ/kg. The gas leaves the compressor at a temperature of 245℃, a pressure of 0.6 MPa, and an enthalpy of 534.5 kJ/kg. There is no heat transfer to or from the gas as it flows through the compressor. The external work done per unit mass of gas when the gas velocity at entry is 80 m/s and that at exit is 160 m/s is nearer to.

- 250.4

- 152.9

- 168.9

- 160.5

Course Name: Process Industries

- The chamber process is:

- Preferred over contact process for producing 98 to 100% H2SO4 and various oleums.

- Non-catalytic and operates only on pyrites.

- A batch process for directly producing high strength (98 to 100%) H2SO4.

- None of these.

- Sucrose content in the raw juice extracted from sugar cane is about __________ percent.

- 1 – 2

- 15 – 20

- 50 – 60

- 80 – 85

- Antibiotic

- Inhibits/destroys the growth of microorganisms.

- Is used as a pain reliever.

- Is an antimalarial.

- Is an anaesthetic.

- Penicillin, an antibiotic drug was discovered by

- Alexender Flaming

- Dr. Zivago

- Frederick

- none of these

- SO2 is bubbled through hot sugar cane juice to

- Act as an acidifying agent.

- Increase its concentration.

- Increase the amount of molasses.

- Increase the crystal size.

- Chemical formula of oleum is

- H2SO3

- H2SO4

- H2S2O7

- H2SO7

- Gypsum is chemically

- Calcium chloride

- Potassium sulphate

- Sodium sulphate

- Calcium sulphate

- In multistage equilibrium conversion of SO2 to SO3 (2SO2 + O2  2SO3), the reverse reaction becomes appreciable at a temperature of 550° C. The percentage equilibrium conversion of SO2 to SO3 can be increased by

- Increasing the oxygen concentration.

- Putting more quantity of V2O5 catalyst in the converter.

- Removing some quantity of SO3 during intermediate stage.

- Maintaining low temperature & pressure in the converter.

- Oxidation of SO2 to SO3 is favoured by:

- Low temperature and low pressure.

- Low temperature and high pressure.

- High temperature and low pressure.

- High temperature and high pressure.

- All enzymes are made of

- Fats

- Carbohydrates

- Proteins

- Amino acids

- Frasch process is for

- Making oxygen

- Producing helium

- Mining sulphur

- Making nitrogen

- Cement mainly contains

- CaO, SiO2, Al2O3

- MgO, SiO2, K2O

- Al2O3, MgO, Fe2O3

- CaO, MgO, K2O

- Which of the following is not components of cement clinker

- Di-calcium silicate

- Tri-calcium silicate

- Gypsum

- None

- Which one of the following is a method used for industrial synthesis of ammonia:

- Steam reforming of natural gas

- Partial oxidation of heavy oils

- Water electrolysis

- All

- Chlorine gas is produced by the electrolysis of brine (NaCl solution with solid NaCl make up) in mercury electrolytic cell. Which of the following is the anodic reaction?

- Oxidation of Na+ ions

- Oxidation of Cl- ions

- Reduction of Na+ ions

- Reduction of Cl- ions

- Massacuits is a terminology used in the __________ industry.

- Paint

- Oil hydrogenation

- Soap

- Sugar

- Mercury electrolytic cells are preferred over diaphragm electrolytic cell (for production of caustic soda), as it

- Has larger production capacity per unit cell.

- Consumes less power per ton of Cl2 produced.

- Produces high purity (70%) caustic soda directly.

- All

- In the Solvay process, the product from the calciner is

- Light soda ash

- Dense soda ash

- Sodium bicarbonate

- Dehydrated soda ash

- Pencillin is made employing __________ fermentation process.

- Continuous

- Aerobic batch

- Anaerobic batch

- None of these

Course Name: Fluid Machines in Chemical Engineering            Course Code: ChEg3086

- The principle of conservation of energy in fluid machines is governed by which law?

a) Bernoulli’s Equation b) Newton's Second Law c) Boyle's Law d) Charles's Law

- Which of the following is an example of a fluid machine?

a) Electric Motor b) Centrifugal Pump c) Diesel Engine d) Gearbox

- A Pelton wheel turbine is most suitable for:

a) High head and low flow b) Low head and high flow c) Medium head and medium flow d) High head and high flow

- Cavitation in fluid machines occurs when:

a) The pressure drops below the vapor pressure of the fluid b) The fluid flow velocity decreases c) The temperature of the fluid decreases rapidly d) A fluid changes direction in a pipe

- Which term defines the ratio of the actual work delivered by a fluid machine to the theoretical work input?

a) Efficiency b) Power Coefficient c) Reynolds Number d) Specific Speed

- In a centrifugal pump, the function of the impeller is to:

a) Increase the pressure of the fluid b) Reduce the frictional losses c) Convert mechanical energy into fluid energy d) Both a and c

- Which of the following turbines is considered an impulse turbine?

a) Francis Turbine b) Pelton Wheel c) Kaplan Turbine d) Propeller Turbine

- The specific speed of a pump is a measure of:

a) The shape of the impeller b) The type of fluid being handled c) The speed at which the pump operates at maximum efficiency d) The power consumed by the pump

- Which of the following parameters affects the efficiency of a hydraulic turbine?

a) Head b) Flow rate c) Velocity triangle at the blades d) All of the above

- A draft tube in a reaction turbine is used to:

a) Increase the flow rate b) Recover pressure from the kinetic energy of the fluid c) Control the rotational speed of the turbine d) Prevent cavitation

- In a reciprocating pump, the purpose of the air vessel is to:

a) Reduce power consumption b) Minimize friction losses c) Smoothen the flow and reduce pulsations d) Prevent overheating of the pump

- In fluid dynamics, which number represents the ratio of inertial forces to viscous forces?

a) Mach Number b) Reynolds Number c) Froude Number d) Prandtl Number

- The performance of a hydraulic pump is evaluated based on its:

a) Flow rate b) Pressure head c) Efficiency d) All of the above

- Which type of turbine is preferred for medium head and medium flow conditions?

a) Pelton Wheel b) Francis Turbine c) Kaplan Turbine d) Impulse Turbine

- In fluid machines, the head developed by a pump is measured in:

a) Meters of fluid b) Liters per second c) Newtons per square meter d) Cubic meters per hour

- The Euler equation for turbomachinery relates:

a) Pressure change and heat transfer b) Work done and energy changes in the fluid c) Flow rate and velocity d) Torque and angular velocity

- In hydraulic turbines, draft tubes are required for:

a) Maintaining constant velocity b) Converting kinetic energy to pressure energy c) Reducing turbulence in the fluid d) Cooling the turbine blades

- The function of the volute casing in a centrifugal pump is to:

a) Reduce cavitation b) Reduce the velocity and increase pressure c) Maintain a constant flow rate d) Increase the rotational speed of the impeller

- A characteristic curve of a pump shows the relationship between:

a) Power and efficiency b) Head, flow rate, and efficiency c) Temperature and flow rate d) Torque and rotational speed

- The performance of a Pelton wheel turbine is governed by which non-dimensional number?

a) Mach Number b) Froude Number c) Reynolds Number d) Weber Number

- Which type of pump is ideal for moving highly viscous fluids?

a) Centrifugal Pump b) Reciprocating Pump c) Rotary Pump d) Jet Pump

- Slip in a reciprocating pump refers to:

a) The difference between theoretical and actual discharge b) The increase in temperature due to friction c) The change in head pressure during operation d) The loss of energy in the suction pipe

- The main advantage of axial flow pumps over centrifugal pumps is:

a) High head generation b) High flow rate at low head c) Compact design for high efficiency d) Ability to pump highly viscous fluids

- In a reaction turbine, energy transfer occurs due to:

a) Pressure and velocity changes in the fluid b) Only velocity changes in the fluid c) Only pressure changes in the fluid d) Momentum changes in the fluid flow

- Which of the following is a positive displacement pump?

a) Centrifugal Pump b) Axial Flow Pump c) Rotary Gear Pump d) Jet Pump

- The term "specific speed" of a turbine helps to classify turbines based on:

a) Efficiency b) Size of the runner c) Head and flow conditions d) Type of working fluid

- Hydraulic power in fluid machines is expressed as the product of:

a) Flow rate and pressure head b) Torque and speed c) Pressure and velocity d) Efficiency and heat transfer

- A Kaplan turbine is an example of:

a) A reaction turbine with fixed vanes b) An impulse turbine with fixed vanes c) A reaction turbine with adjustable blades d) An impulse turbine with adjustable blades

- The purpose of a diffuser in a fluid machine is to:

a) Reduce fluid velocity and recover pressure energy b) Increase fluid velocity and reduce pressure energy c) Maintain a constant flow rate d) Eliminate cavitation

- Which of the following factors influences cavitation in a centrifugal pump?

a) Suction pressure b) Discharge velocity c) Impeller material d) Motor speed

- The hydraulic efficiency of a turbine is defined as:

a) The ratio of water power to shaft power b) The ratio of mechanical power to hydraulic power c) The ratio of runner power to water power d) The ratio of water power to overall power

- The discharge through a centrifugal pump varies:

a) Directly as the speed b) As the square of the speed c) As the cube of the speed d) Independently of the speed

- The purpose of guide vanes in a reaction turbine is to:

a) Minimize turbulence in the fluid b) Direct the fluid onto the runner blades at the proper angle c) Maintain constant flow rate d) Prevent cavitation

- A pump operating under cavitation will typically:

- Experience a drop in efficiency b) Generate more power c) Produce smooth and continuous flow d) Increase discharge pressure

- Vortex formation in centrifugal pumps is undesirable because:

- It reduces volumetric efficiency

- It leads to cavitation

- It causes vibrations and noise

- All

- Which type of hydraulic turbine is considered suitable for very high heads?

a) Kaplan Turbine b) Pelton Wheel c) Francis Turbine d) Propeller Turbine

- The term "head loss" in fluid flow refers to:

a) The loss of velocity in a fluid b) The energy loss due to friction and turbulence c) The difference between discharge and suction pressure d) The change in fluid direction

- Which of the following is an example of a dynamic pump?

a) Gear Pump b) Centrifugal Pump c) Reciprocating Pump d) Diaphragm Pump

- The unit of specific speed for a pump is:

a) Dimensionless b) Liters per second c) Meters per second d) Watts per kilogram

- In hydraulic turbines, the ratio of the velocity of the fluid at the runner inlet to the theoretical velocity is called:

a) Flow Ratio b) Velocity Coefficient c) Speed Ratio d) Discharge Coefficient

- Which property of a fluid determines its resistance to flow?

a) Density b) Viscosity c) Surface Tension d) Compressibility

- The flow in a turbine passage is classified as radial when:

a) The flow enters axially and leaves axially b) The flow enters radially and leaves radially c) The flow enters axially and leaves radially d) The flow enters radially and leaves

- Which term is used to describe the amount of energy imparted to the fluid in a pump or turbine?

a) Hydraulic Head b) Specific Speed c) Energy Coefficient d) Hydraulic Power

- The suction lift of a centrifugal pump is defined as:

a) The vertical height from the pump to the discharge point b) The horizontal distance from the pump to the fluid source c) The vertical distance between the pump and the fluid source d) The head loss due to friction in the suction pipe

- The efficiency of a hydraulic turbine is maximum when:

a) Flow rate is highest b) Frictional losses are negligible c) Velocity of water matches the velocity of the runner d) Cavitation is eliminated

- The working principle of a centrifugal pump is based on:

a) Conservation of energy b) Centrifugal force c) Bernoulli’s principle d) Law of continuity

- In a fluid machine, the term "priming" refers to:

a) Starting the motor of the machine b) Removing air from the pump casing and suction pipe c) Balancing the flow rate in the system d) Increasing the speed of the impeller

- The performance of fluid machines is analyzed using:

a) Navier-Stokes equations b) Dimensionless numbers c) Thermodynamic principles d) Fluid surface tension

- Which of these is an example of a reaction turbine?

a) Pelton Wheel b) Francis Turbine c) Impulse Turbine d) Turgo Turbine

- The volumetric efficiency of a reciprocating pump is defined as:

a) The ratio of actual discharge to the theoretical discharge b) The ratio of power output to power input c) The ratio of head loss to pump head d) The ratio of fluid velocity to pump speed

- The best performance of a Kaplan turbine is achieved when:

a) The runner blade angle is fixed b) The guide vane angle is adjusted c) The draft tube is eliminated d) Both guide vane and blade angles are adjustable

- Which device is typically used to measure flow rate in fluid systems?

a) Pitot Tube b) Orifice Meter c) Venturi Meter d) All of the above

- In a mixed flow turbine, the flow of water:

a) Enters radially and exits axially b) Enters axially and exits radially c) Enters both radially and axially, exiting axially d) Is parallel to the axis of the turbine

- Which type of impeller is best suited for pumping solids-laden or slurry fluids?

a) Closed impeller b) Semi-open impeller c) Open impeller d) Multistage impeller

- The cavitation number is defined as:

a) The ratio of NPSH to the vapor pressure b) The ratio of actual head to suction head c) A dimensionless parameter indicating cavitation risk d) The ratio of cavitation pressure to atmospheric pressure

- Hydraulic machines that transfer energy from the fluid to the machine are called:

a) Pumps b) Compressors c) Motors d) Turbines

- In an impulse turbine, the pressure change across the runner is:

a) Significant b) Negligible c) Equal to the velocity head d) Dependent on cavitation

- Which of the following improves the efficiency of a centrifugal pump?

a) Use of a volute casing b) Increasing impeller diameter c) Ensuring adequate NPSH d) All of the above

- The main function of bearings in a fluid machine is to:

a) Support rotating components b) Reduce leakage c) Increase the pressure head d) Prevent cavitation

- The term "critical speed" in fluid machines refers to:

a) The maximum allowable speed of the machine b) The speed at which vibrations are amplified c) The speed at which cavitation occurs d) The speed corresponding to maximum efficiency

- The degree of reaction in a turbine is defined as:

a) The ratio of static head to total head b) The ratio of pressure energy to kinetic energy c) The ratio of static pressure drop in the rotor to the total static pressure drop d) The ratio of theoretical head to actual head

- The term "multistage pump" refers to:

a) Pumps with multiple suction lines b) Pumps with multiple impellers mounted on the same shaft c) Pumps with adjustable flow rates d) Pumps with both centrifugal and axial flow capabilities

- The hydraulic grade line (HGL) in a pump system represents:

a) The total energy head available at each point in the system b) The pressure head and velocity head in the system c) The pressure head in the fluid d) The total mechanical energy lost due to friction

Course Name: Fluid Mechanics for Chemical Engineering     Course Code: ChEg3081

- Buoyancy arises from the fact that

- fluid pressure increases with depth

- the increased pressure is exerted in all directions

- there is an unbalanced upward force on the bottom of a submerged object

- all

- The drag and lift forces depend on

    - the density of the fluid

    - the upstream velocity

    - The size, shape, and orientation of the body.

    - All

- The location of fluid separation point depends on:

    - the  Reynolds number

    - the surface roughness

    - the level of fluctuations in the free  stream

    - All

- How do you know the occurrence of fluid separation?

    - When fluid recirculating and back-flows occur.

    - Due to the pressure drag.

    - Because of the reduced velocity in the downstream

    - it is impossible to know unless we see a separated region between the body and the fluid stream through our naked eye

- Newton’s law of viscosity relates

- intensity of pressure and rate of angular deformation

- shear stress and rate of angular deformation

- shear stress, viscosity and temperature

- viscosity and rate of angular deformation

- Pascal-second is the unit of

- pressure

- kinematic viscosity

- dynamic viscosity

- surface tension

- A three-wheeled vehicle (Bajaj) with a drag coefficient (𝐶𝑑) of 0.4 and a frontal area (𝐴) of 2.21 m² is modified to reduce its height, resulting in a frontal area of 1.95 m². The vehicle travels a distance (𝑑) of 19,000 km per year at an average speed (𝑣) of 80 km/h. The fuel used is benzene, which has a density (𝜌benzene) of 0.87 kg/L, a price of 75 birr/L, and a heating value (𝐻𝑣) of 41.8 MJ/kg. The air density (𝜌air) is 1.2 kg/m³, and the drivetrain efficiency (𝜂) of the vehicle is 27%. Based on these conditions, how much fuel and money are saved per year due to the height modification?

- 60 liters and 4,500 birr b) 500 liters and 40,000 birr c) 400 liters and 30,000 birr d) 3 liters and 200 birr.

- Equation of continuity is based on the principle of conservation of

- mass

- energy

- momentum

- none of the above

- The continuity equation pi V,A,= p2V2A2 is based on the following assumption regarding flow of fluid

- steady flow

- uniform flow

- incompressible flow

- Frictionless flow where pi and p2 are mass densities.

- Stream lines and path lines always coincide in case of

- steady flow

- laminar flow

- uniform flow

- none

- In steady flow of a fluid, the total acceleration of any fluid particle

- can be zero

- is never zero

- is always zero

- is independent of coordinates

- The pitot tube is used to measure

- velocity at stagnation point

- stagnation pressure

- static pressure

- dynamic pressure

- The major loss of energy in long pipes is due to

    - sudden enlargement

    - sudden contraction

    - gradual contraction or enlargement

    - friction

- Which of the following statements is correct?

- Lower critical Reynolds number is of no practical significance in pipe flow problems.

- Upper critical Reynolds number is significant in pipe flow problems.

- Lower critical Reynolds number has the value 2000 in pipe flow

- Upper critical Reynolds number is the number at which turbulent flow changes to laminar flow.

- For a sphere of radius 15 cm moving with a uniform velocity of 2 m/sec through a liquid of specific gravity 0.9 and dynamic viscosity 0.8 poise, the Reynolds number will be

- 300

- 337.5

- 600

- 675

- The shear stress distribution for a fluid flowing in between the parallel plates, both at rest, is

- constant over the cross section

- parabolic distribution across the section

- zero at the mid plane and varies linearly with distance from mid

- plane

- zero at plates and increases linearly to midpoint

- If a sphere of diameter 1 cm falls in castor oil of kinematic viscosity 10 stokes, with a terminal velocity of 1.5 cm/sec, the coefficient of drag on the sphere is

- less than 1

- between 1 and 100

- 160

- 200

- In case of an airfoil, the separation of flow occurs

- at the extreme rear of body

- at the extreme front of body

- midway between rear and front of body

- any where between rear and front of body depending upon Reynolds number

- When an ideal fluid flows past a sphere,

- highest intensity of pressure occurs around the circumference at right angles to flow

- lowest pressure intensity occurs at front stagnation point

- lowest pressure intensity occurs at rear stagnation point

- total drag is zero

- In which of the following the friction drag is generally larger than pressure drag?

- a circular disc or plate held normal to flow

- a sphere

- a cylinder

- an airfoil

- For hydro-dynamically smooth boundary, the friction coefficient for turbulent flow is

- constant

- dependent only on Reynolds number

- a function of Reynolds number and relative roughness

- dependent on relative roughness only

- Separation of flow occurs when

- the pressure intensity reaches a minimum

- the cross-section of a channel is reduced

- the boundary layer comes to rest

- all of the above

- The ratio of average velocity to maximum velocity for steady laminar flow in circular pipes is

- 1/2

- 2/3

- 3/2

- 2

- The discharge of a liquid of kinematic viscosity 4 Cm2/sec through a 8 cm diameter pipe is 3200n cm/sec. The type of flow expected is

- laminar flow

- transition flow

- turbulent flow

- not predictable from the given data

- The losses are more in

- laminar flow

- transition flow

- turbulent flow

- critical flow

- The wake

- always occurs before a separation point

- always occurs after a separation point

- is a region of high pressure intensity

- none of the above

- The maximum thickness of boundary layer in a pipe of radius r is

- 0

- r/2

- r

- 2r

- The hydraulic grade line is

- always above the centre line of pipe

- never above the energy grade line

- always sloping downward in the direction of flow

- all of the above

- In series-pipe problems

- the head loss is same through each pipe

- the discharge is same through each pipe

- a trial solution is not necessary

- the discharge through each pipe is added to obtain total discharge

- Select the correct statement.

- The absolute roughness of a pipe de-creases with time.

- A pipe becomes smooth after using for long time.

- The friction factor decreases with time.

- The absolute roughness increases with time.

- Drag force is a function of

i) Projected area of the body

ii) Mass density of the fluid

iii) Velocity of the body

The correct answer is

- (i) and (ii)

- (i) and (iii)

- (ii) and (iii)

- (i), (ii) and (iii)

- For laminar flow in circular pipes, the Darcy’s friction factor f is equal to

- 16/Re

- 32/ Re

- 64/ Re

- None of the above where R,, is Reynolds number.

- If a person studies about a fluid which is at rest, what will you call his domain of study?

a) Fluid Dynamics
b) Fluid Mechanics
c) Fluid Statics
d) Fluid Kinematics

- When is a fluid called turbulent?

a) High viscosity of fluid
b) Reynolds number is greater than 2000
c) Reynolds number is less than 2000
d) The density of the fluid is low

- When a fluid is subjected to resistance, it undergoes a volumetric change due to __________

a) Cohesion
b) Strain
c) Compressibility
d) Adhesion

- Which among the following is an assumption of Hagen-Poiseuille equation?

a) Fluid is uniform
b) Fluid is laminar
c) Fluid is turbulent
d) Fluid is compressible

- Which among the following is not global parameters of fluid?

a) Mass flow rate
b) Density
c) Viscosity
d) External diameter

- which of the following is the mathematical technique used to predict physical parameters?

a) Dimensional analysis
b) Temperature analysis
c) Pressure analysis
d) Combustion analysis

- Pressure variation for compressible fluid is maximum for which of the following kind of process?
a) Adiabatic
b) Quasi Static
c) Isothermal
d) None of the mentioned

- Which of the following principle is used for calculating the centre of pressure?
a) Principle of balancing of momentum
b) Principle of momentum
c) Principle of conservation of energy
d) None of the mentioned

- When the body is completely or partially immersed in a fluid, how much its weight be distributed for it to be in stable equilibrium.

a) Is independent of weight distribution
b) Around the lower part
c) Around the upper part
d) None of the mentioned

- Which of the following equation must be perfunctorily satisfied while dealing with fluid flow problems?
a) Newton’s third law
b) Law of conservation of momentum
c) Continuity equation
d) Newton’s second law

- Open channel flow takes place _______
a) In a pump
b) Within a cylindrical depth
c) On a free surface
d) In the pipe

- Which of the following is a type of fluid based on viscosity?
a) Real Fluid
b) Ideal Fluid
c) Newtonian Fluid
d) All of the mentioned

- Pressure intensity or force due to pressure gradient for fluid at rest is considered as which of the following kind of force?
a) Body force
b) Force due to motion
c) Surface force
d) None of the mentioned

- What is the primary principle governing fluid mechanics?

- Conservation of mass b) Conservation of energy c) Conservation of momentum  d) All of the above

- The Reynolds number is used to determine:

  - The type of fluid b) The flow regime (laminar or turbulent) c) The viscosity of the fluid d) The compressibility of the fluid

- In fluid statics, the pressure at a point in a fluid is:

- The same in all directions b) Greater in the horizontal direction c) Greater in the vertical direction d) Dependent on the velocity of the fluid

- Bernoulli’s equation is applicable to:

- Compressible flows only b) Incompressible flows only c) Both compressible and incompressible flows d) Flows with high viscosity

- The continuity equation is based on the principle of:

- Conservation of energy b) Conservation of mass c) Conservation of momentum d) Conservation of pressure

- Which of the following is a dimensionless number?

- Reynolds number b) Froude number c) Mach number d) All of the above

- The velocity profile for laminar flow in a pipe is:

- Parabolic b) Linear c) Uniform d) Exponential

- The property of a fluid that determines its resistance to shear stress is:

- Density b) Viscosity c) Surface tension d) Compressibility

- The pressure drop in a pipe is directly proportional to:

- The square of the velocity b) The length of the pipe c) The viscosity of the fluid d) All of the above

- In a Venturi meter, the pressure difference is used to measure:

- Velocity of the fluid b) Flow rate c) Density of the fluid d) Viscosity of the fluid

- The term "hydrostatic pressure" refers to:

- Pressure due to fluid motion b) Pressure due to fluid at rest c) Pressure due to fluid viscosity d) Pressure due to fluid turbulence

- The drag force on a sphere moving through a fluid depends on:

- The velocity of the sphere b) The viscosity of the fluid c) The diameter of the sphere d) All of the above

- The Navier-Stokes equations describe:

- The motion of solid objects b) The motion of fluids c) The energy transfer in fluids d) The compressibility of gases

- The term "boundary layer" refers to:

- The region of fluid near a solid surface where viscosity effects are significant b) The region of fluid far from a solid surface c) The region of fluid with uniform velocity d) The region of fluid with zero pressure

- The Froude number is used to analyze:

- Compressible flows b) Open channel flows c) Laminar flows d) Turbulent flows

- The term "specific gravity" is defined as:

- The ratio of the density of a fluid to the density of water b) The ratio of the viscosity of a fluid to the viscosity of water c) The ratio of the velocity of a fluid to the velocity of water d) The ratio of the pressure of a fluid to the pressure of water

- The Mach number is defined as the ratio of:

- Velocity of the fluid to the speed of sound in the fluid b) Pressure of the fluid to the atmospheric pressure c) Density of the fluid to the density of air d) Viscosity of the fluid to the viscosity of air

- The term "capillary action" is due to:

- Surface tension b) Viscosity c) Density d) Compressibility

- The Hagen-Poiseuille equation is valid for:

- Turbulent flow in pipes b) Laminar flow in pipes c) Compressible flow in pipes d) Flow in open channels

| Course Name: Introduction to Biochemical Engineering | Course Code: ChEg5171 |
| --- | --- |

- What does biotechnology involve?

  - The exploration of galaxies and space travel.

  - The exploitation of living microorganisms and their by-products for safe food additives, medicines, and cosmetics.

  - The exclusive application of mathematics in biology.

  - The replacement of engineering with pure biological sciences.

- Which of the following is a characteristic of traditional biotechnology?

  - It uses genetic engineering to modify organisms at the molecular level.

  - It focuses exclusively on developing pharmaceutical drugs.

  - It involves the use of advanced techniques such as CRISPR.

  - It includes ancient practices like fermentation for making bread, beer, and cheese.

- The catalytic efficiency of an enzyme is measured by

  - Kcat

  - Vmax

  - Km

  - Kcat/Km

- Which of the following is a key property of enzymes?

  - They are consumed during the reactions they catalyze.

  - They can only catalyze reactions at extremely high temperatures.

  - They are highly specific to the reactions they catalyze.

  - They work equally well with all substrates.

- Which of the following is true about the activity of enzymes?

  - Enzymes function optimally across all pH and temperature ranges.

  - Enzymes are temperature- and pH-sensitive, functioning best under specific conditions.

  - Enzymes are unaffected by changes in their environment.

  - Enzymes perform better at higher substrate concentrations, without limit.

- What role does activation energy (Ea) play in enzyme-catalyzed reactions?

  - It is the minimum energy required to initiate a reaction.

  - It determines the final equilibrium state of the reaction.

  - It does not affect the speed of the reaction.

  - It decreases as temperature increases, regardless of the presence of an enzyme.

- How do enzymes affect activation energy (Ea) in biochemical reactions?

  - They increase the activation energy needed for the reaction.

  - They increase the overall Gibbs free energy of the reaction.

  - They do not influence the activation energy at all.

  - They decrease the activation energy, making the reaction faster.

- What happens to the Gibbs free energy (ΔG) of a reaction when an enzyme is added?

  - It increases due to enzyme activity.

  - It decreases significantly.

  - It remains unchanged, as enzymes do not affect ΔG.

  - It becomes positive regardless of the reaction type.

- What is the active site of an enzyme?

  - The region where the enzyme stores energy.

  - The part of the enzyme that binds specifically to the substrate and facilitates the reaction.

  - The location where the enzyme breaks down entirely.

  - The inactive region of the enzyme molecule.

- Which of the following is considered a disadvantage to the adsorption method of immobilization?

  - It is possible to separate and purify the enzymes while being immobilized

  - The enzymes are not usually deactivated by adsorption

  - The state of immobilization is very sensitive to solution pH, ionic strength, and temperature

  - The adsorption is a reversible process

- The immobilized technique involving the chemical method is?

  - covalent bond formation dependent

  - Adsorption

  - non-covalent bond formation dependent

  - Microencapsulation

- For a steady state condition, the change of substrate concentration (dCs/dt) is

  - 1

  - >1

  - <1

  - Zero

- Damkohler number(NDa) is

  - the ratio of the minimum reaction rate to the maximum mass transfer rate

  - the ratio of the maximum reaction rate to the maximum mass transfer rate

  - the ratio of the maximum reaction rate to the minimum mass transfer rate

  - the ratio of the minimum reaction rate to the minimum mass transfer rate

- For glucose isomerization by immobilized enzyme, the reactor generally used is

  - CSTR

  - plug flow

  - fluidized bed

  - packed bed

- If Damkohler number, NDa >>1, the reaction rate is much greater than the mass transfer rate and the overall reaction is controlled by the rate of mass transfer. In this case, the enzyme reaction can be described as (where Csb and Cs are the substrate concentration in the bulk of the solution and at the immobilized enzyme surface respectively. Ks mass transfer coefficient and a is the surface area of immobilized enzyme particles)

  - rp = Ksa(Csb-Cs)

  - rp = KsaCsb

  - rp = KsaCs

  - rp = Ksa(Csb+Cs)

- Which of the following is not a physical method of immobilization?

  - Adsorption

  - Microencapsulation

  - Entrapment

  - Crosslinking

- To measure the extent to which the reaction rate is lowered because of resistance to mass transfer, the effectiveness factor of an immobilized enzyme, η can be defined as

  - reaction rate/rate if not slowed by diffusion

  - rate if not slowed by diffusion/ actual reaction rate

  - actual reaction rate/ rate if not slowed by diffusion

  - rate if not slowed by diffusion/ reaction rate

- The slope and intercept obtained from (1/rate) against (1/substrate concentration) of an enzyme- catalyzed reaction are 300 and 2*10^5, respectively. The Michaels-Menten constants of the enzyme in this reaction is

  - 5*10^6M

  - 1.5*10^3M

  - 1.5*10^-3M

  - 5*10^-6M

- For an enzyme-catalyzed reaction, a Lineweaver-Burk plot gave the following data: slope=40s, intercept=4(mmol dm-3 s-1)^-1, if the initial concentration of enzyme is 2.5 *10^-9 mol dm-3, what is the catalytic efficiency (in dm3mol-1s-1) of the reaction?

  - 107

  - 106

  - 105

  - 10

- The enzyme-catalyzed conversion of a substrate at 25 °C has an intercept of 4*103mol-1dm3s and a Michaels-Menten constant of 0.04 mol dm-3. Calculate the catalytic efficiency if the initial concentration of the enzyme is 10-4moldm-3.

  - 62.5

  - 6.25

  - 0.625

  - 625

- Which phase has the condition of specific growth rate “μ = 0”?

  - Lag phase

  - Log phase

  - Death phase

  - Stationary phase

- Which phase has the condition of specific growth rate “μ ≈ μmax”?

  - Lag phase

  - Growth phase

  - Log phase

  - Death phase

- The lag phase constitutes of

  - Change in number but decrease in mass

  - No change in number, but an increase in mass

  - No change in number and decrease in mass

  - Constant number and mass

- In the accelerated phase, the cell starts to

  - decrease and the division rate increases to reach a maximum

  - increase and the division rate decreases to reach a maximum

  - increase and the division rate increases to reach a minimum

  - increase and the division rate increases to reach a maximum

- Which of the following is used to grow bacterial cultures continuously?

  - Thermostat

  - Haemostat

  - Chemostat

  - Coulter counter

- How else can we measure biomass?

  - Spectrophotometer

  - Rheometry

  - Nephelometer

  - Mass spectrometry

- Which of the following is not a primary objective of biochemical engineering?

  - Minimization of production costs

  - Maximization of enzyme activity

  - Optimization of bioprocesses

  - Maximization of waste generation

- Which one is not the use of the Cell counting technique?

  - To determine the health of the cells

  - To determine cell variability

  - To determine the growth rate

  - To determine the genetic structure of cells

- What is the first successful genetically engineered product?

  - Somatotropin

  - Growth hormone

  - Tobacco

  - Insulin

- Which of the following is not the physical method for the cells rupturing?

  - Milling

  - Enzymatic digestion

  - Homogenization

  - Ultrasonication

- Which of the operations does not come under upstream processing?

  - Media preparation

  - Effluent treatment

  - Inoculum development

  - Storage of raw material

- The power number is expressed as

  - P0 / N2Di3ρ

  - P0 / N3Di3ρ

  - P0 / N2Di2ρ

  - P0 / N3Di5ρ

- Surface renewal theory predicts that the mass transfer coefficient is

  - inversely proportional to the square root of the molecular diffusivity

  - directly proportional to the cube root of the molecular diffusivity

  - directly proportional to the square root of the molecular diffusivity

  - indirectly proportional to the cube root of the molecular diffusivity

- The range of width for the baffle in the fermenter is 	times of vessel diameter

  - 1/2 to 1/3

  - 1/10 to 1/12

  - 1/8 to 1/10

  - 1/5 to 1/8

- Why vortexing is undesirable in the agitation of biological systems?

  - Mixing will be uniform.

  - The collision between the cells, impeller, and air bubbles will lead to cell damage.

  - Increasing oxygen transfer efficiency

  - Consistent mixing

- Penetration theory assumes that turbulent eddies travel from the bulk of the phase to the interface where they remain constant for a constant exposure time (te). The model correlatingKL, mass transfer coefficient, and DAB, diffusivity can be expressed as

  - KL = 2(DAB/πte)0.25

  - KL = 2(DAB/πte)0.75

  - KL = 2(DAB/πte)0.5

  - KL = 2(DAB/πte)

- What is the ratio of the depth of liquid in a vessel to the diameter of a tank (H:Dt)?

  - 1:2

  - 1:1

  - 2:1

  - 4:1

- Transient flow range of mixing speed is obtained for

  - Rei<10

  - Rei>10

  - 10<Rei<104

  - Rei>104

- What is the ratio of the diameter of an impeller to the diameter of the tank (Da:Dt)?

  - 2:1

  - 4:1

  - 1:5

  - 1:3

- Which of the following is not considered among the three best-known theories of interphase mass transfer?

  - Calderbank and Moo-Young theory

  - two-film theory

  - penetration theory

  - surface renewal theory

- Gas hold-up, characterizing the hydrodynamics in a fermenter, mainly does not depend on the

  - superficial gas velocity

  - gas concentration

  - power consumption

  - Fermenter Geometry

- The volume of liquid (VL)in a cylindrical reactor can be calculated from the liquid height (HL) and tank diameter (Dt) using the following equation.

  - VL = 4/3 x π x HL x Dt3/8

  - VL=HL x π x Dt2

  - VL= 4 x π x Dt2

  - VL=HL x π x Dt2/4

- The average fractional gas hold up (H) can be given as (where Zf and Zl are the level of aerated and clear liquid respectively)

  - H = Zf/Zl

  - H = (Zf-Zl)/Zf

  - H = Zf/ZfZl

  - H =(Zf+Zl)/Zf

- Power consumption by agitation does not depend on:

  - a function of physical properties

  - a function of operating conditions

  - a function of vessel and impeller geometry

  - The type of organism used in the process

- For indirect heating, the plate and frame heat exchanger is generally more effective than the shell and tube heat transfer due to its

  - smaller heat transfer area

  - lesser cost

  - larger heat transfer area

  - more number of plates

- Model proposed by the Friedlander, correlating the air filtration efficiency by impaction can be expressed as (where NSt is the Stokes number)

  - ηimp = 0.075NSt1.2

  - ηimp = 0.75NSt1.2

  - ηimp = 0.25NSt1.2

  - ηimp = 0.025NSt1.2

- The reduction of the number of cells from 1010 to one will result in the del factor (Δ) of

  - 1

  - 23

  - 10

  - 103

- The collection efficiency by the inertial impaction mechanism is the function of

  - Stokes and Schmidt's number

  - Stokes and Reynolds number

  - Grashoff and Reynolds number

  - Stokes and Grashof's number

- The heated medium passes through a holding section, which is usually maintained in

  - isothermal conditions

  - isotropic conditions

  - isobaric conditions

  - adiabatic conditions

- The highest temperature that appears to be feasible for batch sterilization is

  - 121°C

  - 100°C

  - 105°C

  - 130°C

- Liquefaction of starch to dextrin is carried out by

  - Lipase

  - α-amylase

  - cellulase

  - pectinase

- Milk digestibility is improved by using

  - RNase

  - Sucrase

  - β-amylase

  - lactase

- Which one does not serve as to convert starch into high fructose corn syrup (HFCS)?

  - Maltase

  - α-amylase

  - Gluco-isomerase

  - Glucoamylase

- Catalysts discovered from biological sources are referred to as

  - Vitamins

  - enzymes

  - carbohydrates

  - nucleic acids

- Enzymes are the polymers of

  - amino acids

  - fatty acids

  - hexose carbon

  - organic phosphate

- What does the equation, k = Ae−ΔGRT represent?

  - Michaels Menton equation

  - Lineweaver Burk equation

  - Monod equation

  - Arrhenius equation

- Which is true about enzymes?

  - Make the equilibrium more favorable for the organism

  - Lower the energy of activation of a reaction

  - Lower the energy of the product and increase the energy of the reactant

  - Are altered permanently in the reaction they catalyze

- Most of the vitamins function as

  - Holoenzyme

  - Enzymes

  - Hormones

  - Coenzymes

- The del factor increases as the final number of cells

  - Constant

  - zero

  - Decrease

  - Increase

- The target substrate molecules bind to the active site of the enzyme transforming into products through a series of steps known as the ...................

  - Enzymatic mechanism

  - Enzyme kinetics

  - Chemical kinetics

  - Zero-order reaction kinetics

- The lock and key model was proposed by 	in 1890.

  - Henri

  - Michales and Menten

  - Daniel Koshland

  - Emil Fischer

- The study of rates of chemical reactions that are catalyzed by enzymes is referred to as

  - enzyme kinetics

  - first-order reaction kinetics

  - zero order reaction kinetics

  - chemical kinetics

- The time required to double the population (doubling time) can be expressed as

  - ln 2/D

  - ln 2/µ

  - µ/ln 2

  - µ

- The relationship between the del factor, temperature, and time is given as

  - ∇= A.t. e-E/RT

  - ∇ = 1/(A.t. e-E/RT)

  - ∇ = A.t. eE/RT

  - ∇ = A.t.T

- The overall del factor (Δ) may be represented as

  - ∇overall = ∇heating + ∇holding- ∇cooling

  - ∇overall = ∇heating - ∇holding+ ∇cooling

  - ∇overall = ∇heating - ∇holding- ∇cooling

  - ∇overall = ∇heating + ∇holding+ ∇cooling

- The efficiency of cell disruption in a bead mill does not depend on the

  - Concentration of cells

  - Color of beads

  - Type and rotational speed of agitation

  - Amount and size of beads

- The portion of the growth curve where rapid growth of bacteria is observed is known as

  - Logarithmic phase

  - Lag phase

  - Stationary phase

  - Decline phase

- The lag phase is also known as

  - transitional period

  - period of initial adjustment

  - generation time

  - period of rapid growth

- In which of the following phases secondary metabolites are produced during growth?

  - Lag phase

  - Log/Exponential phase

  - Stationary phase

  - Death phase

- Which phase shows the reproduction rate equal to the equivalent death rate?

  - Log phase

  - Death phase

  - Lag phase

  - Stationary phase

- The average size of the cells in the exponential phase is

  - larger than the initial size

  - equal to the initial size

  - smaller than the initial size

  - may be smaller or larger than the initial size

- What is the equation used to batch culture growth kinetics?

  - Monad’s equation

  - Michaels Menten equation

  - Hardy-Weinberg equation

  - Logistic population equation

- Identify the correct Monad’s equation.

  - μ=μmax+[S]/KS∗[S]

  - μ=μmax∗[S]/KS∗[S]

  - μ=μmax∗[S]/KS+[S]

  - μ=μmax+[S]/KS+[S]

- The process in which all living cells, spores, and viruses are completely destroyed from an object is called

  - Sterilization

  - Pasteurization

  - Disinfection

  - Antisepsis

- What do you mean by sterilization

  - Recovery of products

  - Elimination of contamination

  - Purification of products

  - Separation of products

- The long exposure of batch sterilization may lead to

  - Degradation

  - Recovery of media

  - Good quality of product

  - Complete elimination of contaminants

- The equation for the del factor is

  - ∇ = ln (No/Nt)

  - ∇ = ln (Nt/No)

  - ∇ = ln (No)

  - ∇ = ln (Nt)

- Scientists introduced the term ‘Del factor’ are:

  - Jacob and Monod

  - Michales and Menten

  - Banks and Corbett

  - Deindoerfer and Humphrey

- Which one of the following is the mechanical method of a sterilization process

  - Ultrasonication

  - Radiation

  - Heat

  - Filtration

- The most widely used means of sterilization is?

  - Ethylene Oxide sterilization

  - Heat sterilization

  - Plasma sterilization

  - Filtration

- Which sterilization technique is usually employed in the disruption of cells to extract intracellular constituents rather than as a sterilization technique?

  - Heat

  - Sonic or ultrasonic waves

  - Xrays

  - Ultraviolet

- Which one of the following is the most effective technique for air sterilization?

  - Fibrous or membrane filter

  - Moist heat

  - X-ray

  - Absolute filter

- In competitive inhibition, the inhibitor molecule binds to the	site of an enzyme.

  - Allosteric site

  - Active site

  - Cofactor

  - Apoenzyme

- Regarding enzymes, which one of the following statements is true?

  - Apoenzyme=Holoenzyme+Coenzyme

  - Holoenyme=Apoenzyme+Coenzyme

  - Holoenzyme=Coenzyme-Apoenzyme

  - Coenzyme=Apoenzyme+Holoenzyme

- Which of the following is not true for non-competitive inhibition?

  - Binds to the allosteric site

  - Inhibits the enzyme activity

  - Binds to the active site

  - Binds noncovalently to the enzyme

- The purpose of aeration is to provide

  - The oxygen to organisms

  - The medium to organisms

  - The carbon dioxide in organisms

  - The water to organisms

- The Aeration is mainly provided to organisms present in

  - Solid State culture

  - Surface culture

  - Batch culture

  - Submerged culture

- The agitator is required to

  - Mixing objectives

  - Provide air

  - Purify the product

  - Sterilize the media

- What part of a bioreactor helps in maintaining the homogeneity of the contents?

  - Sparger

  - Baffles

  - Agitator

  - Thermometer

| Course Name: Introduction to chemical engineering | Course Code: ChEg2082 |
| --- | --- |

- If the measured mass flow rate of methane entering a continuous process unit is greater than the mass flow rate leaving (m_in > m_out), which of the following is a possible explanation?

  - Methane is being consumed as reactant.

  - Methane is leaking from the unit.

  - Methane is accumulating in the unit.

  - All of the above.

- If the process unit is operating at steady state and there is no chemical reaction involving methane in continuous process, and yet the measured input and output flow rates of methane are different (m_in ≠ m_out), what is the most likely cause?

  - Methane is being consumed as a reactant.

  - Methane is being generated as a product.

  - There is a leak in the unit or the measurements are wrong.

  - The process is not actually at steady state.

- A student measures a length to be 3.5 meters. What is the implied uncertainty in the measurement?

  - ± 0.1 meters

  - ± 0.05 meters

  - ± 0.5 meters

  - ± 0.01 meter

- Which of the following is the correct relationship between absolute pressure (P_abs) and gauge pressure (P_gauge)?

  - P_abs = P_gauge - Atmospheric Pressure

  - P_abs = P_gauge + Atmospheric Pressure

  - P_gauge = P_abs × Atmospheric Pressure

  - P_gauge = P_abs / Atmospheric Pressure

- A gas stream contains 40% nitrogen and 60% oxygen by mole. What is the mole fraction of nitrogen?

  - 0.6

  - 0.4

  - 40

  - 60

- Which of the following equations represents the ideal gas law?

  - PV = nRT

  - P/V = nRT

  - PV = R/nT

  - Pn = VRT

- Specific gravity is the ratio of a substance's density to the density of what standard substance (usually)?

  - Air

  - Mercury

  - Water

  - Iron

- What is the principle of dimensional homogeneity?

  - All equations must have the same numerical value.

  - All terms in an equation must have the same units.

  - All calculations must be performed in SI units.

  - All conversion factors must be dimensionless.

- In introduction to the chemical engineering course, what does the term 'mass balance' refer to?

  - The amount of energy consumed

  - The relationship between input and output material

  - The balance between products and reactants

  - None

- What does 'stoichiometry' refer to in chemical engineering?

  - The study of heat transfer

  - The quantitative relationship between reactants and products

  - The measurement of gases or liquids

  - The analysis of impurities

- What happens to the volume of a given mass of gas when it is cooled?

  - It decreases

  - It increases

  - It remains the same

  - It becomes a solid

- What is the main purpose of a material balance?

  - To calculate energy losses

  - To ensure the conservation of mass

  - To assess the environmental impact

  - To analyze financial costs

- In steady-state operation, what can be said about the accumulation term?

  - It is always zero

  - It is always positive

  - It can be negative

  - It varies with time

- In a continuous process, what happens to the system when the inflow and outflow rates are equal?

  - The system goes to shutdown

  - The system accumulates material

  - The system reaches steady-state

  - The system becomes unstable

- For a non-reactive process, what is the general material balance equation?

  - Input = Output + Generation - Consumption

  - Input + Generation = Output + Consumption

  - Input - Output = Generation

  - Input - Output = Accumulation

- Which of the following equations represents a mass balance for a chemical reactor?

  - Reactant In + Generation = Reactant Out + Consumption + Accumulation

  - Input + Output = Accumulation + Generation – Consumption

  - Input - Output = Generation + Consumption

  - Input + Consumption = Output + Accumulation

- What role do “stoichiometric coefficients” play in chemical reaction equations within a material balance?

  - They determine temperature changes

  - They indicate volume changes

  - They define the ratios of reactants and products

  - They affect pressure conditions

- How do you calculate the mass of a component in a mixture if the total mass is known?

  - By multiplying the total mass with the density

  - By using the percentage composition of the component

  - By adding the masses of all other components

  - By subtracting the mass of unwanted material

- In a steady-state process, what is true about the accumulation of mass within the system?

  - Accumulation is positive.

  - Accumulation is negative.

  - Accumulation is zero.

  - Accumulation fluctuates randomly.

- Which of the following is the general balance equation for a conserved quantity?

  - Input + Generation = Output + Consumption + Accumulation

  - Input - Generation = Output - Consumption - Accumulation

  - Input + Output = Generation + Consumption + Accumulation

  - Input + Consumption = Output + Generation + Accumulation

- The degrees of freedom (DOF) in a material balance problem are calculated as:

  - DOF = Number of unknown variables + Number of independent equations

  - DOF = Number of unknown variables - Number of independent equations

  - DOF = Number of independent equations - Number of unknown variables

  - DOF = Number of unknown variables × Number of independent equations

- What is a basis of calculation used for in material balance problems?

  - To complicate the calculations.

  - To simplify the calculations by fixing a specific quantity.

  - To eliminate all unknown variables.

  - To increase the number of equations.

- In a continuous steady-state process, the mass flow rate of the input is:

  - Always greater than the output.

  - Always less than the output.

  - Equal to the mass flow rate of the output.

  - Unrelated to the output.

- What is a tie component in a material balance problem?

  - A component that reacts completely.

  - A component that enters and leaves the process without reacting or separating.

  - A component that is generated in the process.

  - A component that is consumed in the process.

- When performing a material balance on a reactive system, what must be considered in addition to mass flows?

  - Temperature changes only.

  - Pressure changes only.

  - Chemical reaction stoichiometry.

  - Color changes.

- What is the purpose of drawing a process flowchart in a material balance problem?

  - To make the problem more difficult.

  - To visually represent the process and identify streams.

  - To calculate the degrees of freedom directly.

  - To eliminate the need for material balances.

- In a mixing process, if two streams with different compositions are mixed, what is true about the overall mass balance?

  - The total mass of the output is less than the sum of the inputs.

  - The total mass of the output is greater than the sum of the inputs.

  - The total mass of the output is equal to the sum of the inputs.

  - The total mass of the output is unrelated to the sum of the inputs.

- What is a combustion reaction?

  - A reaction where two elements combine

  - A reaction that releases energy in the form of heat and light

  - A reaction that only occurs in the presence of oxygen

  - A reaction that produces only solid products

- Incomplete combustion of a hydrocarbon mainly produces:

  - Carbon dioxide

  - Carbon monoxide

  - Methane

  - Hydrogen

- What is the typical role of oxygen in a combustion reaction?

  - It serves as a catalyst

  - It acts as a reactant

  - It accelerates the reaction

  - It inhibits the reaction

- In the combustion of propane (C₃H₈), what are the products when it combusts completely?

  - CO and H₂O

  - CO₂ and H₂O

  - C and O₂

  - CH₄ and O₂

- What does "wet basis" refer to in moisture content analysis?

  - The moisture content is expressed without considering the water content.

  - The moisture content is considered as a fraction of the total weight.

  - The moisture content is considered only in solid materials.

  - The moisture content is expressed as a fraction of the dry weight.

- What does the term "dry basis" refer to in moisture content analysis?

  - Measurement of moisture in liquid form

  - Measurement of moisture relative to the solid content

  - Measurement of total weight including moisture

  - Measurement without considering the solid content

- If a material weighs 100 g wet, and 20 g of that weight is water, what is its moisture content on a wet basis?

  - 20%

  - 80%

  - 100%

  - 5%

- If a material has a moisture content of 30% on a wet basis, what would be the dry basis

moisture content if the dry weight is 70 g?

  - 21.25%

  - 30.75%

  - 42.85%

  - 100%

- What is the theoretical oxygen requirement in combustion?

  - The exact amount of oxygen needed for complete combustion of a fuel.

  - The maximum amount of fuel that can be burned with available oxygen.

  - The sum of all oxygen used in combustion reactions.

  - The amount of oxygen required for incomplete combustion.

- When discussing combustion, what does "excess oxygen" refer to?

  - Oxygen that is not consumed in the reaction.

  - Additional oxygen supplied beyond the theoretical requirement.

  - Oxygen found in the surrounding environment.

  - Oxygen present in flue gases.

- The stoichiometric ratio in combustion represents:

  - The ratio of fuel to air

  - The optimal ratio for complete combustion

  - The ratio of combustion products to reactants

  - The excess air ratio required for combustion

- In combustion calculations, air is commonly approximated as having:

  - 21% oxygen and 79% nitrogen

  - 25% oxygen and 75% nitrogen

  - 50% oxygen and 50% nitrogen

  - 15% oxygen and 85% nitrogen

- Which form of energy that can be converted to mechanical work completely and directly by an ideal mechanical device such as an ideal turbine.

  - potential energy, kinetic energy, and Flow (pressure) energy

  - Thermal energy, Work, and Heat

  - Internal Energy, Work, and Heat

  - Pressure energy, Work, and Heat

- One of the following pair of energy can’t be stored or contained within the system

  - potential energy, and kinetic energy

  - Work, and Heat

  - Pressure energy, and Work

  - kinetic energy, and pressure energy

- Mass is neither created nor destroyed except in

  - Water flows in a pipe

  - Conversion of reactant to product

  - A woman gives twins (two babies)

  - nuclear fission and fusion

- The form of energy associated with the motion of molecules, atoms, particles and sub atomic particles are called

  - Enthalpy

  - Potential energy

  - Internal energy

  - work and heat

- What is the Energy balance for closed system

  - ∆ 𝐻 + ∆𝐾𝐸 + ∆𝑃𝐸 = 𝑊 + 𝑄

  - ∆ 𝑈 + ∆𝐾𝐸 + ∆𝑃𝐸 = 𝑊 + 𝑄

  - ∆ 𝑊𝑠 + ∆𝐾𝐸 + ∆𝑃𝐸 = 𝑊 + 𝑄

  - ∆ 𝑊𝑓 + ∆𝐾𝐸 + ∆𝑃𝐸 = 𝑊 + 𝑄

- Energy Balances on Open Systems at Steady State is

  - ∆ 𝐻 + ∆𝐾𝐸 + ∆𝑃𝐸 = 𝑄 − 𝑊𝑠

  - ∆ 𝑈 + ∆(𝑃𝑉) + ∆𝐾𝐸 + ∆𝑃𝐸 = 𝑄 − 𝑊𝑠

  - ∆ 𝐻 = 0

  - 𝐴𝑙𝑙 𝑒𝑥𝑐𝑒𝑝𝑡 𝐶

- Rate of work done by the process fluid on a moving part within the system

  - Shaft work

  - Flow work

  - Frictional force

  - Enthalpy

- Rate of work done by fluid at the system outlet minus the rate of work done on the fluid at the system inlet is

  - Shaft work

  - Flow work

  - Frictional force

  - Enthalpy

- In a batch process, mass crosses the system boundaries:

  - Continuously.

  - Only at the beginning and end.

  - At random intervals.

  - At a steady rate.

- Which of the following is an example of a continuous process?

  - Adding reactants to a tank and removing products after equilibrium.

  - Pumping liquids into a distillation column at a constant rate.

  - Allowing gas to escape from a pressurized container.

  - Slowly blending liquids in a tank with no outflow.

- A semi-batch process is characterized by:

  - Inputs and outputs flowing continuously.

  - No mass crossing system boundaries after initial charging.

  - Some mass crossing the boundaries, but not continuously.

  - Operating only at steady state.

- A process is considered to be at steady state when:

  - All variables change significantly with time.

  - All variables remain constant with time, but not necessarily with position

  - All variables remain constant with position, but not necessarily with time

  - Only the flow rates change.

- Which of the following processes is inherently unsteady-state (transient)?

  - Continuous processes.

  - Batch and semi-batch processes.

  - Steady-state continuous processes.

  - All processes.

- Continuous processes are generally preferred for:

  - Small production quantities.

  - Large production rates.

  - Highly variable product specifications.

  - Processes requiring frequent changes.

- A balloon being filled with air at a steady rate of 2 g/min is a:

  - Batch process and steady-state.

  - Semi-batch process and transient.

  - Continuous process and steady-state.

  - Batch process and transient.

- A bottle of milk taken from a refrigerator and left on the kitchen table is a:

  - Continuous process and steady-state.

  - Batch process and transient.

  - Semi-batch process and transient.

  - Continuous process and transient.

- Water boiled in an open flask is a:

  - Batch process and steady-state.

  - Semi-batch process and transient.

  - Continuous process and steady-state.

  - Batch process and transient.

- Carbon monoxide and steam reacting in a tubular reactor, initially containing air, is, initially ------and after a long period is	--.

  - steady state, transient

  - transient, steady state

  - batch, continuous

  - continuous, batch

- Why are batch and semi-batch processes inherently unsteady-state?

  - Because they operate at constant temperatures.

  - Because their process variables change with time.

  - Because they involve continuous flow.

  - Because they are always at equilibrium.

- Under what conditions does the balance equation for "Total moles" in a continuous process take the simple form "input = output"?

  - Only at steady state.

  - Only when there is no change in volume.

  - At steady state and when there is no chemical reaction that changes the total number of moles.

  - Always, regardless of reaction or state.

- Why is it generally recommended to convert volumes to masses or moles before writing material balances?

  - Because volume is not a conserved quantity in most chemical processes.

  - Because volume is difficult to measure accurately.

  - Because volume changes are always negligible.

  - Because volume is only relevant in batch processes.

- For a continuous, steady-state process, when does the balance equation for "Mass of species A" simplify to "input = output"?

  - When species A is reactive.

  - When species A is nonreactive.

  - When there is a change in total moles.

  - When there is a change in total volume.

- If a continuous process is not at steady state, what must be included in the balance equation for total mass?

  - Only the generation term.

  - Only the consumption term.

  - The accumulation term.

  - Nothing, the equation remains "input = output".

- When does the balance equation for "Total mass" in a continuous process take the simple form "input = output"?

  - When the process is transient.

  - When the process is at steady state.

  - When there is a chemical reaction.

  - When there is a change in volume.

- Under which condition is the balance equation "input = output" valid for total moles in a continuous, steady-state process?

  - When a reaction produces more moles than it consumes.

  - When a reaction consumes more moles than it produces.

  - When there is no chemical reaction, or the reaction does not change the total number of moles.

  - When there is a change in volume.

- Why is it generally more reliable to base material balances on mass or moles rather than volume?

  - Because volume is always constant.

  - Because mass and moles are conserved in non-nuclear chemical processes.

  - Because volume is easier to measure.

  - Because volume is always directly proportional to mass.

Course Name: Mass Transfer Unit Operation                          Course code: ChEng4101

- Fick's first law of diffusion describes steady-state diffusion. Which of the following scenarios would best exemplify this condition?

a) Diffusion occurs in a vessel vigorously mixed by the impeller.

b) Diffusion occurring during the start-up phase of a continuous distillation column.

c) Diffusion across a membrane with a constant concentration difference maintained across it over a long period.

d) Diffusion taking place as a result of a non-isothermal process.

- A mass transfer operation involves the transfer of solute A from a liquid phase to a gas phase. The individual mass transfer coefficients are kLa (liquid-side) and kGa (gas-side), and the equilibrium relationship is y = mx, where y is the gas-phase mole fraction in equilibrium with liquid-phase mole fraction x. Which of the following changes would be MOST likely to make the process gas-side mass transfer controlled?

a) Increasing the value of m significantly.  c) Increasing the total system pressure.

b) Decreasing the value of m significantly.   d) Decreasing both kLa and kGa proportionally.

- What is the physical significance of the Sherwood number (Sh) in mass transfer operations?

a)  It's the ratio of thermal diffusivity to momentum diffusivity.

b)  It represents the ratio of convective to conductive heat transfer.

c)  It indicates the relative importance of convective to diffusive mass transfer.

d) It indicates the effectiveness of fluid flow.

- The two-film theory is often used to explain mass transfer at an interface. What is the key assumption of this theory regarding the concentration profile near the interface?

a) The concentration changes smoothly and linearly across the entire film.

b) There's no convection; concentration gradients occur throughout the entire bulk, away from the interface

c) The primary resistance to mass transfer resides within thin, stagnant films on either side of the interface.

d) Turbulence intensity is minimized at each interface side of the mass transfer operation.

- The extraction factor (E) is defined as E = K(S/F), where K is the distribution coefficient, S is the solvent flow rate, and F is the feed flow rate. For effective extraction in a countercurrent multistage system, the extraction factor should ideally be:

a) Much less than 1               b) Approximately equal to 1         c) Greater than 1      d) Equal to 0

- Which dimensionless number is most directly related to the internal mass transfer resistance within the solid particle during leaching?

a) Biot number (Bi) for mass transfer                      c) Reynolds number (Re)

b) Sherwood number (Sh)                                        d) Peclet number (Pe)

- Absorption and adsorption are two common separation techniques. Which property difference is exploited in adsorption?

a) Solubility differences of components in a liquid solvent.

b) Differences in volatility or boiling point of components.

c) Preferential adherence of components to a solid surface.

d) Size differences between components, allowing selective passage through a membrane.

- In a separation process that involves phase addition, what is the primary purpose of introducing a new phase?

a) To increase the overall pressure of the system.

b) To selectively transfer one or more components from the original mixture to the added phase.

c) To chemically react with one or more components of the original mixture, converting them into new compounds.

d) To act as a catalyst, speeding up the separation of the components of a single-phase system.

- Liquid-liquid extraction utilizes what phenomenon as a means of separation?

a) Relative solubilities     b) Relative surface tension      c) Density.      d) Electrical Potential

- Which separation technique relies on phase creation to achieve separation, involving a change in the thermodynamic state?

a) Adsorption              b) Filtration                c) Crystallization              d) Membrane separation

- Reactive Extraction can be used on systems with the following attributes?

a) Systems containing polymers that crosslink during reactions.

b) Systems undergoing photochemical reactions due to exposure to light.

c) Systems that react in situ while simultaneously separating, commonly exhibiting reaction reversibility.

d) No specific system-related characteristic is required for reactive extraction.

- A solid material dissolved in a solution is intended to be separated using a seed in crystallization. Which phenomena will increase separation yield for target component?
a) Solubility of the target is constant throughout the cooling and crystallization.
b) Target component has high molecular weight to prevent adherence to nucleated seeds.
c) Target is at supersaturation throughout a certain duration.
d) Inhibit heterogeneous and Homogenous nucleation

- The "plait point" on a ternary phase diagram for a liquid- liquid extraction system represents...

a) The point of maximum selectivity for the solute.
b) The composition where the extract and raffinate phases become identical.
c) The optimal operating point for a single-stage extraction.
d) The point where the distribution coefficient is equal to 1.

- What is the primary driving force behind the separation of components in a simple distillation process?

a)  Differences in density                              c) Differences in solubility

b)  Differences in boiling points                   d) Differences in molecular size

- Which of the following statements is true regarding fractional distillation compared to simple distillation?

a) Fractional distillation is less effective at separating components with similar boiling points.

b) Fractional distillation employs a shorter distillation column.

c) Simple distillation is typically preferred for mixtures with significant boiling point differences.

d) Simple distillation produces a purer distillate when boiling points are very close.

- What is the purpose of adding boiling chips to the distillation flask?

a)  To increase the boiling point of the mixture.           c)  To catalyze the separation process.

b)  To prevent bumping and promote even boiling.      d)  To change the color of the distillate.

- A mixture of ethanol (boiling point 78°C) and water (boiling point 100°C) is being distilled. Which component will be collected first in the receiving flask?

a)  Water                                  c) A mixture of both in equal proportions

b)  Ethanol                               d) It depends on the initial concentration of the mixture.

- The Kremser equation is a useful tool for analyzing multi-stage liquid-liquid extraction. Which of the following assumptions is NOT typically made when applying the Kremser equation?

a) Constant distribution coefficient (K)

b) Immiscible solvent and diluent

c) Equilibrium is reached in each stage

d) The operating line is curved

- What is an azeotrope?

a)  A mixture that boils at a constant temperature and composition, behaving like a pure substance.

b)  A mixture that separates easily upon distillation.

c)  A type of distillation apparatus.

d)  A substance that is added to prevent bumping.

- Vacuum distillation is used to:

a)  Increase the boiling points of the components.

b)  Decrease the boiling points of the components.

c)  Separate components with very similar boiling points.

d)  Speed up the distillation process without affecting boiling points.

- Which of the following is not a common application of distillation?

a)  Purification of water

b)  Separation of crude oil into fractions

c)  Production of alcoholic beverages

d)  Separation of solid pigments from a paint mixture

- A mixture contains three components: A (BP: 50°C), B (BP: 100°C), and C (BP: 150°C, decomposes at 120°C). You need to isolate pure C. Which distillation method(s) and strategy would be most appropriate, and why?

a)  Simple distillation, because the boiling points are far apart.

b)  Fractional distillation, to get the best possible separation between A and B.

c)  Vacuum distillation, performed at a reduced pressure that keeps C below its decomposition temperature, followed by (if necessary) fractional distillation to remove any remaining A or B.

d)  Steam distillation, because C is the highest-boiling component.

- Which of the following best describes the primary purpose of an absorption column?
a) To separate components of a liquid mixture based on boiling point differences.
b) To remove a specific component from a gas stream by dissolving it in a liquid solvent.
c) To react two different gas streams to form a desired product.
d) To increase the pressure of a gas stream.

- Henry's Law constant (H) is a measure of:
a) The viscosity of the liquid solvent.             c) The solubility of a gas in a liquid.
b) The flow rate of the gas stream                 d) The height of the absorption column.

- In liquid-liquid extraction, "back-mixing" refers to...

a) The intentional recycling of a portion of the extract stream.
b) The undesirable axial mixing of either or both phases within the contractor.
c) The remixing of separated phases due to poor settler design.
d) The reverse transfer of solute from the extract back to the raffinate.

- A gas stream containing 2% (mole fraction) of a solute is contacted with a pure liquid solvent in an absorption column. If Henry's Law constant (H) for this system is 0.5, what is the equilibrium liquid phase mole fraction (x) at the interface in contact with the bulk gas?
a) 0.01                      b) 0.04                            c) 0.02                    d) 0.1

- The driving force for mass transfer in an absorption column is the:
a) Difference in temperature between the gas and liquid streams.
b) Difference in pressure between the top and bottom of the column.
c) Difference between the actual concentration and the equilibrium concentration of the solute.
d) Viscosity of the liquid solvent.

- Which of the following packing materials would typically provide the highest surface area per unit volume in an absorption column?
a) Raschig rings                b) Berl saddles            c) Structured packing       d) Pall rings

- You are investigating a new solvent for a liquid- liquid extraction process. Experimental data shows that the interfacial tension between the new solvent and the feed is significantly higher than with the current solvent. What potential operational problem might this higher interfacial tension cause, and how might you mitigate it?

a) Increased emulsification; use a lower agitation speed.

b) Increased flooding; increase the column diameter.

c) Poorer phase separation; use a coalescer or increase settling time.

d) Lower mass transfer rates; increase the interfacial area by using smaller packing.

- A countercurrent absorption column is used to remove 90% of a solute from a gas stream. The inlet gas contains 0.05 mol fraction of solute. The liquid enters pure. The exit gas mole fraction is closest to:
a) 0.045                    b) 0.005                c) 0.0005                d) 0.05

- If the operating line and equilibrium curve touch at any point in an absorption column, this indicates:
a) The column is operating at maximum efficiency.
b) The minimum liquid-to-gas ratio is being used.
c) The column is flooding.
d) The solute is completely insoluble in the solvent.

- The Height of a Transfer Unit (HTU) represents:
a) The total height of the packed bed.
b) The height of packing required to achieve a concentration change equal to the average driving force.
c) The diameter of the absorption column.
d) The volumetric flow rate of the gas stream.

- Which of the following statements is generally TRUE regarding the effect of agitation in immersion leaching?

a) Agitation primarily increases the solubility of the solute.

b) Agitation reduces the thickness of the liquid film surrounding the solid particles, enhancing mass transfer.

c) Agitation does not affect the rate of extraction if the particles are very small.

d) Agitation only affects the rate of extraction if the solid is soluble in the solvent.

- Flooding in an absorption column is characterized by:
a) A decrease in pressure drop across the column.
b) Liquid being carried out of the top of the column by the gas stream.
c) A decrease in mass transfer efficiency.
d) Both b and c

- Find the relative volatility if x and y are the concentrations in mole fraction.
                          
a. 6.92                      b. 0.14                         c. 1.88                     d. None of the mentioned

- A solid material initially weighs 120 kg and contains 20 kg of water. What is the moisture content on a dry basis?

a) 16.67%                               b) 20%                           c) 25%                       d) 83.33%

- Increasing the solvent-to-feed ratio in a single-stage liquid-liquid extraction will generally...

a) Decrease the percentage extraction of the solute.

b) Increase the percentage extraction of the solute, but with diminishing returns.

c) Have no effect on the percentage extraction of the solute.

d) Increase the concentration of the solute in the raffinate

- The equilibrium moisture content (X) of a solid is:

a) The maximum moisture content the solid can hold.

b) The moisture content at which the drying rate becomes zero.

c) The moisture content at which the drying rate transitions from constant to falling.

d) Always zero.

- The critical moisture content marks the transition between:

a) The unbound moisture and bound moisture.

b) The constant rate period and the falling rate period.

c) The heating-up period and the constant rate period.

d) The wet basis and dry basis moisture content.

- In the falling rate period of drying, the drying rate is generally assumed to be:

a) Constant.

b) Proportional to the free moisture content.

c) Inversely proportional to the free moisture content.          d) Zero.

- A solid with a dry mass of 5 kg is dried from an initial moisture content of 0.4 (dry basis) to a final moisture content of 0.1 (dry basis). The total mass of water removed is:

a) 1.5 kg                                 b) 2.0 kg                         c) 0.5 kg                             d) 2.5 kg

- A system exhibits a type II ternary phase diagram. Which of the following is true?

a) The diluent and solvent are completely miscible.
b) Two pairs of components are partially miscible.
c) Only one pair of components is partially miscible.
d) All three components are completely immiscible.

- The capacity of the rotary drier depends on

a)  R.p.m                 b) Inclination with ground surface      c) Both a & b      d) Neither a nor b

- The "equilibrium" stage in solid-liquid extraction, as represented on a triangular diagram, assumes:

a) The solute concentration in the solid is uniform throughout the particle.

b) The solute concentration in the overflow is equal to the concentration in the entrained liquid in the underflow.

c) The solvent completely penetrates the solid particles.

d) The rate of dissolution is infinitely fast

- Which of the following factors does not typically affect the drying rate during the constant rate period?

a) Air temperature     b) Air humidity         c) Air velocity       d) Internal diffusion resistance of the solid

- Bound moisture in a solid is:

a) Easier to remove than unbound moisture.

b) Moisture that exerts an equilibrium vapor pressure lower than that of pure water at the same temperature.

c) Moisture that is present on the surface of the solid.

d) Moisture that can be removed during the constant rate period.

- During the falling rate period, we can approximate the drying rate as directly proportional to the free moisture content, knowing that the critical free moisture content (Xc - X *) is 0.2 kg water/kg dry solid. If the drying rate at the critical moisture is Rc, what is the drying rate when the free moisture content (X - X *) is 0.05 kg water/kg dry solid?

a) 4Rc                         b) Rc/4                      c) Rc                        d) 2Rc

- Which of the following factors has the least direct impact on the solid-liquid extraction rate?

a) Particle size of the solid

b) Solvent viscosity

c) Density difference between the solid and the solvent

d) Solubility of the solute in the solvent

- According to the Fenske equation, what will be the minimum number of plates required in adistillation column to separate an equimolar binary mixture of components A and B into an overhead fraction containing 99 mol% A and a bottom fraction containing 98 mol% B ? Assume that relative volatility (αAB = 2) does not change appreciably in the column.

a) 5                          b) 9                                c) 12                          d) 28

Course Name: Mechanical Unit Operations

- For any non-spherical particle, Sphericity is defined as

  - 6Vp/DpPs

  - 6Vp/PsSp

  - Vp/DpSp

  - 6Vp/DpSp

- Suppose we have a uniform particle of crushed glass with a diameter of 20mm which was put in a hoper. If the total mass of the particle is 2000kg what will be the number of particles in the sample. (Take ρp = 2400kg/m3 )

  - 190,000

  - 198,944

  - 100,244

  - 188,556

- Which of the following is a type of Mechanical Operations generally used in industries?

  - Size reduction

  - Filtration

  - Screening

  - All of the mentioned

- The inlet pressure in a constant rate filtration

  - Increases continuously

  - Decreases gradually

  - Remains constant

  - None

- Which of the following works on the principle of compression and impact?

  - Fine crusher

  - Jaw crusher

  - Disk mill

  - Gyratory crusher

- Which of the following works on the principle of impact?

  - Gyratory Mill

  - Jaw crusher

  - Ball mill

  - Roll crusher

- Rittingers law is applicable only when

  - New particles are created

  - New surface area is created

  - Energy is created

  - Energy is absorbed

- The volume surface mean diameter can be defined as 	.

  - 6/ΦsAwPp

  - 6/ΦsAwVp

  - 6/ΦsAwDp

  - 6/ΦsAwSp

- Which of the following are used for the separation of coarse particles from slurry of fine particles?

- Thickeners

- Classifiers

- Hydro cyclones

- Decanters

- Which of the following filter may prove unsuitable for filtering volatile liquid?

  - Vacuum

  - Centrifugal

  - Gravity

  - Pressure

- Which of the following device used for mixing, granulation and drying purpose?

  - Fluidized bed granulator

  - Mixer Granulator

  - Tumbling Granulator

  - All

- What does the following equation represent?

  - Arrhenius Law

  - Kick’s Law

  - Bond Law

  - Rittingers Law

- Which one is true about Bond crushing law?

- Calls for relatively less energy for the smaller product particles, than does the Rittinger law

- Is less realistic in estimating the power requirements of commercial crushers

- States that the work required to form particle of any size from very large feed is proportional to the square root of the volume to surface ratio of the product

- States that the work required for the crushing is proportional to the new surface created

- For a ball mill, what will happen to product if the weight of ball increases?

  - Coarser

  - Finer

  - Bigger

  - No change

- Which of the following explains Mechanical Operations correctly?

  - Deals with Science and Technology of particulate matter

  - Deals with Science and Technology of machine operations

  - Deals with Science, Technology of particulate matter and machine operations

  - None of the mentioned

- Which of the following is not categorized as a "mechanical operation”?

  - Agitation

  - Filtration

  - Humidification

  - Size enlargement

- Equivalent diameter of a particle is the diameter of the sphere having the same

- Ratio of surface to volume as the actual volume.

- Ratio of volume to surface as the particle.

- Volume as the particle.

- None of the above.

- Which of the following operation uses gravity for operation?

  - Decantation

  - Sedimentation

  - Settling

  - All the mentioned

- Which of the mentioned is not come under size reduction equipment?

  - Ball mill

  - Roll crusher

  - Jaw Crusher

  - Hoper

- The filtrate flow rate in constant pressure filtration

  - Continuously increases

  - Continuously decreases

  - Constant throughout

  - No effect

- During filtration operation the filtrate mainly encounters the resistance of the,

  - Filter medium

  - Cake

  - Channel carrying the slurry to the upstream side of the cake and filtrate away from the filter medium

  - all

- Ultrafine grinders operate principally by

  - Slow compression

  - Impact

  - Attrition

  - Cutting action

- For the following particulate material distribution what will be the volume surface mean diameter and the mass mean diameter respectively?

| Size range in µm | -704+352 | -352+176 | -176+88 | -88+44 | Pan |
| --- | --- | --- | --- | --- | --- |
| Mass in gm | 25 | 37.5 | 62.5 | 75 | 50 |

  - 20.8 & 70.2 µm

  - 50.4 & 120.6 µm

  - 61.4 & 149.1 µm

  - 55.7 & 134.5 µm

- As the size of the ball mill increase, the size of product will

  - Remains same

  - Decreases

  - Increases

  - No effect

- Rittinger number which designates the new surface produced per unit of mechanical energy absorbed by the material being crushed, depends on the

  - State or manner of application of the crushing force

  - Ultimate strength of the material

  - Elastic constant of the material

  - all

- The most suitable filter for the removal of very small amount of precipitate from very large volumes of water is the 	filter.

  - Vacuum

  - Sand

  - Plate & frame

  - Rotary

- Wet milling (fluid energy) is used for

  - Cutting

  - Grinding

  - Ultra-grinding

  - Crushing

- Which of the following is the most suitable filter for separation of abrasive solids suspended in a corrosive liquid?

  - Sand bed filter

  - Plate and frame filter press

  - Vacuum filter

  - Centrifugal filter

- The specific cake resistance for incompressible sludges is (where ΔP = pressure drop over cake)

  - Proportional to ΔP

  - Proportional to 1/ ΔP

  - Proportional to	√ΔP

  - Independent of ΔP

- Gyratory crushers compared to the reciprocating jaw crushers

  - Have greater capacity per unit of discharge area

  - Crush intermittently

  - Are less suitable for coarse materials

  - Have less steady power consumption

- General mechanism of size reduction in intermediate and fine grinder is by

  - Cutting action

  - Compression

  - Compression and cutting

  - Impact and attrition

- Filtration rate does not depend upon the

  - Pressure drop & area of filtering surface

  - Resistance of the cake & the septum

  - Properties of the cake & the filtrate

  - None of the above

- During washing of filter at the end of constant pressure filtration, the rate of washing equals the rate of filtration

  - At time zero

  - At the end of filtration

  - When half the filtrate has been obtained

  - At the end of filtration, but decreases with time subsequently

- Cyclones are used primarily for separating

  - Solids from solids

  - Solids from gas

  - Solid from liquids

  - Liquids from liquids

- Removal of activated carbon from glycerin is done by

  - Plate and frame filter

  - Rotary vacuum filter

  - Centrifugal filter

  - None

- Grindability of a material does not depend upon its

  - Elasticity

  - Hardness

  - Toughness

  - Size

- Which one of the following device is used for the transportation of ultrafine particles with a complete enclosure?

  - Screw conveyor

  - Pneumatic conveyor

  - Belt conveyor

  - None

- Size reduction mechanism used in Jaw crushers is

  - Attrition

  - Compression

  - Cutting

  - Impact

- Filtration operation carried out by continuous increase of the inlet pressure of slurry is called

  - Constant rate filtration

  - Varying pressure filtration

  - Varying rate filtration

  - Constant pressure filtration

- Which one of the following conveyor is used to transport dry powdery solid materials?

  - Belt conveyor

  - Bucket elevator

  - Screw conveyor

  - None

- In case of a hammer crusher, the final product size depends on the

  - Feed rate

  - Rotor speed

  - Clearance between hammer & grinding plates

  - All of the above

- Which one of the following solid storage device used to store solids for short duration of time?

  - Bin

  - Silo

  - Hoper

  - None

- Which one of the following reduction mechanism used for coarse reduction of hard solids?

  - Impact

  - Attrition

  - Compression

  - Cutting

- During size reduction the ‘work index’ is defined as the

  - Gross energy (kWh/ton of feed) needed to reduce very large feed to such a size that 80% of the product passes through a 100 micron screen

  - Energy needed to crush one ton of feed to 200 microns

  - Energy (kWh/ton of feed) needed to crush small feed to such a size that 80% of the product passes a 200 mesh screen

  - Energy needed to crush one ton of feed to 100 microns

- Reduction ratio of crushers is the

  - Ratio of feed opening to discharge opening

  - Ratio of discharge opening to feed opening

  - Determining factor for minimum diameter of the feed and the product

  - None

- During size enlargement operation the magnitude of these inter-particle forces is governed by

  - The size of the particles

  - The structure of the granule

  - The moisture content

  - The surface tension of the liquid

  - All of the above

- The process by which fine solids is removed from liquids is termed as

  - Decantation

  - Flocculation

  - Sedimentation

  - Classification

- Which of the following crushing laws is most accurately applicable to the fine grinding of materials?

  - Bond's crushing law

  - Kick's law

  - Rittinger's law

  - None of the above

- The energy consumed by a ball mill depends on

  - Its speed

  - Its ball load

  - The density of the material being ground

  - All

- Sphericity is the ratio of the surface area of a spherical particle having the same volume as the particle to the surface area of the particle. Which of the following has the maximum value of sphericity?

  - Sphere

  - Cube

  - Cylinder

  - Circle

- Which one of the following is the aim of size enlargement operations in the process industries?

  - Improving handling and flow ability

  - Reducing dusting or material losses

  - Producing structural useful forms

  - provide a defined metered quantity of active ingredient

  - all of the above

- Tumble or growth agglomeration can be provided by

  - Inclined discs

  - Rotating drums

  - Fluidized beds

  - All of the above

- In which fluidization zone particle flux and suspension concentration are constant with height?

  - Splash zone

  - Disengagement zone

  - Dilute-phase transport zone

  - Freeboard zone

- By which approach can we estimate the pressure drop per unit length of the bed during packed bed column, if the Reynolds number of the flow (Re > 1000)?

  - Carman-Kozeny approach

  - Burke Plummer approach

  - Ergun approach

  - All

- Which one of the following is the factor that can affect the rate of sedimentation process?

  - Concentration

  - Viscosity of the fluid

  - Density of particle

  - All of the above

- Two particles are called to be equal settling, if they are having the same

  - Size

  - Specific gravity

  - Terminal velocities in the same fluid & in the same field of force

  - None of these

- During filtration process a compressible cake has the

  - Maximum porosity at the upstream side

  - Maximum porosity at the filter medium

  - Same porosity throughout the cake thickness

  - None

- With increase in drum speed in a rotary drum filter the filtration rate

  - Increases

  - Increases linearly

  - Decreases

  - Is not affected

- The capacity of a pneumatic conveying system depends upon the

  - Bulk density of materials

  - Pressure of the conveying air

  - Diameter of the conveying line

  - All of he above

- The constants (Kb, Kr and Kk) used in the laws of crushing (i.e., Bond's law,

Rittinger's law and Kick's law) depend upon the

  - Feed material

  - Type of crushing machine

  - Both (a) and (b)

  - Neither (a) nor (b)

- In which one of the following filters the feed suspension flows under pressure at a fairly high velocity across the filter medium?

  - Cake filters

  - Clarifying filters

  - Cross flow filters

  - All of the above

- Which mean diameter of particles is given by 1/Σ (xi/Dpi ⃗ )

  - The arithmetic mean diameter

  - The volume-surface mean diameter

  - The mass mean diameter

  - The volume mean diameter

- Vertical transportation of solid materials can be done by

  - Screw conveyor

  - Pneumatic conveyor

  - Bucket elevator

  - Both (b) and (c)

- As particle size is reduced

  - Screening becomes progressively more difficult

  - Screening becomes progressively easier

  - Capacity and effectiveness of the screen is increased

  - None

- Which one of the following material are not suitable for vertical lifting by Bucket elevators?

  - Fine materials

  - Sticky materials

  - Small lumpy

  - Free flowing

## Course Name: Plant Design and Economics

- An equipment item costs birr 200,000 last year for a standard size. If the cost index was 120 last year and is 150 this year, what is the estimated current cost in birr of a similar equipment item that is three times larger in size, assuming a size scaling exponent of 0.6, using the cost index method?

- 350,000

- 400,000

- 450,000

- 483,200

- A piece of equipment costs birr 100,000 and has a salvage value of birr 5,000 after 10 years. Using the straight-line depreciation method, what is the estimated book value of the equipment at the end of six years in birr?

- 47,000

- 55,000

- 40,000

- 43,000

- A plant is currently producing 20,000 ton/year of a product with an overall yield of 70%. The raw material costs 8,000 birr/ton, and the product sells for 15,000birr /ton. A process modification has been proposed that will increase the yield to 72.5%. The additional investment required is 10,000,000 birr, and the additional operating costs are negligible. Determine the payback period for this investment based on the increased revenue from the process modification.

- 1 year

- 2.5 years

- 3 years

- 4 years

| ### Course Name: Process Dynamics and Control | ### Course Code: ChEg4121 |
| --- | --- |

- Which of the following is a requirement of a chemical plant?

- Safety

- Operational constraints

- Economics

- All of the above

- Which of the following is not constituent of a mathematical model for a process?

- Disturbance variables

- Manipulated variables

- Process parameters

- State variables

- None of the above

- One of the following does not represent the difficulties encountered in developing a realistic mathematical model for a process.

- Size and complexity of a model

- poorly-understood process

- Imprecisely known parameters

- None of the above

- One of the following is not one of the critical steps for proper mathematical model development for a process

- Understanding the control objectives, expected disturbances, and physical phenomena taking place in the process

- Specifying the system, the fundamental quantities, and the additional equations

- Linearization of non-linear mathematical model equations

- None of the above

- Which of the following is not true about Laplace transform

- provides input-output relationships

- is qualitative analysis of process systems

- is a linear operation

- none of the above

- the ratio of out variable to input variable in Laplace domain is

- Laplace transform

- Single-input-single-output process model

- Transfer function

- Inverse Laplace transform

- The roots of denominator in the transfer function in Laplace domain is called the

- poles

- response

- zeros

- none

- Which of the following poles give terms that grow or decay exponentially upon inversion?

- Real, distinct poles

- Multiple, real poles

- Complex conjugate poles

- Poles at the origin

- A and B

- Which of the following does not describe a first-order process?

- A capacity to store mass, energy, or momentum

- Resistance to flow of mass, energy, or momentum

- Are always self-regulating

- A and B

- Which of the following represent the change in output per unit change in input in a process model equation in Laplace domain?

- Time constant

- Process gain

- Steady-state gain

- Static gain

- All except A

- Which of the following is not a parameter for a first-order process?

- Time constant

- Damping factor

- Process gain

- None of the above

- Which of the following characterizes a second-order process?

- Processes possessing inertia

- First-order processes in series

- Processes with their controllers

- Multicapacity systems

- All of the above

- Which of the following is true?

- Overdamped process responses resemble a first-order process

- Overdamped process responses are more sluggish than a critically damped process

- Overdamped process responses are robust

- All of the above

- Which of the following is true about underdamped processes?

- Are always faster than critically damped processes

- Starts oscillating with increasing amplitude as the damping factor decreases

- Oscillation becomes more pronounced with increasing amplitude

- None of the above

- For an underdamped process response, the ratio of the maximum amount by which the response exceeds the ultimate value to the ultimate value of the response represents

- Period of oscillation

- Decay ratio

- Overshoot

- Response time

- For a second-order process, the time needed for the response to reach the ultimate value in an oscillatory manner is called

- Process time constant

- Rise time

- Response time

- Natural period of oscillation

- Which of the following is true?

- A system becomes more sluggish as the number of capacities in series increase

- A first-order process response is always faster than the response of two non-interacting systems in series

- Two non-interacting systems are always faster in their response than two interacting systems

- All of the above

- Which of the following characterizes the control actions for a feedback control system?

- It measures the value of the input

- It compares the value of the output to the desired value

- It measures the value of the output

- The value of the output is supplied to the controller

- B and C

- Which of the following is not correct about a feedback control system?

- The input to the controller is the measured output variable

- The output from the controller is the error (deviation of the output value from the desired value)

- The various types of feedback controllers differ in the way they relate the output variable to the error

- All of the above

- In proportional-integral type controller, the additional parameter to be manipulated compared to proportional type controller is the

- Integral time constant

- Reset time

- Response time

- All of the above

- A and B

- Which of the following feedback controller type is known as anticipatory controller?

- Proportional type

- Proportional-integral type

- Proportional-integral-derivative type

- In most control systems, the manipulated variables are

- Time

- Sensors

- Flow rates

- Valves

- Pressure gauges

- In most control systems, the final control elements are

- Transducers

- Valves

- thermocouples

- Flow rates

- Orifices

- Which of the following is true about a feedback control system?

- The disturbance does not change in a servo problem

- The set point remains constant in a regulator problem

- The controller tries to keep the output value close to the set point in a servo problem

- The control tries to eliminate the impact of the load change in a regulator problem

- All of the above

- One of the following is not true about the effect of proportional controller on first-order systems

- The response remains first-order with respect to load and set point changes

- The time constant is reduced

- The static gains remain constant

- The offset is always not zero

- None of the above

- The closed-loop response of a second-order system with proportional control has the characteristics:

- It remains second-order

- The static gain decreases

- Overdamped process may become underdamped process

- All of the above

- In a second-order system controlled with proportional controller, increasing the controller gain

- Decreases the damping factor

- Increases the overshoot

- Increases the overshoot

- Decreases the period of oscillation

- All of the above

- Choose the correct statement

- Integral control action eliminates any offset

- Integral control action increases the order of the dynamics for the closed-loop response

- Integral control action makes the response of a closed-loop system more sluggish

- All of the above

- For a derivative control action integrated with first-order process, which one is correct?

- The order of the process does not change

- The effective time constant increases

- The process response becomes sluggish

- All of the above

- For a derivative control action integrated with second-order process, which one is correct?

- The damping factor increases

- The natural period of the response decreases

- The process response becomes more robust

- All of the above

- Which one of the following is incorrect?

- Non-oscillatory first-order processes may acquire oscillatory behaviour with proportional controller

- Oscillatory second-order processes may become unstable with a proportional-integral controller

- Increasing the integral control action makes the response of the closed-loop system more sensitive

- None of the above

- All of the above

- Choose the correct statement about process stability

- A system is unstable if its transfer function has at least one positive real pole

- A system is stable if all the root of the characteristic equation have negative real parts only for set point changes

- A system is stable if all the root of the characteristic equation have negative real parts only for load changes

- All of the above

- In Routh-Hurwitz criterion for stability, which one is incorrect?

- A system is stable if all the coefficients of the polynomial are positive

- A system is stable if all the elements in the first row of the Routh array are positive

- The stability characteristics of a closed-loop system does not depend on the value of the gain

- All of the above

- None of the above

- Which one of the following is incorrect about the performance criteria for selecting and tuning of controller?

- Achieve short settling times

- Keep the maximum deviation minimum

- Minimize the integral of the errors

- All of the above

- None of the above

- Which of the following is the most popular simple static performance criterion for a closed-loop response of a system?

- Decay ratio

- Settling time

- Overshoot

- Rise time

- All of the above

- Which of the following dynamic performance criteria suppresses large errors?

- Integral of the square error

- Integral of the absolute value of the error

- Integral of the time-weighted absolute error

- All of the above

- Which of the following dynamic performance criteria suppresses sustained errors?

- Integral of the square error

- Integral of the absolute value of the error

- Integral of the time-weighted absolute error

- All of the above

- Which of the following dynamic performance criteria suppresses small errors?

- Integral of the square error

- Integral of the absolute value of the error

- Integral of the time-weighted absolute error

- All of the above

- Which of the following is correct?

- Different performance criteria lead to different controller designs

- Different input changes lead to different controller designs

- Proportional type controller is mostly tuned for minimum offset

- All of the above

- None of the above

- Which of the following is incorrect?

- Proportional controller accelerates the response of a controlled system

- Derivative controller introduces a stabilizing effect on a closed-loop response of a process

- Increasing the controller gain makes a system more stable

- All of the above

Course Name: Heat and Mass Transfer

- Which of the following is the primary mode of heat transfer in solids?

- The recommended mode of heat transfer is Convection

- The recommended mode of heat transfer is Radiation

- The recommended mode of heat transfer is Conduction

- The recommended mode of heat transfer is Insulation

- Which of the following statements about radiation is true?

- The radiation heat transfer requires a medium to transfer heat.

- The radiation heat transfer can occur through a vacuum.

- The radiation heat transfer only transfers heat in the form of sound.

- The radiation heat transfer is the least efficient form of heat transfer.

- In which situation does convection occur?

- Heating a metal rod at one end.

- Boiling water in a pot.

- Sunlight warming your skin.

- A warm blanket keeping you convenient.

- What is thermal conductivity?

- The measure of a material's ability to transfer heat through conduction.

- The ability of a material to conduct electricity through it.

- The process of heat transfer through moving fluids.

- The emission of energy as electromagnetic waves.

- If a pot is placed on a stove, which heat transfer method primarily heats the pot?

- The heat transfer method is Convection

- The heat transfer method is Radiation

- The heat transfer method is both Radiation and Convection

- The heat transfer method is Conduction

- Which of the following is an example of convection?

- Ice melting in a warm room.

- A metal spoon getting hot in a pot of boiling soup.

- The sun warming the Earth.

- Warm air rising and cold air descending in a room.

- What happens to the air above a heated surface?

- The air heats up and rises

- The air cools down and sinks.

- The air remains unchanged.

- The air conducts heat to the surface.

- What is thermal conductivity?

- The ability to store thermal energy

- The rate of heat transfer per unit area

- The measure of a material's ability to conduct heat

- The total heat energy contained in a material

- In the heat diffusion equation, what does the term ∇²T represent?

- The rate of heat generation

- The thermal conductivity

- The time rate of change of temperature

- The spatial variation of temperature

- What does the term 'initial condition' refer to in heat diffusion problems?

- The temperature distribution at a specific time

- The boundary temperature of the system

- The material properties involved

- The external heat sources applied

- What is the significance of the thermal diffusivity (α) in the heat diffusion equation?

- It defines the heat capacity of a material.

- It indicates how quickly a material can conduct thermal energy.

- It is the ratio of thermal conductivity to heat capacity.

- It determines the maximum temperature a material can withstand.

- If a rod is initially at a uniform temperature and one end is heated, what will happen over time?

- The temperature will remain uniform.

- The temperature will increase at the heated end only.

- The rod will cool down.

- Heat will diffuse along the rod, changing the temperature distribution.

- What is thermal resistance?

- The ability of a material to conduct heat

- The rate of heat transfer per unit area

- The opposition to heat flow through a material or system

- The total heat energy contained in a material

- How is thermal resistance (R) defined mathematically?

- R = q / ΔT

- R = ΔT / q

- R = q × ΔT

- R = q + ΔT

- Which of the following factors affects thermal resistance in a material?

- Thickness of the material

- Thermal conductivity of the material

- Surface area of the material

- All of the above

- What is contact resistance?

- Resistance to heat flow through a solid material

- Resistance due to air gaps in thermal insulation

- Resistance at the interface between two materials in contact

- Resistance to convective heat transfer

- materials in series, how is it determined?

- By adding the resistances together

- By multiplying the resistances

- By taking the average of the resistances

- By dividing the total heat transfer by the temperature difference

- Which equation relates thermal resistance (R), heat transfer rate (q), and temperature difference (ΔT)?

- The relationship equation is q = R × ΔT

- The relationship equation is q = ΔT / R

- The relationship equation is q = R + ΔT

- The relationship equation is q = R / ΔT

- What is the main driving force for mass transfer?

- The main driving force for mass transfer is temperature difference

- The main driving force for mass transfer is pressure difference

- The main driving force for mass transfer is concentration difference

- The main driving force for mass transfer is velocity difference

- Which of the following equations describes mass transfer by diffusion?

- The equations that describes mass transfer by diffusion is Fick's first law

- The equations that describes mass transfer by diffusion is Fourier's law

- The equations that describes mass transfer by diffusion is Newton's law of cooling

- The equations that describes mass transfer by diffusion is Navier-Stokes equations

- In one-dimensional steady-state conduction, why do we assume uniform heat generation in plane walls, cylinders and spheres?

- To determine T∞ and Ts

- To determine T∞  and Tmax

- To determine Ts, and Tmax

- To determine T∞ Ts, and Tmax

- In the case of uniform heat generation in a solid, the governing equation in Cartesian coordinates is derived from:

- Fourier’s law of heat conduction

- The heat diffusion equation with a source term

- The mass diffusion equation

- Newton’s law of cooling

- In Cartesian/cylindrical/spherical coordinates with thickness L or radius r0 & uniform heat generation (q), the temperature profiles for (d2Tdx2+qk=0; 1rddr(rdTdr) +qk=0; 1r2ddr(r2dTdr) +qk=0 ) is typically:

- Linear, Parabolic and Exponential function respectively

- Linear, Linear and Linear function respectively

- Parabolic, Parabolic and Parabolic function respectively

- Linear, Parabolic and Logarithmic function respectively

- The maximum temperatures in spherical coordinates with uniform heat generation occur at:

- The center of the sphere

- The surface of the sphere

- The midpoint between the surface and the center

- A point determined by the material properties

- For a constant surface temperature Ts​, which of the following methods can increase the heat transfer rate in extended surfaces?

- Decreasing the thermal conductivity of the fin material

- Reducing the convection area of the surface

- Increasing the heat transfer coefficient h

- Increasing the surrounding temperature T∞

- Why should you recommend the fin material must have a high thermal conductivity k?

- To limit heat loss through convection

- To reduce the weight of the fins

- To ensure the fins do not deform under heat

- To ensure temperature variation across the fin is minimized

- The efficiency of a fin decreases when:

- The thermal conductivity of the fin material increases

- The heat transfer area of the fin increases

- The length of the fin increases excessively

- The surrounding temperature decreases

- The heat transfer rate from an extended surface is increased by employing fins because:

- Fins reduce the convective heat transfer coefficient

- Fins increase the effective surface area for convection

- Fins reduce the heat transfer resistance

- Fins increase the thermal resistance

- Which of the following is NOT a general consideration for heat transfer from fins?

- The surface temperature Ts should be constant

- Fins should have high thermal conductivity k

- Surrounding temperature T∞​ affects the rate of heat transfer

- Increasing the convection area reduces the heat transfer rate

- What does fin efficiency measure?

- The effectiveness of a fin in transferring a given quantity of heat

- The maximum temperature difference along the fin

- The length of the fin needed for effective heat transfer

- The thermal conductivity of the fin material

- Which factor reduces the fin efficiency significantly?

- A High thermal conductivity of the fin material

- A large temperature gradient along the fin

- A small heat transfer coefficient of the surrounding fluid

- A short fin length

- How is the maximum possible heat transfer from a fin calculated?

- By considering the fin tip to be insulated

- By calculating the average temperature of the fin

- By assuming the entire fin is at the base temperature

- By neglecting heat conduction within the fin

- In practical applications, fin efficiency is used to:

- Measure the ambient temperature around the fin

- Eliminate the need for heat transfer calculations

- Determine the maximum possible heat generation within the fin

- Select the optimal material and geometry for a fin

- If a fin has low efficiency, it means:

- Most of the heat is transferred effectively

- The fin is not transferring heat effectively

- The fin material has poor thermal conductivity

- The fin length is too short

- Why is overall effectiveness a better performance measure than individual fin effectiveness?

- It considers the material properties only.

- It simplifies the heat transfer calculations.

- It accounts for the contribution of all fins and the unfinned surface.

- It depends only on the temperature gradient across the fins.

- Which of the following is considered a better measure of performance for a finned surface?

- The overall effectiveness of the entire finned surface

- The effectiveness of the individual fins

- The thermal resistance of the base material

- The convective heat transfer coefficient

- When is the overall effectiveness of a finned surface low?

- When the fin density is high

- When the fin material has low thermal conductivity

- When the base material temperature is high

- When the surrounding fluid has a low heat transfer coefficient

- What happens to the overall effectiveness if the effectiveness of individual fins is high but the fin density is low?

- Overall effectiveness is high

- Overall effectiveness remains constant

- Overall effectiveness is low

- Overall effectiveness becomes negative

- What the optimal length of a fin balances:

- The heat transfer rate and the convective heat transfer coefficient

- The thermal conductivity and the surrounding fluid temperature

- The temperature difference between the fin tip and the base

- Heat dissipation, material cost, and weight

- Why does the region near the fin tip contribute less to heat transfer?

- The temperature gradient decreases as you approach the tip

- The heat transfer coefficient is zero at the tip

- Thermal conductivity increases near the tip

- The convective heat transfer coefficient increases significantly

- If the fluid temperature T∞ is close to the base temperature Tb:

- The optimal fin length increases

- The optimal fin length decreases

- The thermal conductivity of the fin becomes irrelevant

- Heat transfer becomes independent of fin length

- To minimize material cost & weight while ensuring effective heat transfer, optimal fin length should:

- Be as long as possible to maximize surface area

- Be independent of the heat transfer coefficient

- Be short enough to avoid ineffective heat transfer near the tip

- Equal the distance between the base and the surrounding fluid

- What is the velocity boundary layer in either natural or forced convective heat transfer?

- The region of flow where the velocity is constant

- The distance from the surface where the fluid pressure is uniform

- The area in a flow where the temperature is uniform

- The region of flow near a surface where viscous effects are significant

- What is the main cause of the velocity boundary layer in natural or forced convective heat transfer?

- Differences in temperature across the fluid

- Viscous shearing forces due to fluid viscosity

- Turbulence in the flow

- The density variation of the fluid

- What causes the thermal boundary layer to develop?

- A change in fluid density

- A velocity gradient in the fluid

- A temperature difference between the fluid and the surface

- A constant pressure gradient along the surface

- What happens to the thermal boundary layer thickness as the flow moves downstream?

- It decreases with the flow direction

- It increases with the flow direction

- It remains constant along the surface

- It oscillates depending on the fluid's velocity

- In convective heat transfer, dimensionless numbers help in:

- Designing systems with uniform temperature distribution

- Determining the maximum heat transfer area

- Estimating the velocity of the fluid

- Identifying flow regimes and scaling experimental results to different systems

- Which of the following is a general benefit of using dimensionless numbers in convective heat transfer problems?

- They allow experimental data to be generalized for various geometries and flow conditions

- They simplify calculations by eliminating physical constants

- They provide exact solutions to all heat transfer problems

- They remove the need for boundary conditions in numerical simulations

- What is the main criterion for classifying boiling as subcooled or saturated?

- The velocity of the fluid in the boiling devices relative to the wall

- The bulk liquid temperature relative to the saturation temperature

- The surface roughness of the heating element for the fluid motion

- The type of liquid being boiled in the heating element

- How is pool boiling different from flow boiling?

- Pool boiling occurs with bulk fluid motion, while flow boiling without bulk fluid motion

- Both Pool and flow boiling involve forced fluid flow over a heated surface

- Pool boiling occurs without bulk fluid motion, while flow boiling with forced bulk fluid motion

- Pool boiling requires a heated pipe, while flow boiling occurs only on a flat surface

- Why is heat transfer in film condensation less efficient than in dropwise condensation?

- The liquid film creates resistance to heat transfer

- The surface temperature is higher in film condensation

- The vapor cannot condense effectively in film condensation

- Film condensation requires a higher saturation temperature

- What is the key advantage of using packed beds in heat transfer or condensation processes?

- Low surface area to volume ratio

- High velocity of the fluid within the system

- Minimal interaction between fluid and solid materials

- High surface area to volume ratio for efficient interaction

- What makes fluidized beds suitable for high-rate condensation processes?

- The large surface area provided by fixed particles

- The uniform and dynamic interaction between solids and fluid

- The reduced heat and mass transfer rates

- The stationary nature of the particles

Basic Environmental Engineering

- Which of the following statements is “FALSE”?

- The refuse materials typically have a homogeneous nature.

- The generation rate and composition of refuse vary from one community to another.

- The generation rate and composition of refuse vary as people’s lifestyles change.

- The generation rate and composition of refuse vary from time to time in a community.

- Which of the following “Is Not” among the objectives of solid waste management?

- To prevent the spread of disease and the likelihood of fires.

- To increase the aesthetic insults due to the organic matter degradation.

- To dispose of the discarded items in an environmentally sound manner.

- To remove the discarded items from inhabited places promptly.

- Which of the following “Is Not” among the major causes of the greenhouse effect?

- Water vapor

- Methane (CH4)

- Sulfur dioxide (SO2)

- Carbon dioxide (CO2)

- Which of the following “Is Not” among the gaseous air pollutant controlling techniques?

- Absorption

- Combustion or incineration

- Adsorption

- Electrostatic precipitation

- Which of the following “Is Not” among the particulate air pollutant controlling techniques?

- Flaring

- Scrubbing

- Fabric filtering

- Cyclone settling

- Which of the following “Is Not” among the major causes of ozone layer depletion?

- Nitrogen oxides (NOX)

- Methyl bromide (CH3Br)

- Methyl chloroform (CH3CCl3)

- Carbon dioxide (CO2)

- Which of the following statements best describes the concept of material recovery in waste management?

  - The process of extracting useful or valuable components from waste materials

  - The complete disposal of waste without any resource utilization

  - The uncontrolled dumping of mixed refuse without separation

  - The direct incineration of waste without prior sorting

- Which of the following statement is “FALSE” about solid waste?

- Includes refuse, garbage, rubbish, sludge, and hazardous wastes.

- Any liquid or slurry material unwanted by the holder and purposely thrown away.

- Consist of several types of discarded items, whose composition can vary significantly.

- Consist of several types of discarded items whose generation rates varies significantly.

- Which of the following statements is “FALSE” about reuse?

- Involves the return of waste materials into the industry for remanufacturing.

- Extends the life of the original product, thus reducing the generation of waste.

- Recycling of wastes back into the original process to be used for the same purpose.

- Either the repeated use of waste for the same purpose or puts it into a secondary use.

- Which of the following “Is Not” among the effects of improper solid waste management?

- Infiltration of leachate, which results in groundwater contamination.

- Proliferation of rats, flies, and mosquitoes, which are important disease vectors.

- Causing odor, explosion, soil productivity problems, and greenhouse effects.

- Expelling of birds, which are both a nuisance and a problem to low-flying airplanes.

- Which of the following statements is “FALSE”?

- Extracting silver from the waste photography films is an example of recovery.

- Turning waste plastics into toys and papers into cartons are examples of disposal.

- Building a new polymer from the waste polymers processed into monomers is recycling.

- Use of waste paper and plastic bags during the final disposal of trash is an example of reuse.

- Which of the following statements is FALSE regarding recycling?

  - The use of recovered plastic materials to produce insulation products.

  - The further use of waste materials for lower-quality applications (down-cycling).

  - The use of waste materials in a new life cycle after processing to replace virgin materials.

  - The direct disposal of waste materials without processing as a form of recycling.

- Which of the following statement is “FALSE” about landfills?

- Sanitary landfill sites should be downwind from any nearby community.

- A small sanitary landfill site with limited capacity is generally acceptable.

- Landfills survive long time, thus limiting the use of the land for other purposes.

- No landfill is tight enough to avoid contamination of groundwater by the leachate.

- Which of the following “Is Not” among the pollution prevention measures?

- Using raw materials with low levels of polluting or pollution-generating components.

- Screening the potential pollutants from the exhaust emission streams before discharge.

- Reducing the amount of solvents used by using high-boiling-point and less-hazardous solvents.

- Screening the potential pollutants from the input materials and auxiliaries before its use.

- Which of the following statements is “FALSE”?

- Emissions from industrial smoke stacks is an example of point source.

- Emission from industrial smoke stacks is an example of a single source.

- Emission from industrial smoke stacks is an example of a stationary source.

- Heavily travelled highways and the leading edges of forest fires are fugitive sources.

- Which of the following is NOT a standard method for classifying sources of air pollutants?

  - Classification as stationary or mobile sources based on physical characteristics

  - Classification as natural or anthropogenic sources based on origin

  - Classification as point, area, or line sources based on spatial distribution

  - Classification as single, multiple, or line sources based on spatial distribution

- Which of the following statement is “FALSE”?

- Nitrogen dioxide (NO2) is a secondary air pollutant, while acid rain is its effect.

- Nitrogen dioxide is a secondary air pollutant, while photochemical smog is its effect.

- Nitrogen dioxide (NO2) is a secondary air pollutant, while eutrophication is its effect.

- Nitrous oxide (N2O) is a secondary air pollutant, while ozone layer depletion is its effect.

- Which of the following statement is “FALSE”?

- Processing of waste organic materials to generate power or electricity is recovery.

- Processing of waste plastics and papers as filling for packaging is a down-cycling.

- Recycling and recovery involves the waste collection and processing into new product.

- Recycling involves industrial separation of waste materials, whereas recovery involves the separation of waste materials by voluntary individuals.

- Which of the following statement is “FALSE”?

- Ozone (O3) is a secondary air pollutant, while global warming is its effect.

- Sulfur dioxide (SO2) is a secondary air pollutant, while acid rain is its effect.

- Acidic mists/vapors are secondary air pollutants, while acid rain is their effect.

- Ozone (O3) is a secondary air pollutant, while photochemical smog is its effect.

- Which of the following statement is “FALSE”?

- Determination of the fixed carbon content of solid waste is proximate analysis.

- Determination of the ash and moisture content of solid waste is proximate analysis.

- Determination of the volatile combustible matter of solid waste is ultimate analysis.

- Determination of the % C, H, N, S, and O contents of solid waste is ultimate analysis.

- Which of the following statements about air pollution sources is FALSE?

  - Point sources have a fixed location and a single identifiable emission point.

  - Area sources represent diffuse emissions over a relatively large region.

  - Multiple sources refer to emissions originating from several distinct points within a defined area.

  - Single sources are typically mobile sources, such as vehicles with varying emission locations

- Which of the following is NOT a standard method for classifying types of air pollutants?

  - Classification as primary or secondary pollutants based on their formation

  - Classification as gaseous or particulate pollutants based on physical state

  - Classification as organic or inorganic pollutants based on chemical composition

  - Classification as stationary or mobile pollutants based on source characteristics

- Which of the following statement is “FALSE”?

- Municipal solid waste management depends on the characteristics of both the waste materials and the disposal methods.

- Proximate and ultimate analyses have a similar objective, which is to determine the chemical composition of municipal solid waste.

- As the greenhouse gases concentration in the atmosphere increases, global warming will increase, while the greenhouse effect diminishes.

- Integrated solid waste management is the selection of a combination of technologies, techniques, and management programs to achieve health, environmental, and resource conservation goals.

- Which of the following statements is “TRUE”?

- Open dumping is the placement of solid wastes on land, while sanitary landfill is an engineered operation, which is designed and operated according to accepted standards.

- Primary air pollutants are those that are emitted directly into the atmosphere from their sources and exist in the atmosphere in a different form than they did in source emissions.

- Air pollution control aims at designing processes for absolute zero emissions of pollutants, whose atmospheric presence can cause adverse effects.

- Refuse-derived fuel is the noncombustible portion of solid wastes that is separated from the combustible portion by shredding, screening, and air classifying processes.

- Which of the following statements is “FALSE” about Total Suspended Solids in Water?

- Improve the efficiency of chlorine disinfection.

- Diminish the aesthetic quality and aquatic photosynthesis.

- Provide adsorption site for biological and chemical agents.

- Degraded biologically and result in objectionable byproducts.

- Which of the following statements is “NOT” among the effects of environmental pollution?

- Acid Rain

- Global Warming

- Drought – Food Shortage

- Natural Resource Exploitation

- Which of the following statements is “TRUE”?

- Hardness and alkalinity usually occur simultaneously.

- The BOD test results can exceed the COD test results.

- At neutral pH, total alkalinity can be taken to be equal to the carbonate concentration.

- Higher BOD signifies clean water, while lower BOD signifies nondegradable pollutants

- Which of the following statements is “NOT” among the sources of environmental pollution?

- Urbanization

- Industrial Growth

- Population Growth

- Disease Vector Transmission

- Which of the following statements is “TRUE”?

- Non-carbonate hardness has a temporary nature that can be removed by boiling.

- Carbonate hardness has a permanent nature that can be removed by chemical softening.

- The most effective means of controlling the secondary air pollutants is to control their emissions before they are discharged into the atmosphere.

- Air pollution control strategies are devised to control the emissions from anthropogenic sources, to maintain the self-dilution capacity of the atmosphere.

- Which of the following statements is “FALSE” about Water Hardness and Alkalinity?

- Carbonate hardness is typically associated with the carbonate and bicarbonate anions.

- Noncarbonate hardness is associated with all anions except carbonate and bicarbonate.

- The form of the hardness affects the type of treatment to be used for hardness removal.

- Alkalinity is not the same as pH, because water needs to be basic to have higher alkalinity.

- Which of the following statements is “FALSE”?

- If alkalinity ≥ total hardness, non-carbonate hardness will be zero

- Turbidity can be used to evaluate the efficiency of water treatment unit operations.

- If the water sample has a taste without odor, the possible cause can be organic substances.

- While True Color is due to dissolved solids, Apparent Color is due to suspended matter.

- Which of the following statements is “FALSE”?

- Biochemical Oxygen Demand (BOD) is not a specific pollutant.

- Biochemical Oxygen Demand (BOD) is not an accurate quantitative measure.

- Biochemical oxygen demand measurement can be affected by temperature, light, and time

- Coagulation is a physical process that produces differential velocities in water for the colloidal particles to collide and stick together and then grow in size.

- Which of the following is “NOT” among the properties of Surface Water sources?

- Tests and Odors

- Constant Composition

- Microorganisms Present

- Possible Chemical Toxicity

- Which of the following is “NOT” among the major Sources of Environmental Pollution?

- Acid Rain

- Industrial Growth

- Population Growth

- Natural Resource Exploitation

- Which of the following is “NOT” among the potential Effects of Environmental Pollution?

- Urbanization

- Global Warming

- Ozone Layer Depletion

- Drought and Food Shortage

- Which of the following statements is “FALSE” about ground- and surface-water sources?

- Groundwater sources have higher turbidity levels than surface water sources.

- Groundwater sources have a higher hardness level than surface water sources.

- Groundwater sources have higher mineralization levels than surface water sources.

- Groundwater sources has lower dissolved oxygen levels than surface water sources.

- Which of the following factors is NOT typically considered in the siting of sanitary landfills?

  - Soil type and geotechnical properties

  - Depth of the groundwater table

  - Wind direction and rainfall patterns

  - Proximity to airports

- Which of the following factors does NOT influence the selection of air pollution control equipment?

  - Particulate size and concentration (loading)

  - Required collection efficiency and allowable pressure drop

  - Properties and flow characteristics of the carrier gas

  - Color of the emitted gas plume

- Which of the following statements is FALSE regarding sources of water pollution?

  - Point source pollution is generally easier to monitor and regulate than nonpoint source pollution.

  - Nonpoint source pollution originates from diffuse sources over a wide area.

  - Point source pollution consists of pollutants entering water bodies through identifiable discharge points.

  - Point source pollution typically arises from diffuse runoff over large areas without a specific discharge point.

- Which of the following is NOT a typical property of an effective coagulant used in water treatment?

  - Non-toxic in the treated water

  - Ability to form insoluble precipitates

  - High solubility to remain completely dissolved at neutral pH

  - Capability to destabilize colloidal particles

- Which of the following is NOT a typical characteristic of groundwater?

  - Higher hardness due to dissolved minerals

  - Higher turbidity compared to surface water

  - Higher mineral content (mineralization)

  - Lower color and generally low dissolved oxygen

- Which of the following statements is “FALSE”?

- Total volatile solids are organic solid substances that can volatilize at 550°C.

- Total fixed solids are inorganic solid substances that do not volatilize at 550°C.

- Turbidity can be used to evaluate the efficiency of water treatment unit operations.

- Apparent color is due to dissolved solids that remain after removal of suspended solid.

- Which of the following statements is “FALSE” about the common water treatment plants?

- Chemical doses are much higher in softening plants than in filtration plants.

- Simple disinfection plants are used for groundwater sources with high water quality.

- The recarbonation unit operation is employed in filtration plants, not in softening plants.

- Filtration plants are used to treat surface water sources, while softening plants are used to treat groundwater sources.

ANSWER KEY

| # 1 | # C | # 216 | # A | # 431 | # C | # 646 | # B |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # 2 | # B | # 217 | # C | # 432 | # C | # 647 | # B |
| # 3 | # B | # 218 | # A | # 433 | # D | # 648 | # A |
| # 4 | # A | # 219 | # A | # 434 | # C | # 649 | # C |
| # 5 | # A | # 220 | # B | # 435 | # D | # 650 | # C |
| # 6 | # B | # 221 | # D | # 436 | # B | # 651 | # B |
| # 7 | # D | # 222 | # A | # 437 | # A | # 652 | # D |
| # 8 | # A | # 223 | # A | # 438 | # B | # 653 | # B |
| # 9 | # B | # 224 | # B | # 439 | # B | # 654 | # B |
| # 10 | # D | # 225 | # A | # 440 | # D | # 655 | # C |
| # 11 | # A | # 226 | # A | # 441 | # A | # 656 | # B |
| # 12 | # A | # 227 | # A | # 442 | # B | # 657 | # D |
| # 13 | # C | # 228 | # D | # 443 | # D | # 658 | # B |
| # 14 | # D | # 229 | # B | # 444 | # B | # 659 | # D |
| # 15 | # B | # 230 | # A | # 445 | # B | # 660 | # A |
| # 16 | # A | # 231 | # C | # 446 | # D | # 661 | # D |
| # 17 | # C | # 232 | # A | # 447 | # B | # 662 | # C |
| # 18 | # D | # 233 | # A | # 448 | # A | # 663 | # B |
| # 19 | # B | # 234 | # C | # 449 | # B | # 664 | # A |
| # 20 | # A | # 235 | # D | # 450 | # A | # 665 | # B |
| # 21 | # A | # 236 | # A | # 451 | # A | # 666 | # A |
| # 22 | # B | # 237 | # D | # 452 | # A | # 667 | # D |
| # 23 | # A | # 238 | # A | # 453 | # B | # 668 | # C |
| # 24 | # B | # 239 | # C | # 454 | # B | # 669 | # A |
| # 25 | # A | # 240 | # A | # 455 | # D | # 670 | # B |
| # 26 | # B | # 241 | # D | # 456 | # D | # 671 | # C |
| # 27 | # D | # 242 | # A | # 457 | # C | # 672 | # C |
| # 28 | # D | # 243 | # D | # 458 | # B | # 673 | # C |
| # 29 | # B | # 244 | # C | # 459 | # A | # 674 | # D |
| # 30 | # C | # 245 | # D | # 460 | # D | # 675 | # D |
| # 31 | # B | # 246 | # C | # 461 | # C | # 676 | # B |
| # 32 | # C | # 247 | # D | # 462 | # B | # 677 | # D |
| # 33 | # D | # 248 | # C | # 463 | # C | # 678 | # C |
| # 34 | # A | # 249 | # A | # 464 | # A | # 679 | # C |
| # 35 | # B | # 250 | # D | # 465 | # D | # 680 | # C |
| # 36 | # A | # 251 | # B | # 466 | # C | # 681 | # D |
| # 37 | # A | # 252 | # C | # 467 | # D | # 682 | # B |
| # 38 | # B | # 253 | # B | # 468 | # B | # 683 | # C |
| # 39 | # A | # 254 | # D | # 469 | # D | # 684 | # B |
| # 40 | # A | # 255 | # D | # 470 | # C | # 685 | # D |
| # 41 | # A | # 256 | # C | # 471 | # C | # 686 | # A |
| # 42 | # A | # 257 | # A | # 472 | # A | # 687 | # D |
| # 43 | # B | # 258 | # C | # 473 | # D | # 688 | # D |
| # 44 | # A | # 259 | # C | # 474 | # D | # 689 | # B |
| # 45 | # C | # 260 | # D | # 475 | # C | # 690 | # B |
| # 46 | # A | # 261 | # C | # 476 | # B | # 691 | # A |
| # 47 | # D | # 262 | # C | # 477 | # D | # 692 | # D |
| # 48 | # B | # 263 | # D | # 478 | # C | # 693 | # B |
| # 49 | # D | # 264 | # D | # 479 | # C | # 694 | # B |
| # 50 | # D | # 265 | # A | # 480 | # D | # 695 | # A |
| # 51 | # C | # 266 | # A | # 481 | # D | # 696 | # C |
| # 52 | # D | # 267 | # D | # 482 | # D | # 697 | # D |
| # 53 | # D | # 268 | # A | # 483 | # B | # 698 | # C |
| # 54 | # A | # 269 | # E | # 484 | # B | # 699 | # C |
| # 55 | # D | # 270 | # D | # 485 | # D | # 700 | # A |
| # 56 | # A | # 271 | # E | # 486 | # C | # 701 | # A |
| # 57 | # C | # 272 | # D | # 487 | # B | # 702 | # E |
| # 58 | # B | # 273 | # B | # 488 | # B | # 703 | # C |
| # 59 | # D | # 274 | # C | # 489 | # C | # 704 | # C |
| # 60 | # D | # 275 | # E | # 490 | # B | # 705 | # D |
| # 61 | # B | # 276 | # B | # 491 | # C | # 706 | # A |
| # 62 | # D | # 277 | # B | # 492 | # D | # 707 | # E |
| # 63 | # D | # 278 | # D | # 493 | # A | # 708 | # D |
| # 64 | # B | # 279 | # B | # 494 | # B | # 709 | # B |
| # 65 | # C | # 280 | # A | # 495 | # D | # 710 | # B |
| # 66 | # C | # 281 | # B | # 496 | # B | # 711 | # D |
| # 67 | # B | # 282 | # B | # 497 | # D | # 712 | # C |
| # 68 | # D | # 283 | # B | # 498 | # C | # 713 | # A |
| # 69 | # B | # 284 | # A | # 499 | # A | # 714 | # A |
| # 70 | # D | # 285 | # D | # 500 | # B | # 715 | # D |
| # 71 | # D | # 286 | # A | # 501 | # B | # 716 | # C |
| # 72 | # D | # 287 | # C | # 502 | # D | # 717 | # C |
| # 73 | # D | # 288 | # A | # 503 | # A | # 718 | # B |
| # 74 | # C | # 289 | # D | # 504 | # B | # 719 | # D |
| # 75 | # A | # 290 | # A | # 505 | # D | # 720 | # A |
| # 76 | # C | # 291 | # B | # 506 | # A | # 721 | # B |
| # 77 | # D | # 292 | # D | # 507 | # B | # 722 | # D |
| # 78 | # C | # 293 | # B | # 508 | # A | # 723 | # D |
| # 79 | # C | # 294 | # C | # 509 | # D | # 724 | # B |
| # 80 | # D | # 295 | # A | # 510 | # B | # 725 | # D |
| # 81 | # A | # 296 | # B | # 511 | # D | # 726 | # E |
| # 82 | # A | # 297 | # A | # 512 | # C | # 727 | # D |
| # 83 | # A | # 298 | # C | # 513 | # A | # 728 | # C |
| # 84 | # D | # 299 | # A | # 514 | # D | # 729 | # B |
| # 85 | # B | # 300 | # D | # 515 | # A | # 730 | # C |
| # 86 | # A | # 301 | # B | # 516 | # B | # 731 | # A |
| # 87 | # C | # 302 | # D | # 517 | # A | # 732 | # E |
| # 88 | # C | # 303 | # A | # 518 | # D | # 733 | # C |
| # 89 | # A | # 304 | # C | # 519 | # B | # 734 | # E |
| # 90 | # C | # 305 | # C | # 520 | # A | # 735 | # B |
| # 91 | # A | # 306 | # D | # 521 | # B | # 736 | # E |
| # 92 | # D | # 307 | # C | # 522 | # C | # 737 | # D |
| # 93 | # D | # 308 | # D | # 523 | # D | # 738 | # A |
| # 94 | # B | # 309 | # B | # 524 | # C | # 739 | # C |
| # 95 | # C | # 310 | # A | # 525 | # A | # 740 | # D |
| # 96 | # A | # 311 | # A | # 526 | # C | # 741 | # A |
| # 97 | # B | # 312 | # A | # 527 | # A | # 742 | # E |
| # 98 | # C | # 313 | # C | # 528 | # B | # 743 | # B |
| # 99 | # C | # 314 | # D | # 529 | # A | # 744 | # E |
| # 100 | # B | # 315 | # D | # 530 | # A | # 745 | # C |
| # 101 | # B | # 316 | # B | # 531 | # D | # 746 | # C |
| # 102 | # B | # 317 | # C | # 532 | # D | # 747 | # B |
| # 103 | # C | # 318 | # C | # 533 | # B | # 748 | # E |
| # 104 | # C | # 319 | # A | # 534 | # B | # 749 | # C |
| # 105 | # B | # 320 | # C | # 535 | # D | # 750 | # D |
| # 106 | # B | # 321 | # D | # 536 | # B | # 751 | # E |
| # 107 | # C | # 322 | # B | # 537 | # B | # 752 | # D |
| # 108 | # B | # 323 | # D | # 538 | # C | # 753 | # A |
| # 109 | # C | # 324 | # D | # 539 | # A | # 754 | # D |
| # 110 | # B | # 325 | # A | # 540 | # D | # 755 | # D |
| # 111 | # C | # 326 | # B | # 541 | # A | # 756 | # A |
| # 112 | # B | # 327 | # A | # 542 | # C | # 757 | # C |
| # 113 | # B | # 328 | # B | # 543 | # D | # 758 | # E |
| # 114 | # D | # 329 | # A | # 544 | # C | # 759 | # E |
| # 115 | # B | # 330 | # A | # 545 | # B | # 760 | # A |
| # 116 | # B | # 331 | # A | # 546 | # B | # 761 | # C |
| # 117 | # B | # 332 | # D | # 547 | # B | # 762 | # A |
| # 118 | # B | # 333 | # B | # 548 | # A | # 763 | # D |
| # 119 | # B | # 334 | # A | # 549 | # C | # 764 | # C |
| # 120 | # B | # 335 | # D | # 550 | # B | # 765 | # C |
| # 121 | # D | # 336 | # B | # 551 | # B | # 766 | # B |
| # 122 | # B | # 337 | # C | # 552 | # B | # 767 | # B |
| # 123 | # B | # 338 | # B | # 553 | # A | # 768 | # A |
| # 124 | # B | # 339 | # D | # 554 | # B | # 769 | # D |
| # 125 | # B | # 340 | # B | # 555 | # A | # 770 | # D |
| # 126 | # C | # 341 | # A | # 556 | # C | # 771 | # A |
| # 127 | # C | # 342 | # B | # 557 | # D | # 772 | # C |
| # 128 | # B | # 343 | # B | # 558 | # A | # 773 | # D |
| # 129 | # B | # 344 | # B | # 559 | # C | # 774 | # A |
| # 130 | # B | # 345 | # B | # 560 | # B | # 775 | # B |
| # 131 | # B | # 346 | # B | # 561 | # C | # 776 | # D |
| # 132 | # B | # 347 | # C | # 562 | # A | # 777 | # C |
| # 133 | # B | # 348 | # A | # 563 | # B | # 778 | # B |
| # 134 | # B | # 349 | # B | # 564 | # B | # 779 | # D |
| # 135 | # A | # 350 | # A | # 565 | # C | # 780 | # C |
| # 136 | # B | # 351 | # C | # 566 | # B | # 781 | # A |
| # 137 | # B | # 352 | # C | # 567 | # C | # 782 | # B |
| # 138 | # A | # 353 | # A | # 568 | # B | # 783 | # C |
| # 139 | # B | # 354 | # C | # 569 | # C | # 784 | # A |
| # 140 | # B | # 355 | # A | # 570 | # B | # 785 | # C |
| # 141 | # D | # 356 | # A | # 571 | # B | # 786 | # B |
| # 142 | # B | # 357 | # C | # 572 | # B | # 787 | # C |
| # 143 | # B | # 358 | # A | # 573 | # B | # 788 | # A |
| # 144 | # C | # 359 | # B | # 574 | # B | # 789 | # C |
| # 145 | # A | # 360 | # A | # 575 | # B | # 790 | # D |
| # 146 | # B | # 361 | # D | # 576 | # A | # 791 | # C |
| # 147 | # B | # 362 | # B | # 577 | # C | # 792 | # B |
| # 148 | # B | # 363 | # B | # 578 | # A | # 793 | # D |
| # 149 | # B | # 364 | # B | # 579 | # B | # 794 | # A |
| # 150 | # B | # 365 | # A | # 580 | # B | # 795 | # B |
| # 151 | # B | # 366 | # B | # 581 | # A | # 796 | # C |
| # 152 | # A | # 367 | # B | # 582 | # A | # 797 | # D |
| # 153 | # B | # 368 | # B | # 583 | # B | # 798 | # B |
| # 154 | # B | # 369 | # A | # 584 | # D | # 799 | # C |
| # 155 | # A | # 370 | # C | # 585 | # C | # 800 | # A |
| # 156 | # C | # 371 | # C | # 586 | # B | # 801 | # B |
| # 157 | # B | # 372 | # B | # 587 | # A | # 802 | # C |
| # 158 | # B | # 373 | # B | # 588 | # A | # 803 | # D |
| # 159 | # B | # 374 | # B | # 589 | # B | # 804 | # A |
| # 160 | # B | # 375 | # B | # 590 | # B | # 805 | # B |
| # 161 | # C | # 376 | # A | # 591 | # B | # 806 | # C |
| # 162 | # C | # 377 | # D | # 592 | # C | # 807 | # D |
| # 163 | # C | # 378 | # D | # 593 | # B | # 808 | # B |
| # 164 | # D | # 379 | # C | # 594 | # B | # 809 | # C |
| # 165 | # A | # 380 | # C | # 595 | # B | # 810 | # B |
| # 166 | # D | # 381 | # C | # 596 | # B | # 811 | # D |
| # 167 | # B | # 382 | # D | # 597 | # B | # 812 | # A |
| # 168 | # D | # 383 | # B | # 598 | # B | # 813 | # B |
| # 169 | # C | # 384 | # D | # 599 | # B | # 814 | # C |
| # 170 | # A | # 385 | # A | # 600 | # B | # 815 | # A |
| # 171 | # B | # 386 | # B | # 601 | # C | # 816 | # D |
| # 172 | # B | # 387 | # C | # 602 | # A | # 817 | # B |
| # 173 | # B | # 388 | # B | # 603 | # B | # 818 | # A |
| # 174 | # A | # 389 | # C | # 604 | # C | # 819 | # B |
| # 175 | # A | # 390 | # D | # 605 | # B | # 820 | # D |
| # 176 | # B | # 391 | # D | # 606 | # C | # 821 | # E |
| # 177 | # C | # 392 | # D | # 607 | # B | # 822 | # A |
| # 178 | # D | # 393 | # A | # 608 | # C | # 823 | # E |
| # 179 | # C | # 394 | # B | # 609 | # A | # 824 | # A |
| # 180 | # D | # 395 | # C | # 610 | # C | # 825 | # B |
| # 181 | # B | # 396 | # C | # 611 | # C | # 826 | # C |
| # 182 | # D | # 397 | # A | # 612 | # C | # 827 | # D |
| # 183 | # D | # 398 | # A | # 613 | # A | # 828 | # B |
| # 184 | # C | # 399 | # A | # 614 | # C | # 829 | # D |
| # 185 | # B | # 400 | # A | # 615 | # B | # 830 | # B |
| # 186 | # D | # 401 | # B | # 616 | # A | # 831 | # B |
| # 187 | # D | # 402 | # D | # 617 | # C | # 832 | # D |
| # 188 | # D | # 403 | # A | # 618 | # C | # 833 | # D |
| # 189 | # A | # 404 | # D | # 619 | # C | # 834 | # C |
| # 190 | # D | # 405 | # C | # 620 | # B | # 835 | # D |
| # 191 | # B | # 406 | # C | # 621 | # B | # 836 | # A |
| # 192 | # A | # 407 | # D | # 622 | # C | # 837 | # C |
| # 193 | # A | # 408 | # D | # 623 | # B | # 838 | # D |
| # 194 | # D | # 409 | # D | # 624 | # B | # 839 | # D |
| # 195 | # C | # 410 | # B | # 625 | # D | # 840 | # C |
| # 196 | # A | # 411 | # C | # 626 | # A | # 841 | # A |
| # 197 | # B | # 412 | # A | # 627 | # B | # 842 | # A |
| # 198 | # B | # 413 | # A | # 628 | # D | # 843 | # E |
| # 199 | # A | # 414 | # C | # 629 | # C | # 844 | # A |
| # 200 | # B | # 415 | # B | # 630 | # B | # 845 | # D |
| # 201 | # D | # 416 | # C | # 631 | # C | # 846 | # D |
| # 202 | # B | # 417 | # B | # 632 | # B | # 847 | # D |
| # 203 | # A | # 418 | # B | # 633 | # B | # 848 | # D |
| # 204 | # C | # 419 | # D | # 634 | # C | # 849 | # C |
| # 205 | # B | # 420 | # D | # 635 | # C | # 850 | # D |
| # 206 | # B | # 421 | # C | # 636 | # C | # 851 | # B |
| # 207 | # C | # 422 | # C | # 637 | # B | # 852 | # A |
| # 208 | # B | # 423 | # B | # 638 | # B | # 853 | # A |
| # 209 | # B | # 424 | # C | # 639 | # B | # 854 | # A |
| # 210 | # B | # 425 | # B | # 640 | # B | # 855 | # D |
| # 211 | # B | # 426 | # D | # 641 | # D | # 856 | # D |
| # 212 | # A | # 427 | # A | # 642 | # A | # 857 | # D |
| # 213 | # A | # 428 | # A | # 643 | # B | # 858 | # C |
| # 214 | # B | # 429 | # D | # 644 | # B | # 859 | # B |
| # 215 | # C | # 430 | # B | # 645 | # B | # 860 | # D |
|  |  |  |  |  |  | # 861 | # C |