Building Construction model exam multiple choice questions

- Which of the following is NOT a characteristic of Type 4 (Heavy Timber) construction?

- A. Exterior bearing walls (0–1 hrs)

- B. Floor construction (0–1 hrs)

- C. Roof construction (0–1 hrs)

- D. Structural frames (0–1 hrs)

Answer: A

- Which one is different from the others?

- Pile foundation

- . Mat foundation

- Well foundation

- . Pier foundation

Answer: B

- Which type of roof sheet material is highly ductile and malleable, allowing it to be cold-worked into a required profile?

- A. Aluminum Sheet

- B. Fiber Cement Profiled Sheets

- C. Thatch roof coverings

- D. Copper Sheet

- E. All of the above

Answer: D

- Which one is different from the others?

- A. Elevator

- B. Lifts

- C. Ramps

- D. Escalator

- E. None

Answer: C

- Which type of roof slopes in four directions toward the walls?

- Pitched Roof

- Shed Roof

- Gable Roof

- Hip Roof

- All of the above

Answer: D

- Which type of lintel is preferred over all others due to its strength, rigidity, fire resistance, and economy?

- A. Stone lintels

- B. Brick lintels

- C. Steel lintels

- D. Reinforced concrete lintels

Answer: D

- Which one of the following roof elements runs between the eaves and ridge?

- Hip

- Truss

- Valley

- Verge

- None

Answer: C

- is the process of finishing of mortar joints in exposed brick or stone masonry.

- Painting 	B. Distempering   C. White washing      D. Pointing

Answer: D

- Which one of the following is true about Gauge?

- G-30< G-24 in thickness

- G-24 > G-26 in length

- G-24 > G-26 in thickness

- G-28< G-30 in length

- None

Answer: C

- Which is not the element of a trussed roof system?

- Trusses

- Purlins

- Girder

- Rafters

- None

Answer: C

- Which one of the following is not true about elements of the staircases?

A. Nosing is the projecting part of the tread beyond the face of the riser.

B. String is the structural member which supports the steps and act as inclined beams.

C. Winders are tapering steps which are provided for changing the direction of stair.

D. Soffit is the underside of the stair.

E. None of the above

Answer: E

- Which one of the following is true about different types of damp prevention mechanisms?

- Adding an impervious layer of rich cement mortar over the exposed surfaces for resisting water pressure as a process of pressure grouting.

- Use of integral damp proofing means using compounds like alkaline silicates, aluminum sulphate, calcium chlorides, etc, which react chemically with concrete to produce water proof concrete.

- Damp proofing Material should not be flexible, so that it can accommodate the structural movements without any fracture.

- Guniting is a process that consists of forcing cement grout, under pressure, into cracks, voids, fissures etc.

- None of the above

Answer: B

- From different types of tests carried out for Building Stones how do we determine the toughness of a stone Toughness index is determined?

A. Crushing Test

B. Attrition Test

C. Impact Test

D. Hardness test

E. None

Answer: C

- Is the Vertical member of a frame, which is employed to sub-divide a window or a door vertically?

- Panel

- B.  Transom

- C.  Mullion

- D. Style

Answer: C

- From the production process of brick, the following actions are performed, but Which of the following is the correct process of production?

A. Tempering > crack control > Blending > sintering

B. Sintering > Blending > tempering > crack control

C. Blending > tempering > crack control > sintering

D. Tempering > sintering > Blending > crack control

Answer: C

- From the following one is not the disadvantage of flat slab flooring, identify?

A. Low shear capacity

B. Low Stiffness

C. Exposed flat ceilings

D. None

Answer: C

- From the following one is obtained by cutting the triangular piece between the center of one end and the center of the other side.

A. King closser

B. Queen Closser

C. Bevelled Closser

D. Mitered Closser

Answer: A

- From the following one is not among the rules of bonding.

      - The brick should be of uniform size. The length of the brick should be twice its width plus one joint.

      - The amount of lap should be minimum ¼ brick along the length of the wall and ½ brick across the thickness of the wall.

      - In alternate courses, the center line of header should coincide with the center line of the stretcher, in the course below or above it.

      - None

Answer: D

- Which of the following is NOT a common cause of foundation failure?

- Unequal settlement of subsoil

- equal height of elevation columns

- Lateral pressure on the walls

- Atmospheric action

- None

Answer: B

- Which of the following does NOT affect the strength of a masonry wall?

- Quality of masonry material

- Quality of water

- Workmanship

- Quality of mortar

- None

Answer: E

- A _______ is a continuous set of steps between floors or landings.

- Flight

- Flier

- Soffit

- Tread

- Run

Answer: A

- Which structural component is used to support the floor covering?

- Floor base

- Beam

- Upper floor

- Solid floors

- Flat Floor

Answer: A

- Which type of masonry consists of blocks of accurately dressed stone with extremely fine joints?

- Rubble Masonry

- Ashlar Masonry

- Brick Masonry

- Concrete Masonry

- None

Answer: B

- Which drawing is used to show a small part of the construction at a larger scale?

- Site Drawing

- Sectional Drawing

- Elevation Drawing

- Detail Drawing

Answer: D