- Which situation represents a defect in consent?

  - Contractor miscalculates cost escalation

  - Contractor signs under threat of losing license

  - Contractor provides bid bond

  - Contractor fails to provide retention security

- Which of the following situations MOST strongly justifies the use of Force Account delivery system?

  - When design responsibility must be separated from construction

  - When private sector lacks capacity for technologically complex projects

  - When competitive bidding is required by public accountability

  - When the project requires early contractor involvement

- Which procurement method is MOST likely to reduce fragmentation between design and construction responsibilities?

  - Design Bid Build

  - Force Account

  - Design Build

  - Serial Tendering

- A Project Manager uses a Descriptive Specification for a high-performance waterproof coating. The contractor follows the material mix and application method exactly as described in the spec. However, the roof slab leaks after the first rain.

  - The Contractor is liable because they didn't achieve the "result."

  - The Engineer/Owner is liable because the spec specified the "method" rather than the "result."

  - Liability is split 50/50.

  - The Contractor is liable for not providing a "Performance Specification" instead.

- Which condition allows local firms to win even when their bid is not the lowest?

  - Prequalification margin

  - Preference margin

  - Serial tender adjustment

  - Advance payment compensation

- A contractor finds the drawing specifies slab thickness = 150 mm, but the BOQ states slab thickness = 175 mm and concrete grade C25. What should the contractor do?

  - Follow BOQ

  - Follow drawings

  - Follow specification and raise variation

  - Use average thickness

- Which statement BEST explains the relationship between technical and financial evaluation in two-stage tendering?

  - Financial evaluation determines technical compliance

  - Technical evaluation precedes financial evaluation

  - Financial evaluation eliminates need for technical review

  - Both evaluations occur simultaneously without separation

- In a standard Take-off sheet, what is the specific purpose of "Column 1"?

  - To enter the dimensions (Length, Width, Height)

  - To state the number of times an item occurs

  - To multiply dimensions to determine the quantity

  - To provide a brief description of the work item

- Which condition would make a contract legally INVALID even if all parties signed willingly?

  - Absence of written form

  - Lack of defined object

  - Absence of payment certificate

  - Lack of performance bond

- Which procurement delivery system allows the contractor to recover investment through operation of the facility?

  - Design Bid Build

  - BOT

  - Outsourcing

  - Alliances

- Which of the following words should replace “must” when specifying contractor duties?

  - Will

  - Shall

  - Should

  - May

- Which contract type exposes the contractor to the highest risk of quantity estimation errors?

  - Unit rate contract

  - Cost plus fixed fee

  - Lump sum fixed price contract

  - Target cost contract

- Specification states: “Provide adequate compaction.” Why is this incorrect?

  - Too short

  - Lacks measurement reference

  - Word “adequate” is indefinite

  - Should be in drawing

- Which bond primarily guarantees subcontractors and suppliers will be paid?

  - Bid bond

  - Payment bond

  - Advance payment bond

  - Retention bond

- In quantity surveying, “taking-off” refers to:

  - Finalizing the BOQ

  - Measuring quantities from drawings

  - Checking contractor’s invoice

  - Preparing bar schedule

- In arithmetic review, which rule is applied when a unit price is left blank?

  - Bid is automatically rejected

  - Price is assumed zero

  - Price is assumed covered elsewhere in the bid

  - Price is replaced with engineer’s estimate

- Minimum removal time for soffit formwork to slab is:

  - 14 days

  - 16 hours

  - 7 days

  - 21 days

- Which event marks the beginning of the Defects Liability Period?

  - Interim payment certificate

  - Variation order approval

  - Certificate of provisional acceptance

  - Final account settlement

- In preliminary estimation, cost per bed in a hospital is an example of:

  - Plinth area method

  - Cubical content method

  - Functional unit method

  - Item rate method

- Which damage type may be suffered by the contractor due to employer delay?

  - Liquidated damages

  - Idle labour cost

  - Defective performance penalty

  - Abandonment liability

- Which option BEST explains why Open Tendering may lead to poor project performance despite producing lower prices?

  - Lack of financial evaluation criteria

  - High probability of collusion among contractors

  - Participation of contractors with inadequate managerial capacity

  - Absence of performance bonds

- Which of the following is considered an "Indirect Construction Cost"?

  - Labor wages for painters

  - Material costs including waste

  - Office running expenses like telephone and internet

  - Rental rates for equipment used on a specific activity

- During a mandatory pre-bid site visit, a contractor discovers that there is no existing access road to the project site, and the bidding documents do not include any BOQ item for access road construction. What is the most appropriate action the contractor should take before submitting the bid?

  - Estimate the cost of constructing the access road and proportionally distribute it among the BOQ items

  - Ignore the issue for now and request additional payment after award

  - Request the Employer to issue an addendum revising the BOQ

  - None of the above

- Which bond ensures the contractor will enter into contract at the offered bid price?

  - Payment bond

  - Performance bond

  - Bid bond

  - Retention guarantee