- The ability of material to resist deformation in elastic range  is measured in

- Toughness 		 B. Stiffness		C. Ductility 		D. All

- Which one is a ductile material

- Cast Iron		B. Stone		C. Glass			D. Structural steel

- A permanent deformation remains in the stressed body after complete removal of its load is:

- Elastic Range 	B. Fracture		C. Plastic Range 	D. none

- Which  one of the following statement is false

- The greater  modulus of elasticity, the smaller the elastic strain

- The greater  modulus of elasticity, the stiffer material

- Modulus of elasticity is slope of non-linear part of stress –strain diagram

- Compression and tension tests are mechanical tests of materials

- When forces are applied to a solid body, the internal force and deformation are developed are called ----------- and----------- respectively.

- Stress, Strain	 B. Stress, Yield	         C. Strain, Stress 	    D. Yield, Strain

- -------- are cements which are neither able to set nor able to hardener be stable in water.

A. Non- hydraulic cements        				C. Non -hydraulic lime

B. Gypsum Plasters       					D. Portland cement   	E. All except

- Pure Gypsum is

A.  Alabaster                     B. Hydrous Gypsum	             C. Plaster of Paris              D. None

- A most abundant cement clinker compound contributes to early strength gain is:

A. C4AF                            B. C3S                                      C.C2S                                  D.C3A

- When water reacts to tri-calcium silicates,--------is formed.

A. C-S-H gel                    B. Ca(OH)2                              C. Gypsum                           D.A&B

- Setting time of cement increased by adding:

A. Lime                         B. Gypsum                                C. Tobermorite                     D. NaOH

- Ultimate strength is gained by

A. C3S                          B.C2S                                           C.C3A                                    D.C4AF

- Which type of cement is recommended in areas where sulphate attack is severe:

A. Type V                    B. Type   I                                    C. Type II                              D. Type IV

- Low heat cement is obtained as a result of:

A. Decreasing C3A amount                                                               C. Decreasing C2S amount

B. Increasing C3S amount                                                                 D. None

- Excess amount of free lime cause:

A. Ettringite                       B. Unsoundness            C. Slaked lime		D.All

- A cement type used in cold construction environment is:

A. Type III                          B.Type II                          C. TypeV                          D. Type I

- Rate of hydration of cement affected by:

A. cement composition			C. Age of paste

B. Fineness of cement                                                             D. All

- For the same proportions, one of the following mortar types gives more workable mix than others

A. Mud Mortar                                                                  C. Cement Mortar

B. Lime Mortar                                                                  D. Compo mortar

- Impurities in concrete mixing water may not cause:

A. Efflorescence             B. Enhanced Strength   C. Volume change D. corrosion of reinforcement

- Which one is not a correct statement

A. Well-graded aggregate reduces cement paste

B. Gradation of aggregate affects workability of cement

C. Angular aggregate gives higher strength

D. Aggregate used for concrete should be flat and elongated

- Organic impurities in fine aggregate is tested by:

A. Vicat Plunger                         B. Jar                C. Le Chatelier           D. Manometer

- An aggregate moisture condition (state) in which all moisture in the aggregates are removed is:

- Air dry     	      B. Saturated surface dry               C. Oven dry		  D. Moist

- A building constructed at a place exposed to freeze and thaw. Which type of admixture is more appropriate to improve durability of concrete?

A. accelerator Admixture 		     B. Air Entraining Admixture

C. Retarding admixture		    D. Super plasticizer Admixture

- A phenomenon of increase in sand volume due to the increase of moisture content.

A. Bulking                               B. Creep	                            C. Abrasion           D. Bleeding

- Sieve analysis is performed to test:

A. Fineness of cement        B. Gradation of aggregate        C.  Strength of concrete    D. None

- Dropping fresh concrete from a height results:

A. Consistency                    B. Segregation          C. Workability                 D. Bleeding

- Which one is a hardened property of concrete:

A. workability            B. Segregation                     C. Strength                      D. All

- Which one is not correct about the requirement limits on the nominal maximum aggregate size as stated in ACI 2000 limit?

- The nominal maximum size of aggregate should not exceed one-fifth of the narrowest dimension of a vertical concrete member.

- The nominal maximum size of aggregate should not less than one-fifth the narrowest dimension of a vertical concrete member.

- The nominal maximum size of aggregate should not exceed three-quarters the clear spacing between reinforcing bars and forms.

- The nominal maximum size of aggregate should not exceed one-third the depth of slabs

Answer Key

| 1. B | 2.D | 3.C | 4.C | 5.A | 6.E | 7.A | 8.B | 9.D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10.B | 11.B | 12.A | 13.A | 14.B | 15.A | 16.D | 17.B | 18.B |
| 19.D | 20.B | 21.C | 22.B | 23.A | 24.B | 25.B | 26.C | 27.B |