- Which of the following is NOT a typical application area of soil mechanics?

- Foundation design

- Slope stability analysis

- Pavement design

- Bridge aesthetics

- Retaining wall design

- None

- The process of soil formation from parent rock is known as:

- Sedimentation

- Weathering

- Erosion

- Compaction

- Cementation

- None

- Residual soil is produced by;

- Deposition of river sediments

- Glacial transport and deposition

- Mechanical and chemical weathering of bedrock in place

- Volcanic ash settling on the ground

- Wind-blown sand accumulation

- None

- Which of the following is a clay mineral?

- Quartz

- Feldspar

- Calcite

- Kaolinite

- A well-graded soil has:

- Uniform particle sizes

- A wide range of particle sizes

- Only fine particles

- Only coarse particles

- A soil sample has a liquid limit of 60% and a plastic limit of 40%. Then, its plasticity index is:

- 20%

- 100%

- 50%

- 20%

- The Atterberg limits are used to describe:

- Soil compaction characteristics

- Permeability of soils

- Consistency of fine-grained soils

- Shear strength of soils

- Soil color variations

- According to the Unified Soil Classification System (USCS), a well-graded sand with no fines is designated as;

- SP

- SW

- SM

- ML

- In the AASHTO soil classification, a soil classified as group A-7 is typically;

- An organic-rich peat soil

- A well-graded gravel

- A high-plasticity clay or silty clay

- A clean sand with no fines

- Darcy's law is valid for;

- Turbulent flow in soils

- Steady flow in saturated soils

- Flow in unsaturated soils

- Flow through rocks

- The coefficient of permeability (k) has units of;

- cm/s

- cm²/s

- s/cm

- cm³/s

- Which soil typically has the highest hydraulic conductivity;

- Clay

- Silt

- Sand

- Gravel

- The governing equation for steady two-dimensional flow of groundwater in a homogeneous isotropic soil is;

- Bernoulli’s equation

- Laplace’s equation

- Navier–Stokes equation

- Darcy’s law equation

- Which method is commonly used to analyze two-dimensional seepage through earth dams?

- Darcy’s Law

- Flow net method

- Terzaghi’s theory

- Boussinesq equation

- In a flow net, the flow lines and equipotential lines are always;

- parallel to each other

- Perpendicular (orthogonal) to each other

- Coincident (on top of each other)

- At 45° to each other

- Randomly oriented

- None

- In a one-dimensional oedometer test (no lateral strain), the lateral confinement implies an effective Poisson’s ratio of;

- 0.0

- 0.25

- 0.33

- 0.50

- Total stress at any depth in soil is calculated as;

- Effective stress × pore pressure

- Unit weight × depth

- Seepage pressure × depth

- Elastic modulus × strain

- The modulus of elasticity of soil depends primarily on;

- Soil color

- Atterberg limits

- Effective stress and strain

- Permeability

- Shrinkage limit

- None

- Which of the following causes immediate settlement?

- Clay consolidation

- Elastic deformation of dry sands

- Secondary compression

- Seepage

- One-dimensional consolidation of soil under a static load primarily involves;

- A. expulsion of air from the pores

- Gradual expulsion of pore water and increase in effective stress

- Immediate elastic deformation only

- Increase in total stress with no change in pore pressure

- An oedometer test is used to determine;

- The consolidation settlement characteristics of a soil sample under vertical loading

- The shear strength of a soil under triaxial conditions

- The specific gravity of soil

- The Atterberg limits of a soil

- Settlement of soil due to consolidation is highest in;

- Dry sands

- Gravel beds

- Saturated clay

- Unsaturated silts

Answer Key

- D

- B

- C

- D

- B

- A

- C

- B

- C

- B

- A

- D

- B

- B

- B

- A

- B

- C

- B

- B

- A

- C