Hydraulic Structure I Exit Exam Model Questions

- A foundation investigation reveals the presence of a pervious alluvial layer extending 15 meters below the riverbed. A concrete gravity dam is to be constructed. Which foundation treatment technique is most appropriate for controlling under-seepage and preventing piping?

- Surface compaction of the alluvial layer

- Installation of a deep concrete cutoff wall extending to bedrock

- Placement of a thick riprap layer on the upstream bed

- Pressure grouting of the top 2 meters of the foundation

- The selection of a dam site is a critical first step. Which of the following is the most desirable condition for a dam site?

- A wide, open valley to allow for a long, low-cost spillway

- A narrow valley with competent rock formations in the abutments and foundation

- A location with deep, pervious alluvial deposits for easy excavation

- A site with a highly meandering river upstream to increase the reservoir length

- Understanding the principles of reservoir planning is essential. What is the primary purpose of preparing an area-elevation-storage curve?

- To determine the cost of land acquisition for the reservoir

- To calculate the maximum flood discharge, the dam must pass

- To estimate the volume of water stored at different water levels within the reservoir

- To analyze the chemical composition of the water in the reservoir

- An embankment dam is being designed on a pervious foundation. A key design feature to control seepage and prevent piping is a:

- Concrete cutoff wall extending into the impervious layer

- Steep upstream slope to maximize the dam's weight

- Central, impervious core connected to a cutoff trench

- Rockfill zone on the downstream side with no filter

- A concrete gravity dam is subjected to uplift pressure at its base. To reduce this uplift, which of the following is the most common design feature?

- A curved dam axis to increase structural strength

- A series of drainage holes (drainage gallery) near the upstream face

- A heavily reinforced concrete toe slab on the downstream side

- A flatter downstream slope to increase the dam's self-weight

- A homogeneous earth dam is to be constructed from a single material. Which of the following is its primary limitation?

- It is the most expensive type of embankment dam to construct.

- It is only suitable for sites with exceptionally strong foundations.

- It may be susceptible to rapid drawdown instability in the upstream slope.

- It cannot be built with a spillway, requiring a separate structure.

- A piezometer installed in an embankment dam is specifically used to measure:

- The total stress within the dam's foundation

- The rate of erosion of the dam's downstream face

- The pore water pressure within the dam or its foundation

- The horizontal movement of the dam's crest during loading

- You are tasked with designing the foundation treatment for a new dam. The foundation consists of highly fractured rock. Which technique is most suitable for creating an impervious barrier and consolidating the rock?

- Excavating a wide, shallow trench and backfilling with compacted clay

- Pressure grouting by injecting a cementitious material into the fractures

- Constructing a massive concrete keyway that extends the full width of the valley

- Removing all the overburden down to a depth of 50 meters

- During a routine inspection, you observe localized seepage exiting the downstream face of an embankment dam that is carrying fine soil particles. This condition is known as:

- Transpiration, which is a normal, harmless process

- Piping, which can lead to a catastrophic failure if not addressed

- Consolidation, which indicates the dam is settling uniformly

- Abutment leakage, which only occurs at the sides of the dam

- An engineer is analyzing a concrete gravity dam for sliding stability. Which of the following forces would be considered a resisting force in this analysis?

- The uplift force acting on the base of the dam

- The horizontal force from water pressure on the upstream face

- The frictional resistance developed between the dam and its foundation

- The hydrostatic pressure on the downstream face

- Which one of the following force type will not act on elementary profile of gravity dam

- Self-Weight      B)  Water Pressure    C) Seismic force  D) Uplift Force

- Which one of the following is the function of instruments to check safety of the dam?

- Measuring water pressure        C) Measuring structural movement

- Measuring seepage flow          D) Measuring temperature          E) All

- When will not occur Crushing and tension failure of gravity dam?

- When the stress developed at any point in the dam exceed the strength of concrete

- When tensile stress exceeding tensile strength of concrete

- When Compressive stress exceeding compressive strength of concrete

- When the stress developed at the foundation is less than bearing capacity of the foundation

- Which one of the following is not correct about Arch dam design concept

- Uplift fore force is the major dominant force during the design of arch dam

- Elastic arch theory assumes complete transfer of load by arch action only.

- Thin cylinder theory assumes arch is simply supported at the abutments and the stresses are same as in thin cylinder of equal outside radius

- Failure of arch dam can occur as a result of overstress.

- One of the following concept is correct about Modification of elementary profile of gravity dam

- Top width must be provided to resist the force due to accidental loading and impact of floating debris

- The water may overtop over the dam when the waves develop in the reservoir

- Because there are different type of forces which may act on gravity dam that should be considered

- all

- Where do we get appropriate and raw hydrological data for analyzing design discharge of ungauged river in order to design hydraulic structure?

- Ethiopian Mapping Agency                             C) Ethiopian Road Authority

- Ethiopian Geological Survey                          D) Ethiopian Metrological Agency

- Embankment dams will  fail:-

- When there is sufficient  free board

- When Excessive settlement of the foundation and dam will not occur

- When there is underestimated design flood

- When the spillway capacity is  adequate and having properly operated gate

- Which one of the following is not correct  about seepage analysis in an embankment dam

- To facilitate movement of water as erosion within the dam body

- To determine the quantity of water passing through the dam body

- To obtain the distribution of pore water pressure.

- To determine the quantity of water passing through the foundation of dam

- To check healthiness of the dam

- Which one of the following parameter  will not govern the design of an earth dam

- Sufficient Base width is required to prevent overstress in the foundation

- Side slopes must not be stable;

- Dimensions must be sufficient to control seepage;

- All design parameters have to be considered

- One of the following is not the function of dam

- Constructed to raise water level of the river for increasing flood discharge problem

- Constructed to retain debris such as sand, gravel  and floating woods

- Constructed to impound surplus flood water for use in dry period

- Constructed to divert the water during period of construction

- which one of the following is not correct about characteristics of dam

- Concrete dams are suitable to the site topography of wide component rock foundation

- Compacted unit costs of concrete dams are very much lower than embankment dams

- Traditional mass concrete construction is labour intensive, discontinuous and requires certain skills.

- Embankment dams can accommodate separate spillway

- Which one of the following is not a characteristics of the disadvantages of an embankment dam

- Embankment dams are more earthquake resistance than gravity dams

- Susceptible to damage by overtopping or burrowing of animals

- Suitable for narrow gorges with steep slopes

- Fails suddenly without any sign of imminent failure

- One of the force type will not have equal impact on different types of dam (0.5 marks)

- Self-weight       B)  Hydrostatic force        C) Silt load

D) Uplift force        E) Pore water pressure      F) none of the above

- One of the following parameter will govern dam site selection

- Accessibility     C) Environmental Impact   D) Submergence area

- Foundation condition   E) All are governing parameters

- Which one of the following structure will not  be built when dam structure is constructed

- Roads and Bridges structures       C) Intake and gate structures

- Outlet Structures                           D) Spillway structure     E) None of the above

Answer Key

- B

- B

- C

- C

- B

- C

- C

- B

- B

- C

- C

- E

- B

- A

- D

- D

- C

- A

- B

- A

- B

- C

- D

- E

- E