- According to the ERA 2013 design manual, how is the appropriate pavement thickness determined?

- By estimating construction cost of each layer

- By considering traffic loading, subgrade strength, and environmental factors

- By considering traffic loading, and subgrade strength

- By considering traffic loading, subgrade strength, reliability, and environmental factors

- Which is the correct sequence in flexible pavement construction?

- Wearing course → Binder course →Subgrade → Base→ Sub-base

- Capping layer → Sub-base → Base → Binder course → Wearing course

- Base → Sub-base → Subgrade → Binder course → Wearing course

- Subgrade → Base →Sub-base → Binder course → Wearing course

- What does the penetration test of bitumen measure?

- Viscosity of bitumen

- Hardness or consistency of bitumen

- Elasticity of bitumen

- Temperature susceptibility

- Which test determines whether aggregates shapes are suitable for pavement construction?

- Los Angeles abrasion test

- Flakiness and elongation index test

- Impact value test

- Soundness test

- Which property of bitumen is most important for resisting rutting at high temperatures?

- Penetration

- Softening point

- Ductility

- Flash point

- What is the primary function of expansion joints in rigid pavements?

- Increase pavement thickness

- Allow for thermal expansion and contraction

- Improve surface texture and skid resistance

- Allow for construction continuation and contraction

- Marshall stability of a mixture indicates:

- Workability of asphalt mix

- Resistance to plastic deformation under load

- Durability and performance of the mix

- Moisture content and volume of the mix

- What is the main function of the sub-base layer in flexible pavement?

- Provide smooth riding surface

- Distribute loads and improve drainage

- Increase subgrade layer strength

- Increase traffic load resistance

- Skid resistance of pavement surface mainly depends on:

- Pavement thickness

- Surface texture and aggregate properties

- Pavement layers strength

- The amount of bitumen content

- Which method is commonly used to check pavement layer thickness during construction?

- Core cutting method

- Asphalt extraction test

- Penetration test

- Compaction test

- Which one of the following is the primary loading factor considered for pavement design?

- Traffic volume and composition

- Traffic loading

- Lane distribution factor

- Traffic composition and growth rate

- Which test evaluates aggregate resistance to abrasion?

- Impact value test

- Soundness test

- Los Angeles abrasion test

- Crushing test

- Flaky aggregates are undesirable because they:

- Increase strength

- Break easily under load

- Improve interlocking

- Creates flat surfaces

- What is the main purpose of the ductility test of bitumen?

- Measure hardness

- Measure tensile deformation capacity

- Determine viscosity

- Determine flash point

- Which test is used to determine moisture susceptibility of asphalt mix?

- Marshall test

- Stripping test

- Binder content test

- Core cutting test

- Which factor is most critical when selecting a stabilization technique?

- Contractor preference

- Soil type and engineering properties

- Road type

- Project location only

- Which one of the following is true?

- Mechanical stabilization uses additives, while chemical does not

- Chemical stabilization relies on gradation improvement only

- Mechanical stabilization improves soil by blending and compaction, while chemical

stabilization uses binders like cement or lime

- Both mechanical and chemical methods are identical

- Which stabilization method is most suitable for improving highly plastic clay subgrade soils?

- Bituminous stabilization

- Cement stabilization

- Lime stabilization

- Mechanical stabilization

- Flexible pavements distribute loads through:

- Slab action

- Layered system

- Beam action

- Dowel and transverse bars

- Rigid pavements primarily rely on:

- Subgrade strength

- Slab action and flexural strength

- Aggregate interlock

- The number and spacing of joints

- Fatigue cracking in flexible pavement is caused by:

- Temperature variation

- Repeated traffic loading

- Poor drainage condition

- Skid resistance

- Temperature cracking occurs due to:

- Traffic load

- Thermal contraction

- Moisture

- Over compaction

- Design life of pavement depends on:

- Weather condition of the environment alone

- Traffic and environmental factors

- The number of pickups in the traffic composition

- Contractor experience and number of expertise

- Which of the following is a key step in the Marshall Mix Design method?

- Measuring pavement layer thickness

- Preparing asphalt specimens and testing for stability and flow

- Measuring moisture content of the mixture

- Determining the optimum bitumen content