- What type of traffic control device is used to alert drivers about potential hazards or changes in road conditions ahead?

    - Regulatory

    - Warning

    - Informational

    - Guide

- Which of the following best defines traffic flow theory?

- Study of pedestrian movement

- Mathematical study of traffic stream behavior

- Design of road geometries

- Analysis of traffic accidents

- Why are speed humps installed on roads?

- To increase vehicle speed

- To slow down traffic

- To divide lanes

- To provide a pedestrian crossing

- In traffic studies, how is the 90th percentile speed defined?

- The speed below which 90% of vehicles travel

- The posted maximum legal speed

- The speed above which 90% of vehicles travel

- The mean speed of all vehicles

- Based on Greenshields' model, if the free-flow speed (uf) is 80 km/h and the critical density (kc) is 40 vehicles per kilometer, what is the maximum traffic flow (qm)?

- 1200 vehicles per hour

- 1600 vehicles per hour

- 2000 vehicles per hour

- 3200 vehicles per hour

- In a standard traffic signal sequence, what does the yellow light indicate and when is it shown?

- It requires vehicles to stop immediately; shown after green

- It signals drivers to get ready to stop; shown after green

- It allows pedestrians to cross; shown after red

- It indicates uninterrupted traffic movement; shown after red

- In modeling travelers’ choice of transport mode, what is the biggest difficulty when multiple options and factors are involved?

- Limited data availability

- Difficulty in integrating socioeconomic factors

- Complexity in capturing individual preferences and trade-offs

- Lack of computational resources

- What does the Level of Service (LOS) of a roadway primarily measure?

- Road construction and maintenance quality

- Traffic flow performance

- Number of road traffic crashes on the road

- Vehicle fuel efficiency

- Which human factor most contributes to rear-end collisions in urban traffic?

- Vehicle type and speed

- Driver following distance and distraction

- Lane and shoulder width

- Pavement type and slope

- Diverted traffic refers to:

- Traffic volume that decreases due to the availability of alternative transportation modes, such as public transit

- Traffic that is rerouted from its original path to an alternate route as a result of congestion, roadworks, or other network changes

- Daily fluctuations in traffic volumes caused by normal variations in travel demand

- New trips generated by land-use developments or economic activities

- Stopping sight distance (SSD) is defined as:

- The distance a vehicle travels from brake application to a complete stop

- The total distance a vehicle travels during perception and braking to stop safely

- The minimum distance maintained between vehicles under normal flow

- The distance a vehicle needs to accelerate to the desired travel speed

- The primary purpose of a modern roundabout is:

- To increase vehicle speed and maximize throughput on intersecting roads

- To reduce conflict points and improve traffic safety at intersections

- To replace traffic signals while maintaining uncontrolled movements

- To provide exclusive lanes for pedestrians and bicycles only

- Double solid yellow centerlines are primarily used to:

- Prohibit passing in both directions due to limited sight distance

- Separate lanes of traffic moving in the same direction

- Indicate pedestrian-only zones along the center of the roadway

- Warn drivers of upcoming traffic signals

- In the four-step transport planning model, the correct sequence is:

- Trip generation → Trip distribution → Mode choice → Traffic assignment

- Traffic assignment → Trip generation → Mode choice → Trip distribution

- Mode choice → Trip generation → Traffic assignment → Trip distribution

- Trip distribution → Traffic assignment → Trip generation → Mode choice

- Intramodality in transportation refers to:

- Seamless integration of different modes of transport for a single journey

- Designing roads for one mode of transport only

- Assigning trips based solely on private vehicle availability

- Maximizing the number of lanes on a highway

- In transportation planning, mobility primarily refers to:

- The ease of reaching desired destinations or activities

- The integration of multiple transport modes in a journey

- The reliability of travel times under congestion

- The ability to move freely and quickly across a network

Answer Key

- B

- B

- B

- A

- B

- B

- C

- B

- B

- B

- B

- B

- A

- A

- A

- D