Model Exit Exam questions for the courses selected under Structural Engineering Chair

For RC I & II

- Why are load factors used in reinforced concrete design?

- To reduce the actual loads acting on the structure

- To account for uncertainties in loads and ensure safety

- To increase the strength of concrete

- To simplify construction procedures

- Minimum reinforcement in beams is provided to:

- Increase ultimate strength

- Avoid brittle failure due to cracking

- Reduce construction cost

- Increase stiffness only

- Moment capacity of a singly reinforced beam is MOST directly proportional to:

- Width only

- Steel area only

- Lever arm (depth) and steel area

- Concrete cover

- Which of the following best explains the role of stirrups?

- Resist bending moment

- Increase compressive strength

- Resist diagonal tension and shear cracks

- Reduce deflection

- An over-reinforced beam fails when:

- Steel yields first

- Concrete crushes before steel yields

- Shear failure occurs

- Deflection exceeds limits

- If the percentage of tensile reinforcement in a beam increases (keeping section same), the neutral axis depth will:

- Decrease

- Increase

- Remain constant

- Move outside the section

- Two beams have identical width and material strength. Beam A has d = 450 mm, Beam B has d = 600 mm. For the same bending moment, which statement is correct?

- Beam B requires more steel

- Beam A requires less steel

- Beam B requires less steel

- Both require the same steel

- In a two-way slab supported on four sides, load is primarily transferred:

- Only in the shorter direction

- Only in the longer direction

- In both directions to all supports

- Directly to columns without bending

- A slab has Self-weight = 3.75 kN/m², Live load = 3 kN/m², and Finishes = 1.25 kN/m². What is the total service load?

- 6.75 kN/m²

- 7.00 kN/m²

- 8.00 kN/m²

- 8.50 kN/m²

- Which of the following is a Serviceability Limit State?

- Collapse due to bending

- Shear failure

- Excessive deflection

- Buckling of column

- Increasing slenderness ratio of a column leads to:

- Increase in axial capacity

- No effect on strength

- Reduction in load carrying capacity

- Increase in stiffness

- Development length is required to:

- Reduce deflection

- Prevent shear failure

- Ensure proper transfer of stress between steel and concrete

- Increase compressive strength

- A slab is classified as a one-way slab when:

- It is supported on all four sides

- Longer span / shorter span > 2

- Longer span / shorter span < 2

- It has equal reinforcement in both directions

- A slab is designed with Ø12 mm bars @ 150 mm spacing. If replaced with Ø14 mm bars, what is the new spacing to maintain the same steel area?

- 100 mm

- 150 mm

- 204 mm

- 267 mm

- Compression reinforcement in a beam is mainly provided to:

- Resist shear

- Increase moment capacity and ductility

- Reduce dead load

- Replace stirrups

- Minimum eccentricity is considered in column design to:

- Reduce axial load

- Account for unavoidable construction imperfections

- Increase concrete strength

- Eliminate bending moment

- Which is the MOST effective method to reduce deflection in beams?

- Increase reinforcement only

- Increase span

- Increase effective depth

- Reduce concrete strength

- In continuous reinforced concrete beams, moment redistribution may be permitted in design. Under what condition is such redistribution considered acceptable?

- When the beam is over-reinforced

- When the section can undergo sufficient rotation without failing suddenly in critical areas

- When high strength concrete is used throughout the beam

- Only when the beam is of single span

- If a reinforcing bar does not have sufficient anchorage length within the concrete, what do you think would happen in a practical situation?

- The surrounding concrete would gradually compensate and acquire additional strength

- The bar may slip or pull out before reaching its full capacity, resulting in bond failure

- The bar would be more susceptible to corrosion

- The bar would still attain its yield point without any slip occurring

- When considering the behavior of flat slab systems in buildings, which of the following statements is not correct?

- Loads are transferred directly from the slab to the supporting columns

- Drop panels or column capitals can help improve resistance to punching shear

- Punching shear failure of flat slab systems is generally brittle in nature

- Flat slab systems are sufficient in terms of lateral stiffness, and there is no need for shear walls

=================================

For SD course

- Which one of the following is true.

- When wind collides with a structure its potential energy will be converted to kinetic energy.

- Fluctuating wind velocity character is represented by a behavior factor.

- The wind velocity and the velocity pressure are composed of a mean and a fluctuating component.

- The turbulence intensity will increase with increasing wind velocity and equal value for the standard deviation.

- In which ES EN 1991-1-4 Terrain Category will a building experience the highest Peak Velocity Pressure, qp(z), at a given height z>10 m?

- Terrain Category IV (Areas with at least 15% cover of tall buildings).

- Terrain Category II (Area with low vegetation and isolated obstacles).

- Terrain Category III (Areas with regular cover of vegetation or buildings).

- Terrain Category I (Lakes or area with negligible vegetation and without obstacles).

- Positive sign of wind pressure coefficient indicates

- Pressure acting towards the surface

- Pressure acting above the surface

- Pressure acting away from the surface

- Pressure acting below the surface

- Wind pressure at any height of structure does not depend on

- Velocity and density of air

- Angle of wind direction

- Topography of ground surface

- Material of the structure

- Which of the following statements regarding wind loads is not true?

  - Wind pressure varies with building height.

  - Wind pressure varies with building shape and degree of exposure.

  - Suction forces due to wind are very small and therefore negligible.

  - The use of dampers is a means of resisting dynamic effects of wind.

- Under what condition is the Equivalent Static Method (ESM) for seismic analysis, as per ES EN 1998-1, generally permitted for design?

- The design ground acceleration ag is less than 0.1 g.

- The building has a height greater than 60 m and is regular in plan.

- The building is structurally irregular in both plan and elevation.

- The building is regular in plan and elevation, and the fundamental period T1 is below TC or 2 s

- Which of the following factors does not influence earthquake resistance design?

- Geographical location of structure

- Wind of location

- Site soil condition

- Strength of structure

- Which one of the following is incorrect, for the given seismic wave figure below?

- The up-and-down wiggles on the seismogram are variations in air pressure.

- The up-and-down wiggles on the seismogram are variations in ground vibrations.

- Wiggles around C show probably the surface waves.

- Wiggles around A show probably the p-wave.

- In seismic design as per ES EN 1998, the "Design Spectrum" is primarily used to determine:

- The exact time of the next earthquake.

- The elastic forces to be used for design, reduced by a behavior factor (q).

- The maximum possible magnitude of a tectonic shift.

- The soil bearing capacity under static conditions.

- What is the primary distinction between an ES EN 1998-1 Ductility Class Medium (DCM) system and a Ductility Class High (DCH) system?

- DCM systems use a lower behavior factor (q) and have less demanding detailing requirements than DCH systems.

- DCM systems are for Ground Type A/B, while DCH is for Ground Type C/D.

- DCH systems are only permitted for irregular buildings, while DCM is for regular buildings.

- DCH systems are designed to resist wind loads, while DCM systems are only for earthquake loads.

- Seismic weight is typically calculated by taking the sum of the full dead load and:

- 100% of the design live load

- A specific fraction (ψE) of the live load

- Only the weight of the partitions

- The wind load acting simultaneously

- Which structural system is characterized by the use of both moment-resisting frames and shear walls to resist lateral forces?

- Wall system

- Moment resisting frame system

- Dual system

- Core-supported system

- The Center of Rigidity (CR) of a floor level is:

- The geometric center of the floor slab

- The point through which the resultant of the restorative (stiffness) forces acts

- The point where the total weight of the building is concentrated

- Always located at the exact center of the stairwell

- What is the primary purpose of a Shear Wall in a tall building?

- To provide thermal insulation

- To resist lateral forces and control lateral deflection

- To act as a decorative facade

- To support only vertical gravity loads

- A Torsional Effect occurs in a building when:

- The building is made of concrete.

- All columns have the same stiffness

- The wind blows from only one direction

- The Center of Mass (CM) and Center of Rigidity (CR) do not coincide

- Plastic analysis of structures assumes that the material is:

- Perfectly elastic at all times

- Rigid-perfectly plastic or elastic-perfectly plastic

- Brittle and prone to sudden fracture

- Incapable of redistributing moments

- A Plastic Hinge is formed when:

- The first fiber of a section reaches yield stress

- The beam is cut into two pieces

- The entire cross-section reaches its yield stress (Plastic Moment, Mp)

- The concrete starts to crack in the tension zone

- For a rectangular section, the theoretical Shape Factor is approximately:

- 1.0

- 1.15

- 1.5

- 2.0

- In the Simple Strip Method for a rectangular slab, the load is typically shared between:

- Only one direction

- Two orthogonal directions (strips)

- Diagonal directions only

- The corners of the slab only

- When designing a slab with a hole using the Strip Method, the designer must:

- Ignore the hole entirely

- Provide "strong bands" or concentrated reinforcement around the hole edges

- Use only vertical shear reinforcement

- Make the entire slab thicker

- Composite construction refers to members composed of:

- Two different types of concrete

- More than one material (typically steel and concrete) rigidly connected

- Recycled plastic and timber

- Steel members that are bolted together

- What is the primary function of Shear Connectors in composite beams?

- To ensure no relative movement occurs between the steel and concrete

- To prevent the beam from rusting

- To increase the weight of the structure

- To act as spacers for the reinforcement

- Sway Mechanisms in plastic analysis are typically associated with:

- Vertical loads on beams

- Temperature changes in slabs

- Foundations on rock

- Lateral loads on frames

- Why is Shrinkage and Temperature Reinforcement critical in RC Water Tanks?

- To make the tank heavier

- To prevent the concrete from becoming too strong

- To minimize cracking and ensure the tank remains water-tight

- To support the weight of the water roof only

For Fundamentals of Bridge Design Course

- Which type of bridge is suitable when high clearance is required for ship traffic below?

  - Beam Bridge

  - Suspension Bridge

  - Truss Bridge

  - Arch Bridge

Answer: B. Suspension Bridge

- Which type of bridge is generally more cost-effective for short to medium spans?

- Beam Bridge

- Suspension Bridge

- Cable-stayed Bridge

- Arch Bridge

Answer: A. Beam Bridge

- What is a benefit of LRFD's flexibility in design?

- It applies a rigid set of rules without exceptions.

- It allows designers to account for specific conditions and uncertainties in their calculations.

- It requires the use of identical load factors for all materials.

- It avoids considering site-specific load conditions.

Answer: B. It allows designers to account for specific conditions and uncertainties in their calculations.

- What is the purpose of including a dynamic load allowance (impact factor) in bridge design?

- To account for static vehicular loads

- To consider additional forces due to vehicle acceleration, deceleration, and impact

- To increase the dead load of the bridge

- To reduce the live load effect on the bridge

Answer: B. To consider additional forces due to vehicle acceleration, deceleration, and impact

- How should bridge design account for the possibility of overloaded vehicles?

- By using standard load models only

- By ignoring the possibility of overloaded vehicles

- By including a factor of safety to account for potential overloading

- By designing the bridge for the maximum legal load only

Answer: C. By including a factor of safety to account for potential overloading

- What is the use of influence lines in the context of vehicular loads on bridges?

- To determine the maximum moment and shear due to moving loads

- To calculate the dead load of the bridge

- To design the foundation of the bridge

- To assess the impact of wind loads on the bridge

Answer: A. To determine the maximum moment and shear due to moving loads

- What is the role of multiple presence factors in bridge design?

- To account for the simultaneous presence of multiple vehicles on the bridge

- To consider the effect of a single vehicle on the bridge

- To reduce the overall load on the bridge

- To calculate the dead load of the bridge

Answer: A. To account for the simultaneous presence of multiple vehicles on the bridge

- Which load combination is typically used for checking the ultimate limit state in bridge design?

- Dead load only

- Live load only

- Dead load + live load + impact factor + other relevant loads

- Wind load only

Answer: C. Dead load + live load + impact factor + other relevant loads

- What does the girder distribution factor represent in bridge design?

- The distribution of dead loads among girders

- The distribution of live loads among girders

- The ratio of girder weight to total bridge weight

- The distribution of wind loads among girders

Answer: B. The distribution of live loads among girders

- When calculating girder distribution factor, how is load sharing between girders typically accounted for?

- Using influence lines

- Using empirical formulas

- By assuming equal load distribution among all girders

- By neglecting live loads

Answer: B. Using empirical formulas

- What is one advantage of using simplified methods (e.g., AASHTO LRFD empirical formulas) for calculating girder distribution factor?

- They provide the most accurate results

- They are quick and easy to apply in preliminary design

- They do not require consideration of load types

- They ignore the effects of girder spacing

Answer: B. They are quick and easy to apply in preliminary design

- How does the spacing between girders influence the girder distribution factor?

- Larger spacing decreases the distribution factor

- Smaller spacing increases the distribution factor

- Smaller spacing decreases the distribution factor

- Girder spacing does not affect the distribution factor

Answer: C. Smaller spacing decreases the distribution factor

- How does the load distribution typically differ between interior and exterior girders?

- Exterior girders carry more load than interior girders

- Interior girders carry more load than exterior girders

- Load is equally distributed between interior and exterior girders

- Load distribution is not considered for exterior girders

Answer: B. Interior girders carry more load than exterior girders

- Why is the Fatigue Limit State (FLS) important in bridge design?

- To prevent excessive deflections under normal traffic conditions

- To ensure that the bridge can withstand repeated loading cycles without failure

- To account for temperature variations affecting the bridge

- To ensure aesthetic standards are maintained over time

Answer: B. To ensure that the bridge can withstand repeated loading cycles without failure

- How are multiple presence factors used in estimating live loads on bridges?

- By reducing the live load effect based on the number of lanes loaded

- By increasing the live load effect based on the number of lanes loaded

- By applying the same live load to all lanes regardless of vehicle presence

- By ignoring the number of lanes loaded

Answer: A. By reducing the live load effect based on the number of lanes loaded

- What are live loads in the context of bridge design?

- Permanent loads that do not change over time

- Loads due to vehicles, pedestrians, and other moving objects

- Loads caused by the weight of the bridge structure itself

- Loads due to environmental factors such as wind and temperature

Answer: B. Loads due to vehicles, pedestrians, and other moving objects

- How is shear force typically resisted in slab bridges?

- By the vertical stirrups in the slab

- By the horizontal reinforcement in the slab

- By the thickness of the slab and the reinforcement within it

- By the external cables used for prestressing

Answer: C. By the thickness of the slab and the reinforcement within it

- In the analysis of girder bridges, why is both longitudinal and transverse analysis important?

- To account for the effects of wind and earthquakes

- To ensure the bridge can handle traffic in both directions

- To capture the behavior of girders and deck separately, ensuring accurate load distribution and stress calculation

- To reduce the overall cost of the bridge construction

Answer: C. To capture the behavior of girders and deck separately, ensuring accurate load distribution and stress calculation

- Which design philosophy is adopted by AASHTO for bridge design?

- Allowable Stress Design (ASD)

- Load and Resistance Factor Design (LRFD)

- Ultimate Strength Design (USD)

- Working Stress Design (WSD)

Answer: B. Load and Resistance Factor Design (LRFD)

- How are limit states addressed in AASHTO LRFD?

- By ignoring them

- By considering only one limit state

- By considering multiple limit states such as strength, serviceability, and fatigue

- By considering only strength limit states

Answer: C. By considering multiple limit states such as strength, serviceability, and fatigue

- In what situations might the multiple presence factor be used for a single-lane bridge?

- During the design of a pedestrian bridge

- During the design of a bridge with low traffic volume

- During the design of a bridge with high traffic volume

- During the design of a bridge with a single lane

Answer: C. During the design of a bridge with high traffic volume

- How do properly designed distribution bars contribute to the longevity of bridge decks?

- By increasing the rate of corrosion

- By reducing the load-carrying capacity of the bridge deck

- By minimizing the development of cracks and deterioration

- By increasing the weight of the bridge

Answer: C. By minimizing the development of cracks and deterioration

- What does the hydrological assessment of a bridge site involve?

- Evaluating the water quality of nearby rivers

- Measuring the air humidity of the site

- Assessing the water flow, depth, and velocity of nearby water bodies

- Studying the aquatic life in the vicinity

Answer: C. Assessing the water flow, depth, and velocity of nearby water bodies

- In the context of bridge design, what is a "live load surcharge"?

- An additional static load applied to the bridge

- A reduced load to account for future weight increases

- An extra load added to account for unexpected live load increases

- A load factor applied during wind loading analysis

Answer: C. An extra load added to account for unexpected live load increases

- In the context of low-level water crossings, what does the term "overtopping" refer to?

- The maximum allowable height of the crossing

- The water flow exceeding the capacity of the structure

- The process of placing the final layer of pavement

- The design of the handrails for pedestrian safety

Answer: B. The water flow exceeding the capacity of the structure

- What is the primary risk associated with low-level water crossings during flood events?

- Structural failure due to high wind speeds

- Increased wildlife collisions

- Submersion and inaccessibility

- Reduced water quality

Answer: C. Submersion and inaccessibility

- Which design feature helps prevent erosion at the entry and exit points of a low-level water crossing?

- Vertical walls

- Gabion baskets

- High-friction surface coating

- Curved approach ramps

Answer: B. Gabion baskets

- What type of movement is a reinforced elastomeric bearing designed to accommodate?

- Vertical movement only

- Horizontal movement only

- Both horizontal and vertical movements

- Neither horizontal nor vertical movements

Answer: C. Both horizontal and vertical movements

- What is a low-level water crossing primarily designed to accommodate?

- High-speed vehicular traffic

- Heavy industrial equipment

- Seasonal or intermittent water flows

- Permanent deep-water channels

Answer: C. Seasonal or intermittent water flows

- How is the suitability of a bridge site assessed?

- By selecting the most challenging terrain

- By evaluating geological, hydrological, and environmental factors

- By prioritizing aesthetic appeal over functionality

- By disregarding community input

Answer: B. By evaluating geological, hydrological, and environmental factors

For Steel & Timber Structure Design  course:

- Why steel structure is more preferable than concrete?

- Steel structure has low strength

- Its non-Uniformity

- It  has no Elasticity behavior

- Its ductility behavior

- All of the above

- Why are not   tension members design for ultimate loads?

- Because steel tension members can only sustain loads up to the yield load

- Because the elongation of the members at ultimate load is high ( nearly 10-15% of the
original length)

- Because the elongation of the members at ultimate load is high

- All of the above

- Which of the following is true about critical load for compression member?

- If the critical load is equal to section capacity of the compression member, then the member is stable

- If the critical load is greater  than  section capacity of the compression member, then the member is unstable

- If the critical load is less than section capacity of the compression member, then the member is stable

- All of the above

- If the member subjected to tension load fails along its transversal direction, which one of the following is true?

- The member has less shear capacity

- The member has less Tension capacity

- The member has less bearing capacity

- The member has less shear & bearing capacity

- Why holes are considered in tension member but not in compression member?

- Because it decreases its cross section

- Because it increases its cross section

- It results non uniform stress distribution

- The stress is uniform  throughout the section  whether there is hole or not

- Which of the following is true on flexural buckling and lateral torsional buckling resistance?

- Flexural buckling resistance is the deflection due to lateral loads on beams

- Lateral torsional buckling resistance   is deflection  due to lateral loads on columns

- Flexural buckling Resistance is the strength of the compression member  due to axial loads

- Lateral torsional buckling resistance  is deflection  due to lateral on columns

- None