| ### Course Name: Reinforced Concrete Design I | ### Course Code: CEng 3092 |
| --- | --- |

- The characteristic strength of concrete mass is that value of strength below which not more than “X” percent of test result are expected to fall. What is the value of “X”?

- 5

- 15

- 85

- 95

- The 28 days characteristic cylindrical compressive strength of concrete of class C25/30 is:

- 11.33N/mm2

- 14.17N/mm2

- 20N/mm2

- 25N/mm2

- Among the following which can be considered as possible stress-strain curve for concrete as per ES EN 2015:

- A perfect straight line up to failure

- Straight line up to 0.002 strain value and then parabolic up to failure

- Parabolic up to 0.002 strain value and then straight line up to failure

- Hyperbolic up to 0.002 strain value and then straight line up to failure

- Why is it important to limit the depth of neutral axis in reinforced concrete beams?

- To ensure that the beam has sufficient shear capacity

- To ensure that the beam has sufficient flexural stiffness

- To ensure that the beam has sufficient ductility and avoids brittle failure

- To ensure that the beam has sufficient bond strength between concrete and steel

- A simply supported beam is required to carry a permanent load of 21kN/m including self-weight over an effective span of 6.0m. this beam shall be designed for a factored bending moment in limit state method of:

- 31.5kN-m

- 141.75kN-m

- 21.3kN-m

- 127.6kN-m

- In a RCC beam the shear force is carried by:

- Concrete alone

- Vertical stirrups

- Inclined stirrups

- All of the above

- What is the range of variable inclination of concrete struts in the variable strut inclination method for shear design according to Eurocode 1992?

- 1 ≤ sin(θ) ≤ 2.5

- 1 ≤ cot(θ) ≤ 2.5

- 1 ≤ tan(θ) ≤ 2.5

- 1 ≤ cos(θ) ≤ 2.5

- In the case of two way slab, the deflection of the slab is:

- The characteristic strength – 1.64 times standard deviation

- The characteristic strength + 1.64 times standard deviation

- The ultimate strength – 1.64 times standard deviation

- The ultimate strength + 1.64 times standard deviation

- Which limit state deals with deflection, cracking, vibration, durability, fatigue, fire resistance and lightning?

- Serviceability limit state

- Ultimate strength design method

- Ultimate limit state

- Working stress design method

- Under what circumstances does doubly reinforced beams are adopted?

- Primaril When the size of the beam is restricted by the Architect

- When the section of the beam is subjected to inversion stress

- When the beam is subjected to high shear forces

- When the beam is subjected to high axial loads

- Which of the following is not an assumption in the limit state theory for designing of RCC

- Strains are proportional to distance from the neutral axis

- The tensile strength of concrete is neglected as the section is assumed to be cracked up to the neutral axis

- Ultimate limit state of bending failure is assumed to have been reached when the strain in the concrete at the extreme bending compression fiber reaches 0.005

- The stress distribution across compression face will correspond to the stress-strain

- What is the variable strut inclination method for shear design according to Eurocode 1992?

- A method that provides the conceptual basis for the operational design procedures of EN 1992-1-1 (EC2) for shear design of concrete members

- A method that provides the conceptual basis for the operational design procedures of EN 1992-1-1 (EC2) for compression design of concrete members

- A method that provides the conceptual basis for the operational design procedures of EN 1992-1-1 (EC2) for tension design of concrete members

- A method that provides the conceptual basis for the operational design procedures of EN 1992-1-1 (EC2) for bending design of concrete members

- What is the maximum compressive stress that a diagonal section of a concrete member can withstand before it fails in compression?

- Diagonal compression resistance capacity of concrete

- Diagonal tension resistance capacity of concrete

- Both A and B

- None of the above

- The limiting shear stress of C20/25 concrete for 45o diagonal strut inclination as per ES EN 2015 is

- 11.33N/mm2

- 1.67N/mm2

- 5.66N/mm2

- 2.82N/mm2

- In a reinforced concrete member the best way to ensure adequate bond is:

- To provide the minimum number of larger diameter bars

- To provide larger number of smaller diameter bars

- To increase the cover of reinforcement

- To provide the additional stirrups

| ### Course Name: Reinforced Concrete Design II | ### Course Code: CEng 4091 |
| --- | --- |

- Understanding the development of plastic hinges requires knowledge of:

- The member's material strength

- The location of applied forces

- The curvature of the member at specific points

- All of the above

- Studying the curvature of members is important for:

- Determining their weight

- Evaluating their ability to deform without breaking (ductility)

- Predicting their color

- Measuring their surface area

- Which of the following is false when a designer uses a larger strut inclination angle while designing RC beams for torsion:

- The spacing of link bars increase

- The longitudinal reinforcement required will increase

- The notional concrete strut compression capacity will decrease

- None of the above

- The torsional longitudinal reinforcement in reinforced concrete beams should be placed at:

- Tension zone of the beam cross-section

- Compression zone of the beam cross-section

- At least one bar at each corner, the others being distributed uniformly around the inner periphery of the links

- Core of the beam

- According to the Eurocode standards (ES EN 1992-1-1), what are the limitations on negative moment redistribution based on reinforcement class?

- Class A: 10%, Class B: 15%, Class C: 20%

- Class A: 20%, Class B: 30%, Class C: 30%

- Class A: 80%, Class B: 70%, Class C: 70%

- Class A: Not allowed, Class B: 20%, Class C: 30%

- Torsion resisting capacity of a given RC section:

- Decrease with decrease in stirrup spacing

- Decrease with increase in longitudinal bars

- Increase with the increase in stirrup and longitudinal steel

- Does not depend upon stirrup and longitudinal steels

- The figure below (Figure 1) represents a:

Figure 1 question 22

- Dog-legged stair

- Quarter turn stair

- Straight stairs

- Open well stairs

- In the design of stairs spanning longitudinally shown in Figure 2 depth of section for resisting

Figure 2 question 23

- d1

- d2

- d3

- None of the above

- Figure 3 shows main reinforcement details of a stair. Which statement is correct regarding the bar bending detailing near sections when there is changing direction

Figure 3 question 24

- Reinforcement detailing at both “A” and “B” location are not correct

- Reinforcement detailing at both “A” and “B” location are correct

- Reinforcement detailing at “A” location is correct while detailing at “B” location is not correct

- Reinforcement detailing at “A” location is not correct while detailing at “B” location is correct

Answers

| 1 | A |
| --- | --- |
| 2 | C |
| 3 | C |
| 4 | C |
| 5 | D |
| 6 | D |
| 7 | B |
| 8 | B |
| 9 | A |
| 10 | A |
| 11 | C |
| 12 | A |
| 13 | A |
| 14 | D |
| 15 | B |

| 16 | D |
| --- | --- |
| 17 | B |
| 18 | D |
| 19 | C |
| 20 | B |
| 21 | C |
| 22 | A |
| 23 | A |
| 24 | C |