Bahir Dar University

Bahir Dar Institute of Technology

Faculty of Electrical and Computer Engineering

Collected Model Exit Exam Questions

Compiled by:

Tenaw Ayew

Course Name: Introduction to Communication Systems

- Assuming μ as a modulation index of an amplitude modulation, AC and Am amplitudes of carrier and message signals, respectively, when a distortion occurs during the modulation process.

- AC < Am

- AC = Am

- μ<1

- μ=1

- Which measures could improve the signal quality at the demodulator's output?

- Employ a low-pass filter with a higher cut-off frequency at the demodulator's output.

- Increase the carrier wave's amplitude.

- Employ a low-pass filter with a higher edge-steepness at the demodulator's output.

- Decrease the LF signal's amplitude.

- Which one of the following is incorrect in amplitude modulation (AM)

- The source of the Carrier wave is physically independent of the source responsible for generating the message signal.

- AM is a process in which the amplitude of the carrier signal is varied according to the amplitude of the message signal.

- The carrier and message amplitudes are measured in volts.

- Phase reversal will not occur when the modulation index exceeds one.

- Among the communication systems performance matrices, which one is the measure of how well the message signal replicates the original signal at the output?

- Selectivity

- SNR

- Fidelity

- Jitter

- A set of rules that governs data communication is?

- Fidelity

- Jitter

- Sensitivity

- Protocol

- In Multiplexing Techniques, which one of the following gives the same time slot for each device to transmit the data over the link, whether the device has any data to transmit or not

- FDM

- WDM

- STDM

- ASTDM

- As per the Carson’s rule in transmission of FM signal, what would be the bandwidth of a signal with fm = 3 KHz and modulation index 2?

- 6 KHz

- 18 KHz

- 8 KHz

- 32 KHz

- Amplitude modulation is: -

- Change in amplitude of the carrier signal according to the modulating signal

- Change in frequency of the carrier signal according to modulating signal

- Change in amplitude of the modulating signal according to the carrier signal

- Change in amplitude of the carrier signal according to the modulating signal frequency

- Requirements of synchronous detection of AM signals are:

- Local generation of carrier signal

- The frequency of the locally generated carrier must be identical to that of transmitted carrier.

- The phase of the locally generated carrier must be synchronized to that of transmitted carrier.

- All of the above

- When aliasing effect will happen

  - The amplitude of carrier signal is greater than the amplitude of modulating signal

  - The modulation index is less zero

  - The amplitude of carrier signal is less than the amplitude of modulating signal

  - Not occurred at all

- If modulation index of an AM wave is increased from 1.5 to 2, then the transmitted power _

  - Remains the same

  - Increases by 20%

  - Increases by 41%

  - Increases by 50%

- What is the benefit of increasing the sample frequency beyond twice the maximum frequency of an information signal?

  - Improved signal quality.

  - Increased processing time.

  - Increased memory storage capacity.

  - No advantage at all

- During pulse amplitude modulation ...

- Frequencies are modulated.

- Amplitudes are modulated.

- Phases are modulated.

- Both Phase and Amplitudes are modulated

- During pulse code modulation ...

- Digitization is not possible.

- Filtration, e.g., through a band-pass, is unnecessary.

- Signal amplitudes are divided into stages.

- The carrier signal does not need to be modulated

- Which one of the following is a digital multiplexing?

- Frequency division multiplexing

- Wavelength division multiplexing

- Synchronous time division multiplexing

- A and B

- One of the following is not an advantage of digital communication over analog communication

  - Storing capability is increased

  - Cheaper in cost

  - Hardware implementation is flexible

  - Synchronization is required

- Which one of the following is not an advantage of analog communication over digital communication?

- The data rate is low

- Less transmission bandwidth is required

- Synchronization is not needed

- Provide authentication

- Which of the following is not true about Modulation-demodution?

- Square law modulator consists of adder ,non-linear device and band pass filter

- Modulation allows multiplexing

- The advantage of angle modulation over amplitude modulation is noise reduction

- The process of recovering information signal from received carrier is demultiplexing

- If message signal m(t)=10cos(2π103t) AM modulates carrier signal  c(t)=20cos(2π105t), then the modulation index is

- 2

- 1

- 0.5

- 0.25

- Given the three modulation schemes AM, DSB-SC and SSB, which of the following is correct?

  - Complexity increases from AM→DSB-SC→SSB

  - Transmission power increases from AM→DSB-SC→SSB

  - Noise immunity  increases from AM→DSB-SC→SSB

  - Bandwidth decreases from AM→DSB-SC→SSB

- A 100MHz carrier is frequency modulated by 5 KHz wave. For a frequency deviation of 100 KHz, calculate the carrier swing of the FM signal.

- 2000 KHz

- 100 KHz

- 105 KHz

- 200 KHz

- The amount of frequency deviation in FM signal depends on

- Amplitude of the modulating signal

- Carrier frequency

- Modulating frequency

- Transmitter amplifier

- In an Armstrong FM modulator, the figure below represents

- Frequency converter

- Narrowband FM

- Frequency multiplier

- Product modulator

- Which modulation scheme has superior noise performance among the following?

- DSB-SC

- AM

- SSB

- FM

- Which of the following is not a digital pulse modulation technique?

- PCM

- DM

- PAM

- DPCM

- Which of the following is not a binary signaling format?

  - Unipolar NRZ

  - Bipolar NRZ

  - Split phase (Manchester)

  - Frequency division multiplexing

- Which of the following is not purpose of modulation?

- Reducing the effect of noise

- Amplification of information signal

- Multiplexing information signals

- Practicability of antenna

- The two main resources of a communication system are

  - Amplifier and filter

  - Power and bandwidth

  - Antenna and speaker

  - Modulator and demodulator

- Which one is wrong about the AM signal  st=80cos106πt+40cos104πtcos⁡(106πt)?

- Its bandwidth is 10 KHz

- Its maximum amplitude is 120

- The average information signal power is 200

- The upper side frequency is 1.01MHz

- In an Armstrong FM modulator, the figure below represents

BPF

SWBM(t)

Cos2πfcot

O(t)

- Frequency converter

- Narrowband FM

- Frequency multiplier

- Product modulator

- Among the following which one has the least transmission power to achieve the same SNR at the receiver output?

- DSB-SC

- AM

- SSB

- FM

- A compact disc (CD) records audio signals digitally by using PCM. Assume the audio signal bandwidth to be 25KHz.If the Nyquist samples are quantized in to L=512 levels and then binary coded, the number of binary digits /secs required to encode the audio signal is

- 450Kb/sec

- 225Kb/sec

- 12.8Mb/sec

- 600Kb/sec

- Which one is different from the other?

  - PAM

  - PWM

  - PPM

  - PSK

| ### Course Name: Network Analysis and Synthesis | ### Course Code: EEng 3092 |
| --- | --- |

- Which one is correct about a reactance function

  - All poles and zeros lie on the negative real axis

  - All poles and zeros are simple and /or Multiple

  - All poles and zeros are simple and lie on the jω axis only

  - All poles and zeros lie on the negative real axis

- What type of filter is shown in the figure below, and its cutoff frequency?

  - High-pass, 21 Hz

  - Low-pass, 21 Hz

  - High-pass, 2.65 kHz

  - Low-pass, 2.65 kHz

- At a certain frequency, the output voltage of a filter is 6 V, and the input is 12 V. The voltage ratio in decibels is

  - 6.02 dB

  - –6.02 dB

  - 12.04 dB

  - –12.04 dB

- Which of the following is correct for a driving point function?

  - The real parts of all poles and zeros must be negative

  - The real parts of all poles and zeros must be negative and/or zero

  - The real parts of all poles and zeros must be positive and/or zero

  - The real parts of all poles must be positive

- The rate change of amplitude with frequency (or attenuation rate)is called

  - Envelope delay

  - Roll off

  - Ripple

  - none

- The first and the last critical frequencies of a driving point impedance function of a passive network having two kinds of elements are a pole and a zero, respectively. What type of network is it?

  - RC network

  - RL network

  - RC as well as RL

  - LC network

- Two important conditions in network synthesis are

  - Linearity and causality

  - Linearity and stability

  - Causality and stability

  - Nonlinearity and stability

- Two-port impedance network  A    =     and  B=   interconnected in series, what will be  the resulting two-port network parameters

- Two two-port networks are connected in cascade. To determine the response, it is more convenient to use

  - h parameters

  - Open circuit impedance parameters

  - Short circuit admittance parameters

  - ABCD parameters

- Io A transfer function of network analysis can be represented as:

- Attenuation

- Distortion

- Gain

- Both A and C

- Hybrid parameter of h12 in a two – port network used as?

- A voltage source at the output port

- A current source at the output port

- A voltage source at the input port

- A current source at the input port

- The driving-point impedance Z(s) of a network has pole-zero locations as shown below. If Z(0) = 3, then Z (s) is

  - .

- Which one is true for a network realization technique matched

  - Brune's realization of a non-ideal transformer.

  - Cauer realization: ladder realization.

  - Bott Duffin's realization of a non-ideal transformer

  - None

- Network analysis and synthesis are usually done in

  - Time domain

  - Frequency domain

  - Laplace domain

  - Z-domain

- Determine the order of the filter used when the gain increases at the rate of 60dB/decade in the stop band.

  - Third-order high-pass filter

  - First-order low-pass filter

  - Second-order low-pass filter

  - None of the mentioned

- Which one of the following is not true about networks and circuits?

- Every circuit is a network, but all networks are not circuits.

- Every network is a circuit, but all circuits are not networks.

- Active network elements are components which are able to supply energy to the network.

- Passive network elements are components which take energy from the sources.

- The number of links in a connected network graph with 4 nodes and 6 branches is:

- 2

- 4

- 6

- 3

- Which one of the following is true about all-pass filter?

- An all-pass filter has a constant gain across the entire frequency range.

- It’s phase response changes nonlinearly with frequency.

- It filters or passes frequencies within a certain range.

- It filters or passes lower frequencies.

- One of the following is not a correct dual-network element relationship:

- Resistance – Conductance

- Inductance – Capacitance

- Link - Twig

- Resistance – Reactance

- Which one of the following is correct about properties of a driving point function?

- The real parts of all poles must be negative

- The real parts of all poles and zeros must be negative

- The real parts of all poles and zeros must be negative or zero

- The real parts of all zeros must be negative

- One of the following is not true about a causal network.

- impulse response of the network must be zero for t < 0.

- impulse response of the network must be different from zero for t < 0.

- impulse response of the network must be zero for t > 0.

- voltage doesn’t appear between any terminals in the network before a current/voltage is applied.

- A & D

- B & C

- For a reciprocal two port network, one of the conditions hold true:

- Z11 = Z22

- Z12 = Z21

- Z11 = Z12

- Y11 = Y22

- One of the following is the merit of active RC filters over passive filters.

- wider bandwidth

- reduced size and weight

- simpler design than for passive filters

- lesser environmental sensitivity

- A & D

- B & C

- Which one of the following is a passive network element?

- Resistor

- Current source

- Battery

- Ideal voltage source

- If the ratio of voltage at one port to the current at the other port is the same to the ratio of current at one port to the voltage at the other port, then the network is said to be

- Symmetric

- Reciprocal

- Dual

- None of the above

- The current through the 6-Ohm resistor in the following circuit is:

- 2A

- 0.8125

- 1.25A

- 1.5A

- The transform admittance of an inductor is:

- L

- 1/L

- sL

- 1/Sl

- In the circuit shown below, if current is considered as the response signal of the circuit, then the transfer function of the given circuit is:

- Hs= 3s6s2+30s+1

- Hs= 6s2+30s+13s

- Hs= 6s2+20s+12s

- Hs= 2s6s2+20s+1

- The first network element of the dp impedance function Z(s) given below applying first Cauer form is:

- C1 = s

- L1 = s

- C1 = 3/2s

- L1 = 3/2s

- A network function F(s) is positive real if

- F(s) is real for real s.

- The poles and zeros of F(s) all lie in the right half of the s-plane.

- The poles and zeros of F(s) are not simple and lie on the imaginary axis of the s-plane.

- The real part of F(s) less than or equal to zero for the real part of s greater than or equal to zero.

- For the circuit given below, the value of the Z11 and Z21 are _________________.

- Z11 = R2, Z21 = R1 + R2

- Z11 = R2 + R3, Z21 = R2

- Z11 = R1 + R2, Z21 = R2

- Z11 = R2 + R3, Z21 = R3

- For the active low pass filter shown below, its cut-off frequency (fc) is:

- 1 MHz

- 0.3 MHz

- 0.16MHz

- 0.02MHz

- Steps to Modern filter design includes:

- Selection of filter specification

- Approximation

- Realization

- All of the above

- None of the above

- Types of filter approximation used in frequency domain are:

- Butterworth

- Chebyshev

- Elliptic

- A and B

- All of the above

- Which of the following expression are true for the system function H(s).

- Impedance: Hs=VosIins

- Admittance: Hs=IosVins

- Voltage Ratio:  Hs=VosVins

- All of the above

- For the initially inert circuit given below, the input signal Vgt=12cost u(t). The free response of Vot of the circuit is given by:

- 0.4cost ut

- 0.4cost ut+0.2sint u(t)

- -0.4e-2t u(t)

- -0.4e2t u(t)

- Which one of the following techniques are used to determine that a given system function Hs can be realized into a physical network?

- Positive Realness Test

- Hurwitz Polynomial Test

- Even and Odd Function Test

- A and B are the answers

- None

- In one port network synthesis, the expression of the network function corresponding to the removal of a pole at s=0 is:

- FS=DS+F2(S)

- FS=DS+F2(S)

- FS=DSS2+ω12+F2(S)

- S=D+F2(S)

- Which one of the following is not the property of LC immittance functions?

- Poles and zeros lie on jω axis and they interlace

- Degrees of the numerator and denominator must differ by unity

- Poles and zeros lie on non-positive real axis and they interlace

- None

Course Name: Signals and Systems

- The continuous time system described by the equation y(t)=3x(t-2) +4 comes under which category

- causal, linear, and time-invariant

- causal, non-linear, and time-invariant

- non causal, non-linear and time-invariant

- non-causal, linear, and time-variant

- Convolution is used to find

- Multiplication of signals

- Response of the system

- Amount of similarity between signals

- Difference between signals

- The convolution between impulse  δ(n) and unit step u(n) sequences is given by

- δ(n)

- u(n)

- r(n)

- sgn(n)

- A transform method mainly used to simplify the analysis of equations involving differential and integral operations is

- Fourier Transform

- Hilbert Transform

- Laplace Transform

- Z-Transform

- What is the Nyquist sampling rate for the signal s(t)=2 cos(80πt)+sin(400πt)-cos(200πt) ?

- 80 Hz

- 200 Hz

- 800 Hz

- 400 Hz

- Which of the following is not correct?

- u(n)*u(n)=n+1

- u(n)*(δ(n)-δ(n-1))=δ(n)

- Ƒ(δ(n)-δ(n-2))=1-e-j2ω

- ʆ(u(t))=1s2

- The region of convergence of the z-transform of the sequence

xn=13nun--12nu-n-1

  - z>13

  - z<12

  - 13<z<12

  - None

- The property of the Fourier Transform, which states that the compression in the time

domain is equivalent to expansion in the frequency domain, is

- Duality

- Scaling

- Time shifting

- Frequency shifting

- Which property of the Fourier transform gives rise to an additional phase shift of -2πftd for the generated time delay in the communication system without affecting the amplitude spectrum?

- Time Scaling

- Linearity

- Time Shifting

- Duality

- The function that has its Fourier transform, Laplace transform, and Z transform unity is

  - Gaussian.

  - impulse.

  - sinc.

  - pulse

- Consider the sequence x[n]=2nu[n], which one of the following statements is true?

- x[n] is absolutely summable

- The Fourier transform of x[n] converges

- The Fourier transform of r-nx[n] coverges for r>2

- None

- Let xt=cosωx(t+τx+θx), the frequency (hertz) and period of x(t) for: ωx=π3, τx=0,  θx=2π

- f=13Hz, T=3 sec

- f=16Hz, T=6 sec

- f=19Hz, T=9 sec

- f=1Hz, T=1 sec

- Given x[4-n] the shifted version of the signalx[n], which one of the plots represent x[n].

- None

- What is the sum of: n=0523an

- 22(1-25)

- 2N

- 21-3a61-3a

- 21-3a51-3a

- Given the following two systems:
i. yn=xn+x[n-1]
ii. yn=xn+xn-1+x[0]
Which one of the following statements are true?

- i and ii are linear

- i is linear and Time invariant but ii is time varying

- i and ii are time invariant

- none of the above are true

- A&B

- The first order ordinary differential equation yn-ayn-1=x[n], 0<a<1, describes a particular discrete time system initially at rest. Which of the following statements are true?

- Impulse response of the system is hn=anu[n]

- The system is not memoryless

- The system is causal

- The system is stable

- All

- Output of a causal LTI system is related to the input x(t) by the differential equation:

dytdt+2yt=x(t)

The frequency response H(ω) is then:

- ω4+ω2

- 12+jω

- 24+ω2

- None of the above

- Consider the signal xt=3e2tut+4e3tut, for which of the following values of σ does the Fourier Transform of xte-σt converge?

- σ=1

- σ=2.5

- σ=3.5

- None

- For the Z-Transform of a particular signal given by xz=11+0.5z-1, z>0.5, The inverse Z-Transform is:

- xn=-0.5nu[n]

- xn=-0.5nu[-n]

- xn=0.5nu[n]

- All of the above

- For the complex number z=12ejπ4, the angle of z  (∡z)is:

- 24

- 14

- π4

- 1

- With xn=cos(Ωxn+Px+θx) and yn=cos(Ωyn+Py+θy), for which of the following combinations xn and yn are identically equal for all  n in the order of 
Ωx, Px, θx, Ωy,Py, θy

  - (π3,  0, 2π, 8π/3, 0,	0)

  - (3π/4,  2, π/4, 3π/4, 1,	-π)

  - (3/4, 	1,	1/4, 	3/4, 	0,	1)

  - B and C

  - None

- For xn=δn+1u[n], the identical signal is:

- dundn

- k=-∞nδ[k]

- k=0∞12kδ[n-k]

- None

- Consider an LTI system whose response to the signal x1(t) is y1(t) as shown below.

The response of the system to the signal x2(t) Given below is then:

- The Fourier Series coefficient of the signal: xt=1+cos(2πt) is:

- C-1=C1=ejπ62j

- C-1=C1=12 and C0=1

- C-1=π, and C1=-π

- None

- For an LTI system yt=xt*ht. Which of the following expressions are true?

- Yω=X(ω)H(ω)

- YS=X(S)H(S)

- HS=YSXS

- None of the above is true

- All of the above are true

- Consider the sequence xn=2nu[n]. The Fourier Transform of xn:

- Converges as N→∞

- Diverges as N→∞

- None

- Partially Converges as N→∞

- Suppose X(t)=sin10πtπt. The sampling interval Ts so the x(t) can be uniquely represented by the Discrete Time sequence x[n]=x(nTs) is:

- 0.1 sec

- 10 sec

- 0.2 sec

- None of the above

Course Name: Object Oriented Programming

- Which feature of OOP indicates code reusability?

- Abstraction

- Polymorphism

- Encapsulation

- Inheritance

- A single program of OOPS contains _______ classes?

- Only 1

- Only 999

- Only 100

- Any number

- Which of the following definition is incorrect for polymorphism?

- Polymorphism helps in redefining the same functionality

- Polymorphism concept is the feature of object-oriented programming (OOP)

- It always increases the overhead of function definition

- Ease in the readability of the program

- When is the object created with a new keyword?

- At run time

- At compile time

- Depends on the code

- None

- The concept of encapsulation helps in writing which type of classes in the Java programming language?

- Abstract classes

- Wrapper classes

- Mutable classes

- Immutable classes

- Encapsulation is_____?

- technique of combining more than one member functions into a single unit.

- mechanism of combining more than one data member into a single unit.

- mechanism of combining more than one data members and member functions that implement on those data members into a single unit

- technique of combining more than one data members and member functions into a single unit, which can manipulate any data.

- Which access specifier is usually used for data members of a class?

- Protected

- Private

- Public

- Default

- Objects are the variables of the type ?

- String

- Boolean

- Class

- All data types can be included

- Why classes are known as abstract data types (ADT)?

- Because classes are user-defined data types

- Because it supports the theory of hierarchical classification

- Because it allows dynamic binding

- Because it uses the concept of data abstraction

- Which member of the superclass is never accessible to the subclass?

- Public member

- Protected member

- Private member

- All of the mentioned

- Which class in Java is used to take input from the user?

- Scanner

- Input

- Applier

- None of these

- How many primitive data types are in Java?

- 6

- 7

- 8

- 5

- Which exception is thrown when Java is out of memory?

A.  Memory Error

B. out of Memory Error

C.  Memory out of Bounds Exception

D. Memory Full Exception

- Which component is used to compile, debug, and execute the Java programs?

A.  JRE

B. JIT

C. JDK

D. JVM

- A single program of OOPS contains _______ classes?

- Only 1

- Only 999

- Only 100

- Any number

- When is the object created with a new keyword?

- At run time

- At compile time

- Depends on the code

- None

- Which access specifier is usually used for data members of a class?

- Protected

- Private

- Public

- Default

- Objects are the variables of the type?

- String

- Boolean

- Class

- All data types can be included

Course Name: Digital Logic Design

- Which one of the following is not an analog system?

- Calculators

- Audio amplifiers

- magnetic tape-recording equipment

- None

- What is the output of an XOR gate if both inputs are the same?

- 0

- 1

- Depends on other factors

- Can't be determined

- Which rule of Boolean algebra is incorrect?

- A + AB = A

- A + A’B = A + B

- (A+B) (A+C) = A + C

- A + A = A

- ----------------- is a digital circuit that produces the arithmetic sum of two binary numbers in parallel.

- Binary Adder-Subractor

- Binary Parallel Adder

- Full Adder

- All

- Which combinational circuit selects binary information from one of many input lines and directs it to a single output line?

  - Decoder

  - Multiplexer

  - DE-multiplexer

  - Encoder

- Which of the following is not true about the Gated D-Latch?

  - When the D input is HIGH and the EN input is HIGH, the latch will set.

  - When the D input is LOW and EN is HIGH, the latch will reset

  - The output Q follows the input D when EN is HIGH.

  - None

- In a sequential circuit, what is the clock signal used for?

  - To generate output directly

  - To synchronize the operations of flip-flops

  - To reset the circuit

  - To select input data

- Which of the following types of counters can count both upwards and downwards?

  - 2-bit up counter

  - Ring counter

  - Johnson counter

  - B and C

- How many data inputs and outputs does an 8-bit piso shift register have?

  - Eight parallel data inputs and two complementary serial outputs

  - Eight parallel data inputs and eight complementary serial outputs

  - Four parallel data inputs and four complementary serial outputs

  - Eight parallel data inputs and one serial output

- Which of the following logic gates is a universal logic gate?

  - NOR gate

  - AND gate

  - NOT gate

  - X-OR gate

- A symbol used to represent a logical quantity is called?

  - Complement

  - Boolean algebra

  - Variable

  - Gate

- We call it the mathematics of digital systems:

  - Boolean algebra

  - Literal

  - Logic gates

  - A complement

- A principle that states for every algebraic expression deducible from the postulates of Boolean algebra remains valid if the operators and identity elements are interchanged.

  - De’Morgans Principle

  - Duality principle

  - Associative principle

  - K-map

- A circuit that contains logic gates as well as memory cells is known as?

  - Combinational circuit

  - Latches

  - A counter

  - Sequential circuit

- A combinational circuit that converts binary information from n-input lines to a maximum of 2n unique output lines is called?

  - Decoder

  - Encoder

  - Demultiplexer

  - Multiplexer

- An extra bit that is attached to a code group that is being transferred from one location to another is?

  - Parity checker bit

  - Parity generator bit

  - Parity bit

  - Even parity

- A group of flip-flops capable of storing one bit of information is called?

  - Counter

  - Register

  - Flip-flops

  - Latches

- A register capable of shifting its binary information in one or both directions is called?

  - Parallel register

  - Shift register

  - Cascaded register

  - Dual transfer register

- Which of the following is the characteristics  Digital representation

  - Digital systems are high prone to error.

  - They can vary over a continuous range of values.

  - Real world in nature is Digital.

  - The representation of data in binary form

- What is the 2’s complement representation of -43 using 8 bit representation?

  - 00011100

  - 00011110

  - 11001110

  - 11100011

- Which Gate is represented using an expression F= AB + A’B’

  - Exclusive-NOR Gate

  - Exclusive-OR Gate

  - NAND Gate

  - NOR Gate

- Which of the following is the characteristics of NAND Gate

  - Inverted AND Gate

  - Universal Gate

  - Inverted OR Gate

  - A&B

- How many multiplexers required to build a 32 x 1 multiplexer?

- Three 16 x 1 mux

- Two 16 x 1 mux and two OR gate

- Four 8 x 1 mux and one 4 x 1 mux

- Two 8 x 1 mux

- What determines the output from the sequential logic circuit in Digital Circuit?

  - Input signals from the past condition

  - The present values of its input only

  - The sequence of past inputs with a Clock signal

  - The present value of its input signals and the sequence of past inputs

- What is meant by parallel-loading the register?

  - Momentarily disabling the synchronous SET and RESET inputs

  - Loading data in all four flip-flops at the same time

  - Loading data in two of the flip-flops

  - Shifting the data in all flip-flops simultaneously

Course Name: Computer Architecture and Organization

- Which of the following is not a computer organizational attribute of a computer system?

  - Control signals

  - Instruction set

  - Device implementation technology

  - Low-level design aspect of the CPU

- What does the term “Stored Program Computer” refer to?

  - The ability to store instructions along with data in memory, reading them from memory

  - The ability to hardwire programs by interconnecting various hardware components in the desired configuration for particular computation

  - The ability to store programs in a relational database and retrieving them one by one

  - None of the above

- Which register is used to store the fetched Instruction from memory at the end of the Fetch cycle of an instruction cycle?

  - Program Counter

  - Fetch Register

  - Instruction Pointer

  - Instruction Register

- What is a “Bus” ?

  - One type of interconnection structure

  - A shared transmission medium

  - Enables components of the computer system to exchange information

  - All of the above

- Which of the following is not “Element/part of Machine Instruction”?

  - Opcode

  - Label

  - Source operand address

  - Result operand address

- Which of the following statements is true concerning “Addressing Modes” of a machine instruction?

  - It enables an instruction to specify the location of an operand using different techniques

  - It refers to the width of an address bus of a computer system

  - It is a low-level/assembly language programmer visible feature of the computer system

  - All of the above

  - A & C

- Which of the following is false about “Control Unit” part of the CPU?

  - It generates control signals in a prescribed fashion

  - It performs complex data processing functions

  - It is the engine that runs the entire computer system, by activating the right control signals at the right time

  - It is the policeman/traffic manager of the CPU, directing traffic/data to different parts of the computer system

- As one goes down a “Memory Hierarchy” Diagram, which trend is observed

  - Memory capacity increases

  - Cost decreases

  - Access time/latency increases

  - All of the above

- Which of the following I/O techniques is best suited for moving a large block of data from/to memory via an I/O module to/from I/O devices?

  - Programmed I/O

  - DMA

  - Interrupt-Driven I/O

  - All of the above

- Why “External Devices/Peripherals” are not directly connected to the computer system bus?

  - Peripherals work using different data formats the CPU

  - Peripherals have varying principles of operations, and it is difficult to incorporate the necessary logic inside the processor

  - Peripherals operate at a varying speed than the CPU

  - All of the above

  - A and C

- Which of the following is an architectural attribute of a computer system?

- Control signals

- Instruction set

- Device implementation technology

- Low-level design aspect of the CPU

- Which of the following statements are true about two computer systems that have similar architecture?

- The computer systems have a similar organization

- The computer systems have a different organization�on

- The computer systems have different microarchitectures

- The computer systems could have a similar or different organization

- Which task is performed by the computer during the Fetch Phase of an Instruction Cycle ?

- Copying an instruction from memory location to IR register

- Interpreting instruction stored in IR register

- Decoding instructions stored in IR register

- Processing an instruction

- Which part of the computer system enables major components of the computer system to exchange information with one another?

- CPU Interconnect

- I/O module

- Memory Unit

- Interconnection�on structure

- Which of the following is a characteristic of a processor in a contemporary computer system?

- It is made of multiple processing units, where each unit is called a core

- Contemporary computers are multicore computers

- Each core in a multicore computer is made up of the following functional elements: Instruction Logic, ALU, Load/Store Logic...

- All of the above

- Modern computer systems employ which type of interconnection structures?

- Bus

- Multi-Bus

- Point to point

- None of the above

- A symbolic representation of a Machine instruction is called __________________.

- High-level language statement

- Opcode

- Assembly statement/instruction

- Operand

- Which of the following is an “Element of Machine Instruction”?

- Opcode

- label

- Comment

- All of the above

- What does the term “Instruction set completeness” refer to?

- List of instructions in the instruction set

- Types of instructions in the instruction set

- Components of Machine Instruction

- The sequence of instructions on execution

- Which of the following instructions does not belong to the data transfer category of instruction?

- Pop

- Move

- LD

- Store

- Shift

- What does the term “Addressing Mode” refer to?

- It is one type of Machine Instruction

- It is one way of specifying the location of an operand

- It is an organizational attribute of a computer system

- All of the above

- What does the term “Effective Address” refer to?

- It refers to the addressing mode of an instruction

- It refers to the actual address of the operand

- It refers to the address contained in PC

- All of the above

- Which of the following is false about “Control Unit” of the CPU?

- It generates control signals in a prescribed fashion

- It performs complex data processing functions

- It is the engine that runs the entire computer system, by activating the right control signals at the right time

- None of the above

- Which trend is observed going down the memory hierarchy diagram?

- Memory capacity decreases

- Latency increases

- Frequency of access increases

- Cost increases

- Which of the following I/O techniques is best suited for moving a large block of data from/to memory via an I/O module to/from I/O devices?

- Programmed I/O

- DMA

- Interrupt-Driven I/O

- All of the above

- Why “External Devices/Peripherals” are not directly connected to the computer system bus?

  - Peripherals operate using different data formats than CPU

  - Peripherals have varying principles of operations and it is difficult to incorporate the necessary logic inside the processor

  - Peripherals operate at a varying speed than CPU

  - All of the above

  - A and C

Course Name: Fundamentals of Electrical Engineering

- The equivalent resistance of the circuit in Fig. below is:

- 4 kΩ

- 5 kΩ

- 8 kΩ

- 14 kΩ

- The value of current……………..when potential difference and resistance is decreased to half?

- Same

- Increase

- Decrease

- Zero

- If there are two bulbs connected in series and one blows out, what happens to the other bulb?

- The other bulb continues to glow with the same brightness

- The other bulb stops glowing

- The other bulb glows with increased brightness

- The other bulb also burnt out

- A load is connected to a network. At the terminals to which the load is connected, RTh = 10Ω and VTh = 40 V. The maximum power supplied to the load is:

- 160 W

- 80 W

- 40 W

- 1 W

- A parallel RLC circuit has L = 2 H and C = 0.25 F. The value of R that will produce a unity damping factor is:

- 0.5 Ω

- 1Ω

- 2Ω

- 4 Ω

- To determine the polarity of the voltage drop across a resistor, it is necessary to know

- Value of current through the resistor

- Direction of current through the resistor

- Value of resistor

- e.m.f. in the circuit

- A quantity that contains all the power information in a given load is the

- Power factor

- Apparent power

- Average power

- Complex power

- In a delta system, a phase voltage of 100 V produces a line voltage of:

- 58 V

- 71 V

- 100 V

- 173 V

- In a parallel bank with unequal branch resistances

- The highest resistance has the highest current

- The lowest resistance has the highest current

- The lowest resistance has the highest voltage

- The highest resistance has the highest voltage

- In the interconnection of ideal sources shown in the figure below, it is known that the 60 V source is absorbing power. Which of the following can be the value of the current source I?

      - 10 A

      - 13 A

      - 15 A

      - 28 A

- When capacitors are connected in series, the total capacitance is _____________.

A) Greater than the smallest capacitor

B) Equal to the largest capacitor

C) Smaller than the smallest capacitor

D) Equal to the smallest capacitor

- The switch in the circuit in the below figure has been closed for a long time, and it is opened at t = 0, then VCtfor t≥0 is

      - VC(t)=15e-t V

      - VC(t)=12e-0.2t V

      - VC(t)=15e-5t V

      - VC(t)=12e-5t V

- At which of the following frequencies voltage across a capacitor have a maximum value for a series RLC circuit?

- Equal to a resonance frequency

- Slightly below a resonance frequency

- Slightly above a resonance frequency

- Zero frequency

- In an RLC series circuit, if the voltage across a capacitor is greater than the voltage across the inductor, then the power factor of the network is_________.

- Lagging

- Leading

- Unity

- Zero

- Which of the following is an incorrect statement in any AC Circuit?

- Peak value is always equal to RMS value*Peak factor.

- Average value is always equal to RMS value/Form factor.

- The algebraic sum of currents entering and leaving a junction must always be zero.

- A pure capacitor connected across an AC voltage consumes 50W.

- The current passes through a 4Ω resistor in the figure below is______. What happens if the number of stator slots in

- 5A

- 20A

- 0A

- 2.2A

- Which of the following statements is true for both series and parallel D.C. circuits?

- Currents are additive

- Voltages are additive

- Powers are additive

- Elements have individual currents

- ______ The current IS in Amps through the voltage source, and voltage VS in Volts across the current source, respectively,are_____.

- 13, -20

- -8, 20

- 8, -10

- -13, 20

- The current through 1Ω resistor in Amps is__________.

- 2

- 10

- 3.33

- 12

- A source of angular frequency 1 rad/sec has source impedance consisting of 1 Ω resistance in series with 1 F capacitance. The load that will obtain the maximum power transfer is____.

- 1 Ω resistance

- 1 Ω resistance in parallel with 1 H inductance

- 1 Ω resistance in series with 1 F capacitance

- 1 Ω resistance in series with 1 H inductance

- Consider the circuit in the figure below (All the resistor values are in kΩ). Which of the following can be the value of the current I?

    - 10 mA

    - 1 mA

    - 5 mA

    - None

- Three incandescent bulbs with 40W, 60W, and 100W ratings are connected in parallel across 220V mains. Then___.

- The bulbs together consume 200W

- All lamps will glow equally bright

- The 100 W bulb glows brighter

- The 40-W bulb glows brighter

- The average power delivered to an impedance (4 + j3) Ω by a current 6 cos (100t - 100) A is_______________.

- 144 W

- 54 W

- 72 W

- 90 W

- A 24 Ω resistor, an inductor with a reactance of 120 Ω, and a capacitor with a reactance of 120 Ω are in series across a 60 V source. The voltage across the inductor is________.

  - 300 V

  - 60 V

  - 660 V

  - None

Course name: Introduction to Electrical Machines

- The pearmibility of a ferromagnetic material is ______.

- Very low

- High

- Low

- Very High

- The magnetic field of a magnetized iron bar when strongly heated

- Becomes Stronger

- Becomes Weaker

- Reverses in direction

- Unchanged

- What is the magneto-motive force (mmf) of a wire with 8 turns carrying three amperes of current?

  - 2400 At

  - 240 At

  - 24 At

  - 2.4 At

- The strength of an electromagnet can be increased by

  - Increasing the cross-sectional area

  - Increasing the number of turns

  - Increasing current supply

  - All

- What is a Transformer?

- A device used to convert a low alternating voltage to a high alternating voltage

- A device used to convert alternating current to direct current

- A device used to convert low alternating current to a high alternating current

- are used only for low alternating voltage

- What is the function of a transformer?

A) A transformer is used to step down or up the AC voltages and currents

B) A transformer is used to step down or up the DC voltages and currents

C) The transformer converts DC to AC voltages

D) A transformer converts AC to DC voltages

- What is the working principle of a Transformer?

  - A transformer works on the principle of self-induction

  - A transformer works on the principle of mutual induction

  - A transformer works on the principle of Ampere's law

  - A transformer works on the principle of Coulomb's law

- Transformer ratings are given in _____________

A) kVA

B) HP

C) kVAR

D)  kW

- Current transformers are __________________

A) parallel-connected type of instrument transformers

B) series-connected type of instrument transformers

C) parallel-connected normal transformers

D) series-parallel connected type of instrument transformer

- The purpose of the transformer core is to provide ____________

A) Low reluctance path

B) High inductive path

C) High capacitive path

D) High reluctance path

- Which type of flux does transformer action need?

A) Alternating electric flux

B) Alternating magnetic flux

C) Increasing magnetic flux

D) Constant magnetic flux

- Primary winding of a transformer ______________

A) Could either be a low voltage or high voltage winding

B) Is always a high-voltage winding

C) Cannot be determined

D) Is always a low-voltage winding

- For a transformer with primary turns 400, secondary turns 100, if 20A current is flowing through the primary, we will get ___________

A) 800A at secondary

B) 40A at secondary

C) 80A at secondary

D) 5A at secondary

- The voltage regulation of a transformer is given by _____________

A) V2-E2/E2

C) V2-E2/V2

C) E2-V2/V2

D) E2-V2/E2

- Power transformers other than distribution transformers are generally designed to have maximum efficiency around ______

A) 10% overload

B) Near full-load

C) Half-load

D) No-load

- Which of the following is the main advantage of an auto-transformer over a two-winding transformer?

A) Eddy losses are totally eliminated

B) Copper losses are negligible

C) Saving in winding material

D) Hysteresis losses are reduced

- For the parallel operation of two single-phase transformers, it is necessary that they should have ________

A) same efficiency

B) same polarity

C) same kVA rating

D) same number of turns on the secondary side

- Transformers operating in parallel mode of operation will share the load depending upon their ___________

A) leakage reactance

B) per unit impedance

C) efficiencies

D) rating

- In terms of slip, the actual speed (N) of the induction motor can be expressed as,

A) Ns (1-s)

B) Ns (s-1)

C) Ns / (1-s)

D) Ns / (s-1)

- In an induction motor, rotor slots are usually not quite parallel to the shaft, but are given a light skew

A) To reduce the magnetic hum

B) To reduce the locking tendency of the rotor

C) Both (a) and (b) above

D) To increase the speed of the motor

- In an induction motor, the rotor runs at a speed

A) Equal to the speed of stator field

B) Lower than the speed of stator field

C) Higher than the speed of stator field

D) Having no relation to the speed of stator field

- Which one of the following statements is not true about magnetic materials and the property of magnetism?

- A current-carrying conductor placed in a magnetic field experiences a force.

- If a conductor is cutting or cut by a magnetic flux, a voltage is induced on it.

- A current-carrying conductor produces a magnetic field in its surroundings.

- A ferromagnetic material can be magnetized and demagnetized without energy loss.

- Under ideal conditions, a power transformer changes the magnitude of

- Frequency

- Voltage and current

- Power factor

- Apparent Power

- Two or more transformers can be connected in parallel, fulfilling the following conditions, except

- The transformers can have different polarities

- The transformers should be operated at the same frequency

- The transformers should have the same voltage ratios

- The transformers should have the same XeqReq ratio.

- Which one of the following statements is not true about the operation principle of three-phase induction motors?

- Three-phase induction motors set up a rotating magnetic field in the stator winding when supplied from a phase ac source.

- The rotating magnetic field runs at synchronous speed, which is less than the motor shaft speed.

- When the rotating magnetic field cuts the rotor windings, an electromotive force, emf, is induced in the rotor windings.

- The rotor experiences a rotational force due to the relative speed between the stator and rotor magnetic fields.

- While starting wound rotor induction motors, an external rheostat can be added to the rotor circuit via slip rings for the following reasons, except

- Decreasing the starting current

- Increasing the starting torque

- Controlling the speed at which maximum torque occurs

- Changing the direction of the rotor shaft speed.

- A three-phase, 50 Hz, 4-pole induction motor runs at a slip of 5%. What will be the shaft speed of this induction motor?

- 1300 rpm

- 1400 rpm

- 1425 rpm

- 1450 rpm

- The terminal voltage of a DC generator can be controlled by the following methods, except

- Phase reversal of the applied field voltage

- Varying the speed of the prime mover

- Varying the field current through a variable rheostat

- Varying field winding flux through variable rheostat

- Which of the following properties of material has no units?

- Permeability

- Moment of a magnet

- Magnetic susceptibility

- Permittivity

- Induction Motor is known as a rotating transformer with an _____ and ___________.

- Air gap & short-circuited secondary

- Air gap & Open-circuited secondary

- Coreless & short-circuited secondary

- Coreless & Open-circuited secondary

- Hunting in a synchronous motor result in___________________.

- Loss of synchronism

- Mechanical and electrical stresses on the motor

- A large current is drawn by the motor

- All of the above

- Which one of the following statements is incorrect?

- In ferromagnetic substances, all the domains get oriented in the direction of the magnetic field and remain as such even after removing the magnetic field.

- A current-carrying conductor produces a magnetic field in its surroundings

- A current-carrying conductor placed in a magnetic field experiences a force.

- Paramagnetic substances are highly attracted by magnetic fields.

- Which of the following is the main advantage of an auto-transformer over a two-winding transformer?

- Eddy losses are totally eliminated

- Copper losses are negligible

- Saving in winding material

- Hysteresis losses are reduced

- In a slip ring induction motor, the rotor resistance is__________.

- High during starting and low at normal operating speed

- Low at starting and high at normal operating speed

- Remains the same during starting and normal operating speed

- It is impossible to determine the relation between a starting and normal operating speed, rotor resistance

    - What happens if the number of stator slots in an induction motor is equal to the number of rotor slots or an integral multiple of rotor slots?

- A motor will run at a very high speed

- A motor will run at a very low speed

- A motor will fail to start

- Does not get affected

    - When the slip of an induction machine is negative, it is running as a____________.

- Motor

- Generator

- Synchronous machine

- None

    - Which one of the following statements is incorrect?

- A shunt DC generator has its field winding linked in parallel with its armature winding such that the generator's terminal voltage is applied across it.

- Separately excited DC generators are machines that transform mechanical energy into electrical energy by creating a magnetic field with a separate source of direct current.

- Electrical transformers are dynamic AC machines that change the level of alternating current.

- Self-induced EMF occurs when an EMF is generated in a conductor or coil as a result of a change in its magnetic flux linkage.

    - A machine operating as a motor may go into regenerative braking mode if its speed becomes______.

- Sufficiently high to make back emf greater than the supply voltage

- Sufficiently low to make back emf greater than the supply voltage

- Sufficiently high to make back emf half of the supply voltage

- Sufficiently low to make back emf half of the supply voltage

    - Why we can’t reduce the length of the air gap beyond a certain value in a three-phase induction motor design?

- The inductance of the stator windings will be decreased

- To avoid collision between the stator and rotor

- To reduce magnetic locking

- To produce higher torque with smaller hysteresis and eddy current losses

    - A three-phase, 50 Hz, 4-pole induction motor runs at a slip of 10%. What will be the shaft speed of this induction motor?

  - 1300 rpm

  - 1400 rpm

  - 1425 rpm

  - 1350 rpm

Course Name: Introduction to Power Systems

    - The skin effect of a conductor will reduce as the

- The resistivity of the conductor material increases

- Permeability of conductor material increases

- Diameter increases

- Frequency increases

    - The power transmitted will be maximum when

- Corona losses are minimum

- Receiving end voltage is high

- Reactance is high

- Sending end voltage is high

    - Which of the following voltage regulations is considered best?

- 10%

- 20%

- 100%

- 4%

    - For a short line, if the receiving end voltage is equal to the sending end voltage under loaded conditions

- The receiving end power factor is leading

- The sending end power factor is leading

- The receiving end power factor is unity

- The sending end power factor is unity

    - In terms of ABCD constants, the characteristic impedance of overhead transmission line is __.

- AD12

- CB12

- ABCD12

- CD12

    - Transmission efficiency of an overhead transmission line increases with as____________.

- Both voltage and power factors increase

- Both voltage and power factors decrease

- Voltage increases but power factor decreases

- Voltage decreases but power factor increases

    - Which type of insulator is used where there is a dead end of the line, a corner, or a sharp curve for a high voltage line?

- Pin-type

- Suspension type

- Strain type

- Shackle type

    - Voltage distribution across the disc of strings of suspension insulator assembly is ______.

- Same for all discs

- Maximum for unit nearest to the tower

- Maximum for unit nearest to the line

- Equal to transmission line voltage rating

    - Which one of the following statements is correct?

- Corona loss increases in solid conductors than in stranded conductor

- Corona loss is larger if the conductor temperature is low than the high temperature

- The corona loss is more on the old conductor than on the new conductor

- Corona loss will be larger if the conductor is placed at more height than less height

    - What happens if the separation between the three phases of the transmission line is increased?

- The inductance will increase and capacitance will remain unchanged

- Both inductance and capacitance will decrease

- Inductance will increase and capacitance will decrease

- Inductance will decrease and capacitance will increase

    - An overhead transmission lines are transposed to_________________.

- Reduce corona loss

- Balance line voltage drop

- Prevent short circuits between any two lines

- Reduce skin effect

    - As the height of the transmission tower is increased, the line capacitance and line inductance are respectively

- Decreases, decreases

- Increases, decreases

- Decreases, remains unaltered

- Increases, increases

    - The advantage of the underground cables over overhead transmission lines is

- easy maintenance

- low cost

- can be used in congested areas

- can be used in high-voltage circuits

- None of these

    - The insulator is so designed that it should fail primarily by________

- flash over

- puncture

- either or

- None

    - Which one of the following statements is true?

  - Resistance of a conductor decreases, and the internal inductance increases as the frequency is increased

  - Resistance and internal inductance of a conductor both increase with an increase in frequency

  - Resistance of a conductor increases, and the internal inductance decreases as the frequency is increased

  - None of these

    - Use of a bundle conductor increases

- GMR

- GMD

- potential gradient

- radius of the conductor

    - Inductance is equal to

  - Flux/current

  - Flux linkages/current

  - EMF/flux

  - EMF/current

    - Which of the following is a primary advantage of AC (Alternating Current) transmission over DC (Direct Current) transmission in electrical power systems?

A. Requires less complex infrastructure for long-distance transmission

B. Higher efficiency over long distances

C. Easier to transform one form of voltage levels to another voltage level

D. Reduced electromagnetic interference

    - Which of the following statements is correct regarding electrical loads and power characteristics?

A. For a purely resistive load, the current leads the voltage by 90 degrees

B. For a purely inductive load, the current leads the voltage by 90 degrees.

C. The instantaneous power absorbed by a capacitor is a double-frequency sinusoid with a

non-zero average value.

D. For a purely capacitive load, the current leads the voltage by 90 degrees.

    - Which statement inaccurately reflects the advantages and functionality of interconnected power systems?

  - Interconnected operation also gives the flexibility to meet emergency loads

  - The economic advantage of interconnection is to increase the reserve generation capacity in each area.

  - Interconnection allows borrowing power from adjoining areas in case of a sudden load increase or generation loss.

  - The concept of "spinning reserve" is necessary in interconnected systems to meet sudden load increases.

    - Which of the following statements accurately describes the factors influencing the parameters of a transmission line?

A. The series resistance depends on the geometrical arrangement of the conductors.

B. Both series inductance and shunt capacitance rely on the physical composition of the

conductor at a given temperature.

C. The shunt conductance is due to leakage currents flowing across insulators and air.

D. The shunt conductance and shunt capacitance depend on leakage currents flowing across insulators and air.

    - The phenomenon where, as frequency increases, the current tends to flow toward the surface of the conductor and the current density decreases at the center, causing an increase in effective resistance, is called:

A) Proximity effect

B) Ferranti effect

C) Skin effect

D) Eddy current effect

    - Which of the following statements is true regarding long transmission lines?

A) They have lower capacitance per unit length compared to short lines

B) They are more susceptible to voltage regulation issues than short lines

C) They have lower resistance per unit length compared to medium lines

D) They are typically used for local distribution rather than bulk power transfer

    - Which of the following is not a disadvantage of the corona effect?

A) Reduces the efficiency of power transmission

B) Causes audible noise and radio interference

C) Corona reduces the effects of transients produced by surges

D) Increases the power loss in the transmission line

    - Which of the following is an effective method for reducing the corona effect in transmission lines?

A) Decreasing the spacing between conductors

B) Using conductors with smaller diameters

C) Increasing the operating voltage

D) Using bundled conductors

    - Which type of insulator is typically used in areas with high mechanical stress, such as at dead-end or angle towers in distribution lines?

A) Shackle insulator

B) Suspension insulator

C) Strain insulator

D) Pin insulator

    - Which of the following is an effective method used to equalize potential across the insulator string in high-voltage transmission lines?

A) Using a single long insulator

B) Installing guard ring

C) Increasing the conductor size

D) Decreasing the spacing between conductors

    - Which one is correct for the combination of LC loads according to your lab session

- The source current decreases as the capacitive load starts to supply most of the reactive power required by the inductive load

- The source current increases as compared to other combination of loads due to their property of lagging and leading power factor

- When the two loads have equal reactance, they have positive reactive power deflection as the inductor has more effect

- They have a small positive real power deflection due to the presence of capacitive load

- A and D

    - Corona loss increases with

- Decrease in conductor size and increase in supply frequency

- Increase in conductor size and decrease in supply frequency

- Increase in both conductor size and supply frequency

- Decrease in both conductor size and supply frequency

    - What is the most common cause of failure of overhead line insulators?

      - Flashover

      - Porosity of materials

      - Mechanical stress

      - Improper vitrification

    - Which one of the following statements is correct during RC load?

- The real power affected when the resistive load is switched on and off

- Real power affected when the capacitive load is switched on and off

- The reactive power affected when the resistive load is switched on and off

- Both real and reactive powers are affected slightly when load R is switched on and off

    - What is the reason for unequal distribution of voltage among different unit of suspension insulator?

- Unequal capacitance of different units

- Unequal distribution of charging current caused by stray capacitance

- Unequal resistivity of different units

- Dirt deposited over the insulator disc

    - By using which of the following method hundred percent string efficiency can be achieved?

A) Using long cross arms

B) Static shielding

C) Capacitance grading

D) 100% string efficiency cannot be achieved

    - Which one of the following is true in regards of better system performance?

- Both efficiencies and regulation must be high

- Both efficiencies and regulation must be low

- Efficiencies must be high and regulation must be low

- Efficiencies can have any value but the regulation must be high

    - The only advantage of corona is that

A) Ozone gas is produced

B) makes line current sinusoidal

C) works as a safety valve for surges

D) produces a pleasing luminous glow

E) None

    - Which one of the following is correct?

- Corona loss increases in DC circuit than AC

- Corona loss independent of the supply whether it is AC or DC

- The corona loss on the middle conductor is less as compared with the two outer conductors.

- The corona loss is more on hilly areas than on plain areas

    - The function of guard ring transmission lines is

A) To reduce the transmission losses

B) To increase the earth capacitance of the lowest unit

C) To reduce the earth capacitance of the lowest unit

D) All

    - Temperature increase produces which of the following effect on a transmission line?

- Tension of the conductor and its sag decreases

- Tension of the conductor and its sag increases

- Tension of the conductor decreases and its sag increases

- Tension of the conductor increases and its sag decreases

    - For long distance transmission at higher voltage______________ are employed.

A) Wooden poles

B) Steel poles

C) RCC poles

D) Steel towers.

    - Triangularly arranged three-phase 160km long transmission line with 1.036cm conductor diameter and spaced at 2.44m, is subjected to transmit power at 66 kV supply voltage, frequency of 50 Hz, air temperature of 26.67°C, barometric pressure of 731.5 mm-Hg. If the surface factor Mv and Mo are 0.83 and 0.92, respectively. Then which one of the following statements is correct? (Vd is critical disruptive voltage, Vv is visual critical voltage).

- Corona happens and the value of Vd is 103.12kV(L-L)

- Corona happens and the value of Vv is 132.82kV(L-L)

- Corona happens and the value of Vd is 59.54kV(rms)

- Corona happens and the value of Vv is 76.68kV(rms)

- The system is free from corona effect under the prescribed operating conditions

Course Name: Applied Electronics I

    - Which one of the following is not true about the barrier potential in PN-junctions?

- The direction of the barrier potential is from N-type to P-type layer

- The barrier potential prevents diffusion of majority carriers across the junction

- The barrier potential will assist minority carriers in crossing the PN-junction

- When the doping concentration is high, the width of the depletion region will be large

    - Holes in an N-type semiconductor are:

A. Minority carriers that are thermally produced

B. Minority carriers that are produced by doping

C. Majority of carriers that are thermally produced

D. Majority carriers that are produced by doping

    - When transistors are used in amplifier circuits, they usually operate in the:

- Active region

- Breakdown region

- Saturation region

- Cutoff region

    - A BJT has a βDC of 10 and a base current IB of 0.5 µA. Then the Emitter current, IE is;

- 10.5 µA

- 5 µA

- 5.5 µA

- 0.5µA

    - The primary purpose of supplying biasing DC voltage in amplifier circuits is to;

- Hold the amplifier stable at DC supply voltage (VCC )

- Hold the amplifier stable at the input voltage (vin)

- Ensure proper gain is achieved

- Hold the amplifier stable at a desired operating point

    - One of the following BJT amplifiers is used as a buffer to isolate a low impedance load from a high impedance driving circuit.

- Common Emitter amplifier

- Common base amplifier

- Emitter Follower amplifier

- Current mirror

    - One of the following is not true about the JFET transistor

- They are unipolar devices

- They are voltage-controlled devices

- They are a current-controlled device

- No electrical connection between input and output

    - In a three-stage amplifier, if the individual ideal voltage amplifier stages have gains of 10 dB, 5 dB, and 12 dB, then the total gain is.

- 600 dB

- 24 dB

- 14 dB

- 27 dB

    - When a multistage amplifier is to amplify a D.C. signal, then one must use:

A. RC coupling.

B. Transformer coupling.

C.  Direct coupling.

D.  Capacitor coupling

    - In electronic circuits, there is always a need to limit voltage below a certain level to protect successive circuits. Which electronic circuit is commonly used in the design of such voltage limiter circuits?

- Reverse-bias transistor

- Zener diodes

- Photodiodes

- JFET transistors

    - What is the primary purpose of doping intrinsic semiconductors with impurity atoms?

A. To reduce concentration of charge carriers within semiconductors

B. To improve mechanical strength of semiconductors

C. To improve conductivity of semiconductors

D. To avoid recombination of holes and electrons

    - Which one of the following is not a correct phenomenon when p-type and n-type semiconductors are grown together side by side?

A) At equilibrium, stationary negative ions are created at the n-type material

B) Electrons flow from n-type to p-type until equilibrium condition reaches

C) Holes flow from p-type to n-type until equilibrium condition reaches

D) At equilibrium point, the developed built-in potential is in the direction from n-type to p-type region

    - A silicon diode is connected in series with 1KΩ resistor. If the circuit is driven by a 3V source with the diode being in the forward biased mode, what is the magnitude of the
current through the diode?

A. 3A

B. 2.3mA

C. 2.3A

D. 0A

    - The operating point of transistor amplifiers is determined by __________.

A. The AC input voltage

B. Biasing DC voltage

C. Load connected to the amplifier

D. Operating point is constant and set during fabrication of the transistor.

    - Which one of the following amplifier biasing techniques will provide most stable
quotient point?

A. Fixed biasing

B. Voltage feedback biasing

C. Voltage divider biasing

D. Emitter resistor biasing

    - When you test a good diode with a digital multi meter, it shows;

A. Low resistance when forward biased or reverse biased

B. High resistance when forward biased or reverse biased

C. High resistance when forward biased and Low resistance when reverse biased

D. Low resistance when forward biased and High resistance when reverse biased

    - A zener diode could normally operate within

A. Reverse breakdown region

B. The forward-bias region

C. The reverse-bias region

D. B and C

    - A two-stage amplifier has individual stage gains of 10 and 8. Input and output resistances of the individual stage are 3KΩ and 1KΩ respectively. Then the overall gain of the amplifier is;

A. 18

B. 80

C. 20

D. 240

    - Often a common-collector will be the last stage before the load; the main function of
this stage is to;

A. Provide voltage gain

B. Buffer the voltage amplifiers from the low-resistance load

C. Provide phase inversion

D. Provide a high-frequency path to improve the frequency response

    - Which one of the following is not correct about JFET transistors?

A. Applying negative VGS will decrease channel resistance of the transistor.

B. Drain current is constant after the pinch-off point

C. Current through the base terminal is zero.

D. Applying negative VGS will cause saturation at lower VDS voltage

    - The width of the depilation region of a PN-junction is;

  - Directly proportional to the doping concentration

  - Inversely proportional to the doping concentration

  - Increase when forward biased

  - Decrease when reverse biased condition

    - Each diode of the full-wave bridge rectifier conducts for ___________of its period.

  - 45o

  - 90o

  - 180o

  - 360o

    - ____________ is a type of diode that produces a constant voltage within its reverse breakdown region and is commonly used in voltage regulator circuits.

  - Light Emitting Diode

  - Schottky Diode

  - Zener Diode

  - Varactor

    - In a properly biased Bipolar Junction Transistor within the active region, most of the electrons that enter the Emitter will;

  - Pass through the base into the collector terminal

  - Flow out of the base terminal

  - Recombine with holes at the base layer

  - Are stopped by the junction barrier

    - When transistors are used in amplifier circuits, they usually operate in the:

  - Active region

  - Breakdown region

  - Saturation region

  - cutoff region

    - When the temperature of a doped semiconductor is increased, its conductivity

A) Decreases.

B) Increases.

C) Increases or decreases depending on whether it is p- or n-type.

D) Does not change.

    - Which of the following doping will produce a p-type semiconductor?

- Germanium with phosphorus

- B) Silicon with Germanium

- C) Germanium with Antimony

- Silicon with Indium

    - In an intrinsic semiconductor, the Fermi level is

  - closer to the valence band

  - (B) closer to the conduction band

  - (C) within the valence band

  - (D) midway between conduction and valence band

    - The diode in which impurities are heavily doped is:

    - Varactor diode

    - ( B)  PIN diode

    - Tunnel diode

    - Zener diode

    - The depletion region in a Junction Diode contains:

(A) only charge carriers (of minority type and majority type)

(B) no charge at all

(C) vacuum, and no atoms at all

(D) only ions, that is immobile charges

    - A semiconductor has ………… temperature coefficient of resistance.

- Positive

- Zero

- Negative

- None of the above

    - The random motion of holes and free electrons due to thermal agitation is called ….

- Diffusion

- Pressure

- Ionization

- None of the above

    - In an N-type semiconductor, the position of Fermi-level:

(A) Near valance Band

(B) Near Conduction Band

(C) In valance Band

(D) In conduction Band

    - A diode conducts when it is forward-biased, and the anode is connected to the ________ through a limiting resistor.

(A) positive supply

(B) negative supply

(C) cathode

(D) anode

    - You have an application for a diode to be used in a tuning circuit. A type of diode to use might be

    - an LED.

    - a Schottky diode.

    - a Gunn diode.

    - ( D ) a varactor.

    - Refer to this figure. Which symbol is correct for an LED?

(A)  a

(B)  b

(C)  c

(D)  d

(E)  e

    - The process of emitting photons from a semiconductive material is called:

(A) photoluminescence

(B) gallium arsenide.

(C) electroluminescence.

(D) gallium phosphide.

    - For the BJT to operate in the saturation region, the base-emitter junction must be ________-biased and the base-collector junction must be ________-biased.

(A) forward, forward

(B) forward, reverse

(C) reverse, reverse

(D) reverse, forward

    - A transistor in common emitter mode has:

(A) a high input resistance and low output resistance

(B) a medium input resistance and high output resistance

(C) a very low input resistance and a low output resistance

(D) a high input resistance and a high output resistance

    - In a clamping circuit, the peak-to-peak voltage of the waveform being clamped is:

(A) affected by the clamping

(B) not affected by the clamping

(C) determined by the clamping voltage value

(D) determined by the ratio of rms voltage of the waveform and the clamping voltage

    - The forward characteristic of a diode has a slope of approximately 50mA/V at a desired point. The approximate incremental resistance of the diode is:

(A) 50 Ω

(B) 35Ω

(C) 20 Ω

(D) 10 Ω

    - Identify the condition for a transistor to act as an amplifier.

(A) The emitter-base junction is forward biased and the base-collector junction is reverse.

(B) No bias voltage is required

(C) Both junctions are forward-biased

(D) Both junctions are reverse-biased

    - A collector characteristic curve is a graph showing ………..

- emitter current (IE) versus collector-emitter voltage (VCE) with (VBB) base bias voltage held constant

- collector current (IC) versus collector-emitter voltage (VCE) with (VBB) base bias voltage held constant

- collector current (IC) versus collector-emitter voltage (VC) with (VBB) base bias voltage held constant

- collector current (IC) versus collector-emitter voltage (VCC) with (VBB) base bias voltage held constant

    - In a JFET, at pinch-off voltage applied to the gate:

(A) The drain current becomes almost zero

(B) The drain current begins to decrease

(C) The drain current is almost at saturation value.

(D) The drain-to-source voltage is close to zero volts.

    - For a typical bipolar junction transistor, the doping concentration and cross-sectional area of the Emitter(E), Base(B), and Collector(C) region varies respectively as:

- E > C > B and E > C > B

- (B).  E > C > B and C > E > B

(C) E > B > C and E > C > B

(D)  C > E > B and C > E > B

    - The lowest output impedance is obtained in case of BJT amplifiers for

(A) CB configuration.

(B) CE configuration.

(C) CC configuration.

(D) CE with RE configuration.

    - As the temperature of a transistor goes up, the base-emitter resistance ……………

(A)Decreases

(B) Increases

(C) Remains the same

(D) None of the above

    - Which of the following is not a FET amplifier Configuration?

- Common base amplifier

- Common drain amplifier

(C) Common source amplifier

(D) Common gate amplifier

    - The current amplification factor in CE configuration is

(A) α

(B) β + 1

(C) 1/ β

(D) β

    - With the positive probe on an NPN base, an ohmmeter reading between the other transistor terminals should be ……

(A)open

(B) infinite

(C) low resistance

(D) high resistance

    - The gate voltage in a JFET at which drain current becomes zero is called ________ voltage

(A)  Saturation

(B)  Pinch-off

(C)  Active

(D)  Cut-off

    - A MOSFET differs from a JFET mainly because:

(A).  Of the power rating

(B) the MOSFET has two gates

(C) the JFET has a PN junction

(D) MOSFETs do not have a physical channel

    - The constant-current region of a JFET lies between:

(A) Cut off and saturation

(B) Cut off and pinch-off

(C) Zero and IDSS

(D) Pinch-off and breakdown

    - An n-channel D-MOSFET with a positive VGS is operating in:.

(A) The depletion mode

(B) the enhancement mode

(C) cutoff

(D) saturation

    - Drain current in the constant-current region increases when

(A) The gate-to-source bias voltage decreases

(B) The gate-to-source bias voltage increases

(C) The drain-to-source voltage increases

(D) The drain-to-source voltage decreases

    - In the voltage range, Vp < VDS < BVDSS of an ideal JFET or MOSFET

(A) The drain current varies linearly with VDS.

(B) The drain current is constant.

(C) The drain current varies nonlinearly with VDS.

(D) The drain current is cut off.

    - The total gain of a multistage amplifier is less than the product of the gains of individual stages due to________

(A)Power loss in the coupling device

(B)Loading effect of the next stage

(C)The use of many transistors

(D)The use of many capacitors

    - Enhancement mode device acts as ____ switch, depletion mode acts as _____ switch.

(A) open, closed

(B) closed, open

(C) open, open

(D) close, close

    - Depletion mode MOSFETs are more commonly used as ____________

(A) Switches

(B) Resistors

(C) Buffers

(D) Capacitors

    - Enhancement mode MOSFETs are more commonly used as ____________

(A) Switches

(B) Resistors

(C) Buffers

(D) capacitors

    - What configuration is widely preferred in cascading amplifiers?

(A) Common Emitter

(B)  Common Base

(C) Common Collector

(D) Common Source

    - Compared to a bipolar transistor, a JFET has:

(A) Lower input impedance

(B) Higher voltage gain

(C) Higher input impedance and high voltage gain

(D) Higher input impedance and low voltage gain

    - What happens to bandwidth if the total gain increases in cascade amplifiers?

- Increases

- Decreases

- Constant

- Remain Same

    - The principal advantage(s) of MOSFETs over BJTs is (are)

- Their biasing networks are simpler

- Their drive requirements are simpler.

- They can be connected in parallel for added drive capability.

- All of the above

    - If Av, Ai and Ap represents the voltage gain, current gain and power gain ratio of an amplifier which of the below is not the correct expression for the corresponding values in decibel?

(A) Current gain: 20 log Ai db

(B) Voltage gain: 20 log Av db

(C) Power gain: 20 log Ap db

(D) Power gain: 10 log Ap

    - An amplifier has a voltage gain of 100 V/V and a current gain of 1000A/A. the value of the power gain decibel (db) is:

(A) 30 db

(B) 40 db

(C) 50 db

D) 60 db

    - What is meant by stability of an amplified signal?

(A) The amplified signal must have a finite amplitude

(B) The amplified signal should not have self-oscillation

C) The input and the output signal must be proportional

(D) The ratio of the input and the output signal must be finite

    - Which type of power amplifier is biased for operation at less than 180º of the cycle?

- Class A

- ( B) Class B and AB

- Class C

- ( D) Class D

    - Class ________ amplifiers are normally operated in a push-pull configuration in order to produce an output that is a replica of the input.

(A) A

(B) B

(C) C

(D) AB

    - Which of the following devices has the highest input resistance?

(A) diode

(B) JFET

(C) MOSFET

(D) bipolar junction transistor

    - In an RC coupled amplifier, the voltage gain over mid-frequency rang…….

- Changes abruptly with frequency

- Is constant

- Changes uniformly with frequency

- None of the above

    - In obtaining the frequency response curve of an amplifier, the …………

- Amplifier level output is kept constant

- Amplifier frequency is held constant

- Generator frequency is held constant

- Generator output level is held constant

    - The main characteristics of a Darlington Amplifier are

(A) High input impedance, high output impedance and high current gain.

(B) Low input impedance, low output impedance and low voltage gain.

(C) High input impedance, low output impedance and high current gain.

(D) Low input impedance, low output impedance and high current gain.

    - The upper cutoff frequency of an RC coupled amplifier mainly depends upon:

(A) Coupling capacitor

(B) Emitter bypass capacitor

(C) Output capacitance of signal source

(D) Inter-electrode capacitance and stray shunt capacitance

    - Removing bypass capacitor across the emitter-leg resistor in a CE amplifier causes

(A) increase in current gain.

(B) decrease in current gain.

(C) increase in voltage gain.

(D)decrease in voltage gain.

Course name: Introduction to Control Systems

    - When a unity-gain second-order system is subjected to a unit step input, its transient response contains a first overshoot of 77%, occurring after 32.5 milliseconds has elapsed. Which of the following is correct?

- The damped natural frequency is 90.66 rad/sec

- The system transfer function Gs=943.4s2+1.615s+943.4

- The time to settle down to within ±2% of the final value is 0.386 sec

- The damping ratio is 0.83

    - Which of the following is not correct about the standard test input signal for a control system?

- A step input is used to analyze both transient and steady state response of a control system

- A parabolas input is used in position control systems analysis to represent accelerating targets, such as the missile

- The ramp input is used to test a system's ability to follow a linearly increasing input

- Impulse input is used to analyze the steady state response of a control system

    - Which of the following is not correct about root locus?

- It gives us guidelines for analyzing absolute stability and compensator design

- If the number of open loop poles and zeros are equal, then the root locus will have asymptote angle

- A point on the root locus always satisfy both angle and magnitude conditions

- Center of asymptote of the root locus is always real and may or may not part of the root locus

    - A unity feedback system with a closed loop transfer function, CsRs=2s+1s4+6s3+2s2+s+3 , which of the following is not correct?

- The steady-state error to a unit step input is 1/3

- The steady-state error constant (Kv) to a unit ramp input is 0

- The position error constant is 2/3

- The system is type 0 system.

    - Which of the following is not true in frequency response of a control system?

- The frequency response of a control system due to sinusoidal input is always sinusoidal

- Resonant frequency is the frequency at which peak resonant (Mr) occurs

- If a Nyquist plot of GjωH(jω) for a closed loop system passes through (-2,j0) point in GH plane, the value of gain margin is 6.02dB

- None of the above

    - Which of the following is correct about lead compensator?

- Lead compensator increases the damping ratio of a closed-loop system

- Lead compensator greatly affects the steady state error

- A single lead compensator can provide a lead angle of greater than 60°

- Lead compensator increases overshoot and reduces the bandwidth

    - Open open-loop system is advantageous over a closed-loop system, because of__?

- It is very expensive

- It requires easy maintenance

- It is convenient when output measurement is not difficult

- It requires calibration from time to time

    - Which of the following is correct about PI controller?

- It increases the type of system by one

- It reduces the rise time and settling time

- It improves the transient response and affects the steady-state response

- It will increase the steady-state error

    - Which one of the following is correct about classical control theory?

- An exact description of the internal system dynamics is not needed for classical controller design.

- Classical control theory is difficult to apply in a Multi-Input-Multi-Output (MIMO) system.

- It is based on the principles of feedback and linear time-invariant system theory.

- B & C.

- All are correct.

    - Closed loop control systems should not have which of the following properties?

- Good regulation against disturbances.

- Desirable responses to commands.

- High sensitivity to the changes in the plant parameters.

- None of the above.

    - Which of the following is not correct about transfer function?

- Transfer function is a property of a system itself, independent of the magnitude and nature of the input.

- If the transfer function of the system is unknown, it can be established experimentally.

- Transfer function is applicable to a non-linear-time-varying system.

- The transfer function of many physically different systems can be identical.

    - Consider the R-L-C filter shown in the figure below, with input current Is(t) and output voltage V0t which of the following statement is correct.

- The steady state response of the system depends on the value of R, L & C

- The damping ratio is zero if the resistor is removed.

- The output voltage (response) is strictly under damped system.

- The transient response highly depends on the input current.

- All are correct.

    - Which of the following statements is correct?

- The dynamic behavior or performance of first-order systems is uniquely determined by the time constant.

- As the damping ratio of a second-order system increases from zero to onwards, the system response becomes more oscillatory.

- Sometimes it is possible for the control system response to reach to the steady state before passing the transient time.

- The dynamic behavior (transient period) of a higher-order system cannot be approximated as second-order system behavior.

    - Which of the following statements is not correct?

- The phase lead compensator helps to shift the root locus towards the left in the complex s-plane, and it further increases the system's stability.

- Root Locus is used to find the limit of the system gain K for any point on the root locus

- The Bode plot provides the relative stability of the system in terms of the gain margin and phase margin.

- Routh-Hurwitz Criterion can tell us the stability of the control system and the exact location of closed-loop poles on a complex S plane.

    - An open-loop transfer function is given by  Gs=Ks + 2(s + 3)(s2 + 3s + 2) It has:

- One zero at infinity

- Three zeroes at infinity

- Two zeroes at infinity

- Four poles on left half side of S plane

    - Which of the following is correct in Bode plot?

- If the gain margin of an open-loop system is 0 dB, then the system is unstable.

- If the gain crossover frequency is greater than the phase crossover frequency the system is stable.

- The phase angle of a system with a double pole at the origin is -180 degrees with -40dB/decade slope of the magnitude curve.

- The gain margin is an indication of how much additional gain at a gain crossover frequency can be withstood by the system before the system becomes unstable

    - The controller required to handle fast process load changes is:

- PD controller

- PI Controller

- PID Controller

- None of the above

    - Let we assume that you have designed a lift (elevator) for a residence building which has five floors; and the lift has been approximated as a second order system and its position at each floor has been controlled. The lift was initially at the first floor and a person calls the lift at the fourth floor. Which one of the following could be the best time response characteristics of the lift (elevator) car position?

- Maximum overshot: 15%; rising time: 3 sec; settling time: 6 sec; steady state error: 10%

- Maximum overshot: 15%; rising time: 3 sec; settling time: 60 sec; steady state error: 10%

- Maximum overshot: 0.0%; rising time: 40 sec; settling time: 60 sec; steady state error: 0.0%

- Maximum overshot: 0.0%; rising time: 3 sec; settling time: 6 sec; steady state error: 0.0%

- Maximum overshot: 0.0%; rising time: 10 sec; settling time: 2 sec; steady state error: 0.0%

    - Which one of the following is the importance of control/ automatic control in the production process/ plant?

- Maintain the plant/ process production rate at minimum cost.

- Prevent injury to plant personnel, protect the environment by preventing emission and minimizing waste and prevent damage to the process equipment.

- Maintain product quality (composition, purity, color, etc.) on a continuous basis and with minimum cost.

- All

    - Which one of the following is the advantage of open loop control systems over closed loop control system?

- Have high bandwidth i.e., high operating frequency zone

- Are convenient to use when output is difficult to measure

- Accuracy is very high due to the correction of any arising error

- Less sensitivity to disturbance

- All

    - A negative feedback closed-loop system was subjected to 100V input voltage. The system has a forward path gain of 10 and a feedback gain of 0.9. Determine the output voltage and the error voltage.

- 100V, 20V

- 1000V, 90V

- 10V, 20V

- 100V, 10V

    - Transfer function model of a system is applicable to which one of the following?

- Linear systems

- Nonlinear systems

- Linear and time varying systems

- Linear and time in variant systems

- Nonlinear and time variant systems

    - Which one of the following statements is not correct?

- Stability of the system is directly dependent on the roots of the system characteristic equation.

- For BIBO stability, the roots of the characteristic equation must all lie in the left-half s-plane.

- The experimental determination of Transfer Function is possible in frequency domain.

- Bode plot is the best method for determining the stability and transient response over Root locus.

- None.

    - If the gain of the critically damped system is increased, it will behave as:

- Undamped

- Overdamped

- Underdamped

- Critically damped

    - Which of the following is not correct about a compensator?

- Lead compensator is used to reduce the settling time.

- Lead compensator can reduce the number of branches of root locus that cross the imaginary axis into the right half s plane.

- Lag compensator is used to reduce steady-state error.

- In a lag compensator, the magnitude of the compensator pole is less than its zero.

Course Name: Fundamentals of Computer Programming

    - Which of the following is used for comments in C++?

A. /* comment */

B.// comment */

C. // comment

D. both // comment or /* comment */

    - Which of the following is a correct identifier in C++?

A. 7var_name

B. 7VARNAME

C.  VAR_1234

D.  $var_name

    - What is the size of a boolean variable in C++?

- 1 bit

- 1 byte

- 4 bytes

- 2 bytes

    - Which of the following is an abstract data type?

- int

- float

- class

- string

    - Which of the following is an exit-controlled loop?

- For

- while

- do-while

- all of the mentioned

    - Which one is the correct syntax for declaring arrays in C++?

- Array arr[10]

- Array{10}

- int arr[10]

- int arr

    - How do structures and classes in C++ differ?

- In Structures, members are public by default, whereas in Classes, they are private by default

- In Structures, members are private by default whereas in Classes, they are public by default

- Structures by default hide every member whereas classes do not

- Structures cannot have private members whereas classes can have

    - What should be printed on screen?
int main() {
int x = 3
if(x++ = = 3)
cout<<"Three"<<endl;
else
if(++x = = 4)
cout<<"Four"<<endl;
return 0;
}

- Three Four

- Three

- Four

- None of these

    - Which of the following is NOT a type of algorithm?

- Program

- Flowchart

- Decision Table

- Pseudocode

    - What is a program or set of instructions that a computer can execute called?

- Algorithm

- Script

- Software

- Compiler

    - Which data type is used to store whole numbers in programming?

- Float

- String

- Integer

- Boolean

    - Which of the following functions is used to erase a specified file?

- rewind(FILE *fp)

- ferror(FILE *fp)

- remove(const char *filename);

- fflush(FILE *fp);

- We can’t delete file using code, we have to delete manually

    - Which of the following is not the correct way to define an array variable?

- float scores[2][3] = {{1, 2, 3}, {4, 5, 6}};

- double temperature[10][12][30];

- int stud[50];

- char name[50][100];

- double age;

    - Which of the following functions is used to concatenate 2 string variables in c++

- atoi(char  s[]), atol(char s[]), atof(char s[])

- strcmp(char first[], char second[])

- strcpy(char first[], char second[])

- strcat(char first[], char second[])

- strlen(char str[])

    - Which of the following function parts(attribute) is optional when we define function in c++

- function name

- function return type

- function body

- function parameter

- everything is optional

    - Which loop is ideal for situations where a pretest loop is required?

- For loop

- While loop

- Do…while loop

- Recursive loop

- All

    - If we have x=67 and y=100 initially what will be the final values of x and y after this line of code executed ‘ (x < y) ? x = 10 : y = 10; ‘

- X will have a value of 67 and y will have a value of 100

- X will have a value of 100 and y will have a value of 67

- X will have a value of 67 and y will have a value of 10

- X will have a value of 10 and y will have a value of 100

- X will have a value of 67 and y will have a value of 100

    - Which of the following is the correct syntax for a for loop in C++?

A. for(int i = 0; i < 10; i++) { }

B. for(int i = 0, i < 10, i++) { }

C. for(int i = 0; i < 10; i = i + 1) { }

D. for i in range(10) { }

    - What is the size of an int in C++?

A. 2 bytes

B. 4 bytes

C. 8 bytes

D. 1 byte

    - What does the break statement do in a loop?

A. Exits the current loop and moves to the next iteration

B. Exits the current loop completely

C. Pauses the loop for a specified time

D. Skips the current iteration and moves to the next one

    - Which operator is used to access members of a structure in C++?

A. .

B. ->

C. &

D. *

    - What is the default value of an uninitialized static variable in C++?

A. 0

B. null

C. undefined

D. Garbage value

    - Which of the following is used to declare a function in C++?

A. def functionName()

B. function functionName()

C. void functionName()

D. functionName()

    - What is the scope of a local variable in C++?

A. Throughout the program

B. Inside the function where it is declared

C. Inside the class where it is declared

D. Inside the namespace

    - Which of the following is a valid C++ identifier?

A. 123variable

B. _variable123

C. var-123

D. @var123

    - What does the continue statement do in a loop?

A. Skips the current iteration and continues to the next one

B. Exits the loop completely

C. Pauses the loop for a specified time

D. Terminates the program execution

    - How do you allocate memory dynamically for an array of 10 integers in C++?

A. int arr[10]

B. int* arr = new int[10];

C. int arr = malloc(10 * sizeof(int));

D. int arr[10] = malloc(10);

    - Which function is used to find the length of a string in C++?

A. strlen()

B. length()

C. size()

D. count()

    - Which of the following is a correct way to define a function that returns a pointer in C++?

A. int* func();

B. int func*();

C. *int func();

D. func* int();

    - Which of the following statements is used to terminate a function in C++?

A. stop()

B. exit()

C. return

D. break

    - What is the size of the char data type in C++?

A. 1 byte

B. 2 bytes

C. 4 bytes

D. 8 bytes

    - Which of the following is an example of a reference in C++?

A. int &x = 5;

B. int *x = 5;

C. int x = 5;

D. int &&x = 5;

    - Which loop is guaranteed to run at least once in C++?

A. for

B. while

C. do-while

D. foreach

    - What is the correct way to declare a pointer in C++?

A. int* ptr;

B. ptr int;

C. ptr int;

D. int ptr;

    - Which of the following is true about a 'destructor' in C++?

A. It is used to initialize an object

B. It is invoked when an object is destroyed

C. It must have a return type

D. It can be overloaded

    - Which header file is needed to use the cout function in C++?

A. iostream.h

B. iostream

C. stdio.h

D. string.h

    - Which of the following operators is used to dereference a pointer in C++?

A. &

B. *

C. %

D. ->

    - Which of the following is not a valid operator in C++?

A. &&

B. ||

C. !

D. &!

    - Which of the following is used to free dynamically allocated memory in C++?

A. delete

B. free

C. destroy

D. deallocate

    - Which of the following is not an access modifier in C++?

A. public

B. private

C. protected

D. global

    - Which of the following is NOT a valid way to declare an array in C++?

A. int arr[10];

B. float arr[] = {1.1, 2.2, 3.3};

C. int arr = {1, 2, 3};

D. char arr[5];

ANSWERS

| 1 | A | 60 | D | 119 | A | 178 | D | 237 | D | 296 | A | 355 | E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | C | 61 | A | 120 | C | 179 | C | 238 | A | 297 | A | 356 | C |
| 3 | D | 62 | D | 121 | B | 180 | C | 239 | C | 298 | B | 357 | C |
| 4 | C | 63 | A | 122 | B | 181 | A | 240 | B | 299 | D | 358 | A |
| 5 | D | 64 | C | 123 | D | 182 | B | 241 | C | 300 | D | 359 | A |
| 6 | C | 65 | D | 124 | B | 183 | D | 242 | C | 301 | C | 360 | D |
| 7 | B | 66 | D | 125 | D | 184 | C | 243 | B | 302 | D | 361 | C |
| 8 | A | 67 | D | 126 | D | 185 | C | 244 | B | 303 | C | 362 | D |
| 9 | D | 68 | D | 127 | A | 186 | D | 245 | C | 304 | A | 363 | C |
| 10 | C | 69 | B | 128 | C | 187 | B | 246 | A | 305 | B | 364 | D |
| 11 | C | 70 | D | 129 | A | 188 | D | 247 | C | 306 | A | 365 | D |
| 12 | A | 71 | B | 130 | B | 189 | D | 248 | B | 307 | D | 366 | D |
| 13 | B | 72 | C | 131 | D | 190 | A | 249 | B | 308 | E | 367 | B |
| 14 | C | 73 | A | 132 | A | 191 | C | 250 | C | 309 | C | 368 | D |
| 15 | C | 74 | B | 133 | C | 192 | A | 251 | D | 310 | B | 369 | D |
| 16 | D | 75 | B | 134 | B | 193 | B | 252 | B | 311 | B | 370 | D |
| 17 | D | 76 | C | 135 | B | 194 | B | 253 | D | 312 | B | 371 | B |
| 18 | D | 77 | D | 136 | D | 195 | C | 254 | C | 313 | D | 372 | D |
| 19 | C | 78 | A | 137 | C | 196 | D | 255 | B | 314 | A | 373 | C |
| 20 | A | 79 | C | 138 | A | 197 | A | 256 | C | 315 | B | 374 | C |
| 21 | D | 80 | B | 139 | D | 198 | A | 257 | D | 316 | C | 375 | B |
| 22 | A | 81 | A | 140 | C | 199 | B | 258 | C | 317 | C | 376 | C |
| 23 | B | 82 | B | 141 | D | 200 | A | 259 | B | 318 | C | 377 | C |
| 24 | D | 83 | D | 142 | B | 201 | B | 260 | A | 319 | A | 378 | C |
| 25 | C | 84 | A | 143 | C | 202 | A | 261 | A | 320 | A | 379 | A |
| 26 | D | 85 | D | 144 | A | 203 | B | 262 | A | 321 | D | 380 | D |
| 27 | B | 86 | C | 145 | D | 204 | A | 263 | D | 322 | D | 381 | A |
| 28 | B | 87 | B | 146 | D | 205 | D | 264 | A | 323 | B | 382 | C |
| 29 | B | 88 | E | 147 | B | 206 | C | 265 | D | 324 | C | 383 | C |
| 30 | B | 89 | B | 148 | E | 207 | B | 266 | C | 325 | B | 384 | C |
| 31 | C | 90 | C | 149 | B | 208 | C | 267 | C | 326 | B | 385 | E |
| 32 | A | 91 | A | 150 | D | 209 | B | 268 | D | 327 | B | 386 | D |
| 33 | D | 92 | C | 151 | B | 210 | B | 269 | C | 328 | A | 387 | D |
| 34 | C | 93 | D | 152 | D | 211 | C | 270 | C | 329 | B | 388 | A |
| 35 | C | 94 | B | 153 | B | 212 | C | 271 | D | 330 | A | 389 | C |
| 36 | B | 95 | A | 154 | D | 213 | B | 272 | E | 331 | B | 390 | A |
| 37 | B | 96 | B | 155 | A | 214 | D | 273 | D | 332 | A | 391 | B |
| 38 | B | 97 | E | 156 | D | 215 | B | 274 | A | 333 | C | 392 | B |
| 39 | A | 98 | B | 157 | D | 216 | A | 275 | A | 334 | D | 393 | A |
| 40 | C | 99 | A | 158 | C | 217 | B | 276 | C | 335 | B | 394 | A |
| 41 | A | 100 | D | 159 | C | 218 | D | 277 | D | 336 | D | 395 | C |
| 42 | D | 101 | D | 160 | A | 219 | C | 278 | C | 337 | C | 396 | B |
| 43 | D | 102 | C | 161 | B | 220 | A | 279 | C | 338 | D | 397 | B |
| 44 | C | 103 | A | 162 | E | 221 | C | 280 | B | 339 | B | 398 | A |
| 45 | A | 104 | D | 163 | B | 222 | A | 281 | C | 340 | B | 399 | B |
| 46 | B | 105 | C | 164 | B | 223 | D | 282 | B | 341 | B | 400 | A |
| 47 | C | 106 | B | 165 | B | 224 | D | 283 | C | 342 | C | 401 | A |
| 48 | A | 107 | C | 166 | B | 225 | C | 284 | D | 343 | B | 402 | C |
| 49 | B | 108 | D | 167 | B | 226 | A | 285 | B | 344 | D | 403 | A |
| 50 | D | 109 | C | 168 | D | 227 | C | 286 | B | 345 | C | 404 | A |
| 51 | A | 110 | A | 169 | A | 228 | B | 287 | C | 346 | D | 405 | C |
| 52 | D | 111 | C | 170 | A | 229 | C | 288 | D | 347 | D | 406 | A |
| 53 | C | 112 | B | 171 | B | 230 | A | 289 | A | 348 | D | 407 | B |
| 54 | B | 113 | C | 172 | C | 231 | B | 290 | C | 349 | D | 408 | B |
| 55 | B | 114 | D | 173 | C | 232 | D | 291 | B | 350 | C | 409 | B |
| 56 | F | 115 | A | 174 | B | 233 | C | 292 | A | 351 | C | 410 | D |
| 57 | A | 116 | B | 175 | D | 234 | D | 293 | B | 352 | D | 411 | A |
| 58 | B | 117 | C | 176 | C | 235 | D | 294 | C | 353 | A | 412 | D |
| 59 | C | 118 | A | 177 | B | 236 | C | 295 | C | 354 | C | 413 | C |