Bahir Dar University

Bahir Dar Institute of Technology

Faculty of Electrical and Computer Engineering

Model Exit Exam Questions

Compiled by:

Tenaw Ayew

Exam Year: 2nd Semester, 2026 G.C

- A student is analyzing a pure silicon sample at room temperature and observes that its conductivity is relatively low. He explains that this is due to limited free charge carriers.
Which one of the following best explains this behavior?

A) Silicon has no electrons

B) Equal number of electrons and holes exist with limited availability

C) Silicon behaves like a metal

D)  There is no energy gap

- An engineer compares conductors, semiconductors, and insulators based on their energy band gaps.   Which one of the following correctly describes semiconductors?
A) Very large band gap                                    B) No band gap
C) Moderate (small) band gap                          D) Infinite band gap

- A student who has a strong understanding of semiconductor concepts is better able to analyze electronic circuits and devices.
Which one of the following best reflects this learning outcome?
A) Avoids practical work                                         B) Improves design and analysis skills
C) Reduces theoretical knowledge                          D) Ignores applications

- Semiconductor technology has significantly transformed communication, computing, and automation systems worldwide.
Which one of the following best describes this impact?
A) Increased manual processes                    B) Enabled modern digital and electronic systems
C) Reduced innovation                                 D) Eliminated communication

- A student connects a PN junction diode in forward bias and observes a significant current flow after a certain voltage is applied.
Which one of the following best explains this behavior?
A) Increase in depletion region width              B) Reduction in depletion region
C) Increase in resistance                                 D) Absence of carriers

- A power supply circuit uses a full-wave rectifier instead of a half-wave rectifier.
Which one of the following is the main advantage of this choice?
A) Lower output voltage                          B) Smoother DC output
C) Higher ripple                                        D) Fewer components

- A student increases the base current in a BJT circuit and observes an increase in collector current.  Which one of the following explains this relationship?
A) Current gain property                                 B) Voltage gain only
C) Resistance effect                                         D) Temperature effect

- A BJT is selected for an application requiring signal amplification.
Which one of the following is the key reason?
A) Low gain                B) High current gain                   C) No carriers     D) High resistance

- A FET is used in a circuit where the input signal controls the output current through voltage variation.   Which one of the following describes this operation?
A) Current-controlled                                       B) Voltage-controlled
C) Temperature-controlled                               D) Resistance-controlled

- A MOSFET is used in integrated circuits due to its structure.
Which one of the following is its key feature?
A) PN junction               B) Insulated gate          C) No terminals                D) High current

- FET technology contributes to modern devices such as computers and communication systems.  Which one of the following best explains this role?
A) Reduces efficiency                           B) Enables compact and efficient circuits
C) Increases size                                   D) Blocks signals

- A transistor has a voltage gain of 100 V/V and a current gain of 1000 A/A. Calculate the power gain in dB.

A) 30 Db              B) 40 dB                         C) 50 dB                      D) 60 dB

- A transistor has a collector resistor RC = 2 kΩ, and the collector current IC = 2 mA. Calculate the collector voltage VCE if VCC = 12 V and RE = 1 kΩ.

A) 6 V         B) 8 V                C) 4 V                  D) 2 V

- Explain the difference between saturation and active regions of a BJT. Which region is used for switching applications?

A) Active for switching, saturation for amplification
B) Saturation for switching, active for amplification
C) Cutoff for amplification, saturation for switching
D) Saturation for both switching and amplification

- Explain the purpose of coupling capacitors in multistage amplifiers.

A) Block DC, pass AC signals                       B) Pass DC, block AC
C) Increase gain only                                     D) Reduce output impedance

- A cascade amplifier combines common-emitter and common-base stages. Identify its main advantages.

A) High gain, high input resistance, reduced Miller effect
B) Low gain, low input resistance
C) High input resistance only
D) Low output resistance only

- A multistage amplifier has midband gain 100 V/V and lower cutoff frequency 50 Hz, upper cutoff 20 kHz. Calculate the bandwidth.

A) 19.95 kHz                B) 20 kHz                 C) 50 Hz             D) 19.9 kHz

- Explain why RC-coupled amplifiers are widely used for audio frequency amplification.

A) Simple design, good midband gain, reasonable bandwidth
B) High gain only
C) Low input impedance only
D) Expensive, complex

- Multistage amplifiers are widely used in communication systems.
Which one of the following is the main reason?
A) Low gain
B) High amplification of weak signals
C) Large size
D) High loss

- Recording observations from diode, BJT, and FET simulations before labs reflects:
A) Motivation and appreciation of the learning process
B) Random activity
C) Ignorance
D) Low engagement

- Which statement shows motivation toward lifelong learning in electronics?
A) A student keeps trying new EDA simulations independently
B) A student copies classmates’ results
C) A student avoids new tools
D) A student skips labs

- Which of the following statements is TRUE about analog and digital signals?

A. Analog signals are discrete, while digital signals are continuous

B. Digital signals are more susceptible to noise than analog signals

C. Analog signals can take infinite values within a range

D. Digital signals cannot be processed by computers

- Which of the following decimal numbers will have the same representation in both BCD and binary?

A. 9                   B. 10                     C. 12                                        D. 15

- Which of the following statements about error detection codes is FALSE?

A. Parity bits can detect single-bit errors

B. Error detection codes can always correct errors

C. Redundancy is added to detect errors

D. They improve data reliability

- Which of the following statements is TRUE about universal gates?

A. They can implement only basic gates

B. They cannot implement XOR

C. They can implement any Boolean function

D. They are faster than all other gates

- Which expression is true for a two input Exclusive-NOR Gate

A. F (A,B)=A⊕B

B. F (A,B)= A⊕B’

C. F (A,B)= A’⊕B

D. F (A,B) = AB + A’B’

- A combinational logic circuit for traffic control is designed. Which Gate is suitable to implement the designed control signal without any additional Gates

A. AND

B. EXOR

C. NOT

D. NAND

- Which circuit is used to convert binary information from 2ⁿ inputs to n outputs?

A. Decoder

B. Encoder

C. Multiplexer

D. Adder

- One of the following is application of Flip Flops

A. Temporary storage of binary data in digital systems ✅
B. Amplification of analog signals in communication circuits
C. Conversion of analog signals to digital signals
D. Generation of continuous sinusoidal waveforms

- How many select lines are required for an 8-to-1 MUX?

A. 2                        B. 3                        C. 4                         D. 8

- A basic latch is built using:

A. XOR or XNOR gates

B. NAND or NOR gates

C. NAND only

D. NOT only

- A sequential circuit differs from a combinational circuit because it:

A. Has no inputs

B. Depends only on present inputs

C. Depends on present inputs and past outputs

D. Has no outputs

- A JK flip-flop eliminates the invalid state of:

A. SR flip-flop

B. D flip-flop

C. T flip-flop

D. NAND gate

- What is the effect of asynchronous inputs in flip-flops?

A. Works only with clock edge

B. Forces output immediately regardless of clock

C. Stores data permanently

D. Increases propagation delay only

- A ripple counter is called asynchronous because:

A. All flip-flops are clocked together

B. Only the first flip-flop receives the clock directly

C. It uses no clock signal

D. It is faster than synchronous counters

- The modulus of a counter refers to:

A. Number of flip-flops used

B. Number of inputs

C. Number of states before repetition

D. Clock frequency

- A BCD counter is also known as:

A. MOD-8 counter

B. MOD-10 counter

C. MOD-16 counter

D. MOD-2 counter

- To serially shift a byte of data into a shift register, there must be:

A. One clock pulse

B. One load pulse

C. Eight clock pulses

D. One clock pulse for each 1 in the data

E. Eight clock pulse for each 1 in the data

- What will be the value of a 4 bit register which contains 1011 after shifting 5 times to the right with serial input 1110?

A. 1110                 B. 0000                        C. 0101                          D. 0111

- Analyze the synchronous counter in Figure below and determine the counter’s modulus

A. Mod - 10

B. Mod - 12

C. Mod - 13

D. Mod - 6

- Using Boolean Algebra techniques, simplify Y = AB + (AC)' + AB'C (AB+C)

A.AB                       B.1                               C.0                       D.A’ + C’

- If the distance between two charges is doubled, the force becomes:

A. Half

B. One-fourth

C. Double

D. Four times

- Electric field intensity is defined as:

A. Force per unit charge

B. Energy per unit charge

C. Voltage per unit current

D. Power per unit area

- A 10 C charge moves through 5 V. Energy transferred is:

A. 2 J

B. 50 J

C. 0.5 J

D. 15 J

- According to Faraday’s law, induced EMF depends on:

  - Magnetic flux only

  - Rate of change of flux

  - Coil resistance

  - Current direction

- Mutual inductance exists when:

  - Two coils are isolated

  - Magnetic flux links two coils

  - Current is constant

  - Voltage is zero

- An ideal voltage source has:

A. Infinite internal resistance

B. Zero internal resistance

C. Variable resistance

D. No voltage

- A real current source is modeled as:

  - Ideal source only

  - Ideal source in series with resistance

  - Ideal source in parallel with resistance

  - Pure resistance

- Dependent sources are controlled by:

  - External battery

  - Circuit variables

  - Temperature

  - Frequency only

- KCL is based on conservation of:

  - Energy

  - Charge

  - Power

  - Voltage

- In a loop with only resistors and DC sources, KVL implies:

  - Voltage sum is zero

  - Current sum is zero

  - Power is zero

  - Resistance is zero

- Star–Delta conversion is used to:

  - Increase power

  - Simplify complex networks

  - Reduce voltage

  - Remove sources

- In nodal analysis, unknowns are:

  - Currents

  - Voltages

  - Power

  - Resistance

- Superposition theorem fails when:

  - Linear elements exist

  - Only one source exists

  - Nonlinear elements exist

  - AC sources exist

- Thevenin resistance is found by:

  - Removing load

  - Deactivating sources

  - Shorting output

  - Measuring voltage

- RMS value of sine wave equals:

  - Peak

  - Peak / √2

  - Peak / 2

  - √2 × Peak

- In inductive circuits, current:

  - Leads voltage

  - Lags voltage

  - Is in phase

  - Is zero

- Resonance occurs when:

  - XL = XC

  - R = 0

  - Current is zero

  - Voltage is zero

- Power factor correction uses:

  - Inductors

  - Capacitors

  - Resistors

  - Transformers

- Apparent power unit is:

  - Watt

  - VAR

  - VA

  - Joule

- Time constant of RC circuit is:

  - R/C

  - RC

  - L/R

  - 1/RC

- In RL circuit, current growth is:

  - Linear

  - Exponential

  - Sinusoidal

  - Constant

- At t = 0⁺, capacitor behaves as:

  - Open circuit

  - Short circuit

  - Inductor

  - Source

- At steady state (DC), inductor behaves as:

  - Open circuit

  - Short circuit

  - Capacitor

  - Source

- In balanced 3-phase system, total power is:

  - V × I

  - √3 VL × I L cosφ

  - 3 VL × I L

  - V²/R

- Which statement best explains why feedback improves a control system?

- It increases system gain

- It reduces sensitivity to disturbances

- It eliminates noise completely

- It reduces system order

- A washing machine operates on a fixed timer without sensing load conditions. What is the main disadvantage of this system?

A) High accuracy

B) No need for sensors

C) Inability to correct errors

D) Reduced cost

- If the transfer function of a system is G(s) = 10/(s+2), what is the time constant?

A) 10

B) 2

C) 0.5

D) 5

- Two systems have transfer functions:

G1(s) = 5/(s+1), G2(s) = 5/(s+5)

Which system responds faster and why?

A) G1, because gain is higher

B) G2, because pole is farther left

C) G1, because pole is closer to origin

D) Both same

- Why are initial conditions assumed zero when defining transfer functions?

A) To simplify algebra

B) Because systems always start at zero

C) To focus on system dynamics only

D) To increase stability

- If damping ratio ζ = 0, the system response will be:

A) Overdamped

B) Critically damped

C) Undamped oscillatory

D) Stable without oscillation

- A system shows large overshoot and oscillations. What change will improve performance?

A) Increase gain

B) Increase damping ratio

C) Reduce feedback

D) Remove controller

- If settling time is too large, which parameter should be adjusted?

A) Increase system speed (bandwidth)

B) Reduce gain to zero

C) Increase delay

D) Remove poles

- A system has one pole in the right half-plane and two in the left. The system is:

A) Stable

B) Marginally stable

C) Unstable

D) Critically stable

- Using Routh-Hurwitz, if a sign change occurs in the first column, it indicates:

A) Stable system

B) Number of poles in left half-plane

C) Number of poles in right half-plane

D) System gain

- Root locus helps a designer to:

A) Solve differential equations

B) Predict system stability as gain varies

C) Measure noise

D) Calculate power

- If root locus crosses into the right half-plane as gain increases, the system becomes:

A) More stable

B) Unstable

C) Faster

D) Critically damped

- If phase margin is negative, the system is:

A) Stable

B) Marginally stable

C) Unstable

D) Critically damped

- A system has high gain margin but low phase margin. This indicates:

A) High stability and no oscillations

B) System may oscillate easily

C) No effect on stability

D) Infinite bandwidth

- Which controller action reduces steady-state error but may increase overshoot?

A) Proportional

B) Integral

C) Derivative

D) Feedforward

- A transformer works on the principle of:

A) Self-induction        B) Mutual induction      C) Capacitance    D) Resistance

- Torque of an induction motor is proportional to:

A) Voltage      B) Current    C) Slip (at low slip)    D) Frequency    E) Resistance

- Efficiency of a transformer is maximum when:

A) Copper loss = Core loss               B) Copper loss > Core loss   C) Copper loss < Core loss                                       D) Core loss is zero

- The emf generated in a DC generator depends on:

A) Flux   B) Speed    C) Number of conductors      D) All of the above

- The frequency of the secondary voltage in a transformer is:

A) Higher than primary B) Lower than primary   C) Same as primary D) Zero

- Which of the following motor has high starting torque?

A) Squirrel cage motor B) Slip ring motor   C) Synchronous motor D) DC shunt motor

- The rotor of an induction motor always rotates:

A) At synchronous speed      B) Above synchronous speed      C) Below synchronous speed
 D) At zero speed

- The relation between B and H is:

A) B = H/μ          B) B = μH           C) B = H²               D) B = μ/H         E)     B=H

- Transformer oil is used for:

A) Cooling only                   B) Insulation only                 C) Both cooling and insulation
D) Reducing noise

- Hysteresis loss occurs due to:

A) Reversal of magnetization   B) Eddy currents   C) Resistance   D) Capacitance

- In a DC motor, the direction of force is given by:

A) Fleming’s Right-Hand Rule   B) Ohm’s Law C) Lenz’s Law
D) Faraday’s Law    E) Fleming’s Left-Hand Rule

- In a DC shunt motor, field winding is connected:

A) In series with armature   B) In parallel with armature   C) Separately
D) With load

- The air gap in electrical machines is important for:

A) Cooling only       B) Magnetic coupling between stator and rotor   C) Increasing resistance
D) Reducing voltage      E) Increase speed

- The force on a current-carrying conductor in a magnetic field is given by:

A) Fleming’s Right-Hand Rule   B) Lenz’s Law C) Fleming’s Left-Hand Rule 
D) Coulomb’s Law       E) Lorentz’s Law

- Back EMF in a DC motor:

A) Aids supply voltage          B) Is zero        C) Opposes supply voltage

D) Is constant          E) Aids Current

- According to Faraday’s Law, induced EMF is proportional to:

A) Current B) Resistance C) Rate of change of magnetic flux D) Voltage E) frequency

- In electrical machines, energy conversion is based on interaction between:

A) Voltage and resistance    B) Magnetic field and electric current   C) Current and resistance
D) Power and voltage                E) Torque and current

- Which DC motor has nearly constant speed?

A) Series motor B) Shunt motor C) Compound motor   D) Universal motor E) All

- Stator of induction motor produces:

A) DC field    B) Pulsating field    C) Rotating magnetic field    D) Static field

- A generator converts:

A) Electrical energy to mechanical energy                B) Chemical energy to electrical energy
C) Heat to electrical energy                     D) Mechanical energy to electrical energy

- Which element in a generalized measurement system is responsible for converting the physical variable into an equivalent electrical signal?

- Variable Manipulation Element

- Primary Sensing Element

- Data Presentation Element

- Signal Conditioning Element

- An instrument's ability to reproduce the same output for a constant input under identical conditions is defined as:

- Resolution

- Sensitivity

- Precision

- Accuracy

- Calibration of an instrument is essential because it ensures:

- The instrument is safe to use

- The instrument's battery is full

- The cost of the instrument is justified

- Traceability to national or international standards

- In a capacitive sensor, if the distance between the plates (d) is halved, what happens to the capacitance (C)?

- It remains the same

- It quadruples

- It is halved

- It doubles

- A Platinum RTD (Pt100) has a resistance of 100Ω at 0 ∘C. If the temperature coefficient is 0.00385/ ∘C, what is its resistance at 100 ∘C?

- 200Ω

- 110Ω

- 103.85Ω

- 138.5Ω

- To measure the flow of a non-conductive, clean liquid with high accuracy and no pressure drop, which sensor is most appropriate?

- Venturi Meter

- Orifice Plate

- Ultrasonic (Transit-time) Meter

- Electromagnetic Flow Meter

- An Instrumentation Amplifier is preferred for sensor signal conditioning because it has:

- Low input impedance

- Variable power supply

- Single-ended inputs

- High Common-Mode Rejection Ratio (CMRR)

- Which digital code is used in absolute optical encoders to prevent errors during bit transitions?

- Standard Binary Code

- Gray Code

- BCD (Binary Coded Decimal)

- ASCII

- A 10-bit ADC has a reference voltage of 5V. What is its resolution in terms of voltage?

- 0.5 V

- 4.88 mV

- 1.22 mV

- 50 mV

- Which of the following is a 'Dynamic Characteristic' of a measurement system?

- Linearity

- Dead Zone

- Hysteresis

- Time Constant

- A LVDT (Linear Variable Differential Transformer) is used to measure:

- Light Intensity

- Temperature

- Flow Rate

- Displacement

- Which sensor type requires an external power source to operate?

- Thermocouple

- Strain Gauge

- Piezoelectric Crystal

- Photovoltaic Cell

- When measuring the level of a corrosive liquid in a tank, which method provides the best isolation?

- Float and Tape

- Ultrasonic (Non-contact)

- Hydrostatic Pressure (Bubbler)

- Displacer

- A measurement of a constant voltage is taken 10 times. The readings are tightly clustered but are all exactly 0.5V higher than the true value. This instrument is:

- High precision, low accuracy.

- High accuracy, low precision.

- Low accuracy, low precision.

- Perfectly calibrated.

- Which of the following is an "Absolute" instrument?

- A Digital Multi-meter.

- A Tangent Galvanometer.

- A Bourdon Tube Pressure Gauge.

- A Thermistor.

- Which output presentation element eliminates "Parallax Error"?

- D'Arsonval Meter with a mirrored scale.

- Chart Recorder with a felt-tip pen.

- Digital LCD Display.

- Analog Oscilloscope.

- A digital multi-meter consistently shows a reading that is 0.05 V higher than the true value every time it is used. How should this error be classified and corrected?

- It is a Gross Error; it is caused by human oversight and cannot be corrected.

- It is a Hysteresis Error; it depends on whether the thickness is increasing or decreasing.

- It is a Systematic Error; it can be corrected by calibrating the instrument or applying an offset.

- It is a Random Error; it can be reduced by taking the average of many readings.

- A thermocouple (Type K) is connected to a remote PLC. If the 'Cold Junction' at the PLC terminal block rises from 20∘C to 30∘C but the PLC software does not account for this, how will the reported temperature change?

- The reported temperature will increase by 10∘C.

- There will be no change because thermocouples are 'active' sensors.

- The PLC will trigger an 'Open Burn' alarm due to the impedance shift.

- The reported temperature will decrease by approximately 10∘C.

- Scenario: An Ultrasonic Level Sensor is installed at the top of a sealed tank containing pressurized Nitrogen gas. The liquid level is constant, but as the gas pressure is increased during a filling cycle, the sensor reports that the tank is getting "emptier" (the calculated distance increases). Question: Why is the sensor providing an inaccurate reading?

- The high pressure is physically compressing the ultrasonic transducer.

- The speed of sound increases as gas density/pressure increases, causing the pulse to return faster than the "stored" speed value.

- Nitrogen gas absorbs sound waves, causing the echo to disappear entirely.

- The liquid surface becomes a perfect absorber of sound under high pressure.

- Scenario: You are installing a temperature control system for a pharmacy store. The cooling system turns ON at 22.0 ∘C and OFF at 21.9 ∘C. You notice the compressor is "chattering" (turning on and off every few seconds), which will eventually destroy the motor. Question: Which change to the Discrete Output Sensor logic will protect the equipment?

- Increase the resolution of the temperature sensor to 0.001∘C.

- Implement Hysteresis (Dead band) so the system turns ON at 23∘C and OFF at 20∘C.

- Replace the digital sensor with an analog thermometer.

- Increase the amplifier gain so the signal reaches the threshold faster

- Method of computing unique solutions for a given  network is

- Network topology                              C. Network analysis

- Network theory                                   D. Network synthesizes

- Which network model( type) is applicable in power transmission line analysis

- Linear network                                      C. Lumped network

- Non- linear network                               D. Distributed network

- Network analysis and synthesis is usually done in

- Time domain                                           C. Laplace domain

- Frequency domain                                   D. Z-domain

- If the excitation and response of a network are measured at the same set of terminals is

- Deriving point function                          C. Transformed  function

- Transfer function                                     D. Multi point function

- The driving-point impedance Z(s) of a network has a pole-zero locations as shown below If Z(0) = 3, then Z (s) is

A.                                                       C.

B.                                                         D.

- Two important conditions in network synthesis are

- Linearity and causality                                C. Causality and stability

- Linearity and stability                                  D. None-linearity and stability

- What is the range of ‘K’ for the following polynomial function to be stable

- B.                 C.               D.

- Which one of the following is true?

- RL  impedance is the same Pole –Zero location as RC impedance

- RL  impedance is the same  Pole –Zero location as RC admittance

- RL  impedance is the same pole-Zero location as LC impedance

- RL admittance is the same pole –Zero location as RC admittance

- The two port networks are connected in cascade, the combination is to be represented as a single two-Port network. The parameters of the network are obtained by multiplying the individual matrix

- Z-parameter                                                  C. Y-parameter

- h-parameter                                                  D.ABCD parameter

- For the network given below, the value of the Z12 parameter is 
            
  A.  Z12 = 1 Ω       B.    Z12 = 4 Ω        C.    Z12 = 1.667 Ω                D.Z12 = 2.33 Ω

- Rate change of amplitude with frequency(or attenuation rate)is called

- Envelope delay          B. Roll off         C Ripple            D. Cutoff frequency

- A filter that passes frequencies between two designated cut-off frequencies and attenuates all other frequencies is called?

A. High pass filter                                             C. Band pass filter

B. Band elimination filter                                 D. All pass filter

- Filters with which of the following characteristic provide a very flat amplitude in the pass band and a roll off rate of 20 dB/decade/pole
A. Butterworth
B. Chebyshev
C. Bessel
D. Elliptic

- Transfer function Z(s) of a capacitor in frequency domain is

    - Z(s) = R

    - Z(s) = jwC

    - Z (s) = 1/(jwC)

    - Z(s) = 1/ C

- What is a filter?

- Frequency selective circuit

- Amplitude selective circuit

- Frequency damping circuit

- Both Amplitude and frequency selective circuit

- A network function has zeros only in the left-half of the S-plane, then it is said to be

- Stable function.

- Non-minimum phase function.

- A minimum phase function.

- all-pass function

- A nonlinear network does not satisfy

- Superposition condition only

- Homogeneity condition only

- Both homogeneity as well as Superposition condition

- Homogeneity , Superposition  and associative condition

- What type of filter the figure shown below and its cutoff frequency is

- high-pass, 21 Hz                     C. high-pass, 2.65 kHz

- low-pass, 21 Hz                       D. low-pass, 2.65 kHz

- Which one is true for a network realization technique matched

- Brune's realisation for non - ideal transformer.

- Cauer realisation: ladder realisation.

- Bott Duffin realisation for  non- ideal transformer

- Foster realization is based on partial fraction expansion

- Hybrid parameter of h12 in a two – port network used as?

- A voltage source at output port

- A current source at output port

- A voltage source at input port

- A current source at input port

- Which concept allows a subclass to provide a specific implementation of a method already defined in its parent class?

A. Overloading
B. Overriding
C. Encapsulation
D. Abstraction

- Which block in Java is executed only once when the class is loaded?

A. Instance block
B. Constructor
C. Static block
D. Method

- What is the default value of an instance variable of type int in Java?

A. 1
B. 0
C. null
D. Undefined

- Which keyword is used to inherit a class in Java?

A. implement
B. extends
C. inherits
D.  super

- Which concept allows one interface to be used for different data types?

A. Encapsulation
B. Abstraction
C. Polymorphism
D. Inheritance

- Which of the following is NOT a feature of OOP?

A. Encapsulation
B. Inheritance
C. Compilation
D. Polymorphism

- What will happen if a method is declared as final?

A. It can be overridden
B. It cannot be overridden
C. It must be overridden
D. It becomes abstract

- Which access modifier makes a member visible only within the same class?

A. public
B. protected
C. private
D.  default

- What is constructor overloading?

A. Using one constructor in many classes
B. Defining multiple constructors with different parameters
C. Overriding constructor in subclass
D. Hiding constructor

- Which constructor is called first when an object of a subclass is created?

A. Subclass constructor
B. Parent class constructor
C. Constructor of the first imported class
D. Default constructor of Object class

- Which of the following is used to display output in C++?

A. cin
B. cout
C. print
D. output

- Which of the following is a correct comment in C++?

A. // This is a comment
B. <!-- comment -->
C. # comment
D. ** comment **

- Which keyword is used to define a class in C++?

A. struct
B. class
C. define
D. object

- Which of the following is the correct syntax to declare a constant in C++?

A. const int x = 10;
B. int const = 10;
C. constant int x = 10;
D. int x const = 10;

- Which operator is used to access members of a class through an object?

A. ->
B. ::
C. .
D. *

- Which of the following is used to take input from the user in C++?

A. cout
B. cin
C. input
D. scan

- Which symbol is used to terminate a statement in C++?

A.  :
B.  .
C.  ;
D.  ,

- Which of the following is NOT a valid access specifier in C++?

A. public
B. private
C. protected
D. internal

- Which of the following correctly defines a function in C++?

A. function myFunc()
B. def myFunc()
C. void myFunc()
D. func myFunc()

- Which keyword is used to create an object in C++?

A. class
B. new
C. object
D. create

- The continuous time system described by y(t)=3x(t-2)+4 comes under which

category

- causal, linear and time varying

- causal, non-linear and time- invariant

- non causal, non-linear and time-invariant

- non causal, linear and time-variant

- Fourier transform of rectangular function gives sinc function and vice versa. This property of Fourier transform is

- Shifting

- Scaling

- Duality

- Linearity

- What is the Nyquist frequency for the signal   st=2cos80πt+sin400πt-cos(200πt) ?

- 80 Hz

- 200 Hz

- 800 Hz

- 400 Hz

- Which of the following is not correct?

- un*un=n+1

- un*δn-δn-1=δ(n)

- Ƒδn-δn-1=1-e-jω

- ʆ(u(t))=1s2

- Which transform method is mainly used to simplify the analysis of equations involving differential and integral operations is

- Fourier Transform

- Hilbert Transform

- Laplace Transform

- Z-Transform

- A Discrete signal is said to be even or symmetric if X(-n) is equal to

    - x(n)

    - 0

    - –x(n)

    - –x(-n)

- The frequency response of a system with hn=δ(n-1) is given by

- δ(ω-1)

- e-jω

- u(ω-1)

- ejω

- If y(t)=x(t)*h(t), then which of the following is not true?

- Y(s)=X(s)H(s)

- yt=-∞∞xτhtdτ

- yt=-∞∞hτxt-τdτ

- Y(ω)=X(ω)H(ω)

- The function which has its Fourier transform, Laplace transform, and Z transform unity is

  - Gaussian.

  - impulse.

  - sinc.

  - pulse

- Consider a causal system with impulse response h[n]=2nu[n] .If x[n] is the input and y[n] is the output to this system, then which of the following difference equation describes the system?

- y[n]+2y[n+1]=x[n]

- y[n]-2y[n-1]=x[n]

- y[n]+2y[n-1]=x[n]

- y[n]-12y[n-1]=x[n]

- The region of convergence of z-transform of the sequence

xn=14nun--15nu-n-1

- z>14

- z<15

- 15<z<14

- ∣z∣=1/4

- Which property of Fourier transform gives rise to an additional phase shift of -2πftd for the generated time delay in the communication system without affecting an amplitude spectrum?

a) Time Scaling

b) Linearity

c) Time Shifting

d) Duality

- Convolution is used to find

- Multiplication of signals

- Response of the system

- Amount of similarity between signals

- Difference between signals

- Where is the ROC defined or specified for the signals containing causal as well as anti-causal terms?

a) Greater than the largest pole

b) Less than the smallest pole

c. Between two poles

d. Cannot be defined

- Unilateral Laplace Transform is applicable for the determination of linear constant coefficient differential  equations with

a) Zero initial condition

b)  Non-zero initial condition

c)  Zero final condition

d) Non-zero final condition

- What should be location of poles corresponding to ROC for bilateral Inverse Laplace Transform especially for determining the nature of time domain signal?

a) On L.H.S of ROC

b)  On R.H.S of ROC

c)  On both sides of ROC

d) Outside ROC

- The property of Fourier Transform which states that the compression in time

domain is equivalent to expansion in the frequency domain is

- Duality

- Scaling

- Time shifting

- Frequency shifting

- The convolution between impulse  δ(n) and unit step u(n) sequences  is given by

- δ(n)

- u(n)

- r(n)

- sgn(n)

- Most signals found in nature are

- continuous and discrete

- digital and analog

- continuous and random

- continuous and digital

- Steps to Modern filter design includes:

- a) Selection of filter specification

- b) Approximation

- c) Realization

- d) Verification

- Which of the following are the two primary resources of a communication system?

A. Antennas and filters

B. Bandwidth and power

C. Modulators and demodulators

D. Amplifiers and cables

- Which of the following is not true about amplitude modulation techniques?

A. DSB-SC transmission removes unnecessary carrier power

B. SSB is more appropriate for voice transmission than video

C. SSB has more transmission bandwidth than DSB-SC

D. Standard AM system is simple in construction

- A message signal (𝑡) with a bandwidth of B = 30kHz is passed through the system shown in the block diagram below. The carrier frequency is 𝑓𝑐 (where𝑓𝑐 ≫ 𝐵).What is the bandwidth of the resulting output signal 𝒔4(𝑡)?

A. 40 KHz C. 15 KHz

B. 60 KHz D. 30 KHz

- Which of the following statements is incorrect regarding frequency modulation (FM)?

A. Increasing the modulation index β increases the number of significant sidebands

B. The total transmitted power of an FM signal increases with modulation index β

C. The frequency deviation in FM is directly proportional to the amplitude of the modulating signal

D. In FM, the carrier amplitude remains constant during modulation

- A super heterodyne FM receiver uses an intermediate frequency of 10.7 MHz and is tuned to an FM broadcast station at 98.5 MHz. If the receiver uses high-side injection (𝑓𝐿𝑂 > 𝑓𝑠), the associated image frequency is:

A. 87.8 MHz

B. 109.2 MHz

C. 119.9 MHz

D. 125.4 MHz

- Which digital modulation technique is more susceptible to noise?

A. FSK

B. PSK

C. ASK

D. PAM

- A conventional amplitude modulation (AM) system transmits a signal with a total power 𝑃𝑡. If the modulation index 𝜇 = 1 and the total transmitted power is 100 Watts, how much power is actually utilized to carry the information?

A. 1 Watt

B. 33 Watts

C. 66 Watts

D. 100 Watts

- In a Pulse Code Modulation (PCM) system, a message signal (𝑡) = 2cos(100𝜋𝑡) is sampled at the Nyquist rate. The resulting samples are then processed by a uniform quantizer with a step size of 0.8 V. what is the minimum data rate of the PCM system in bits per second?

A. 100

B. 200

C. 300

D. 400

- Which parameter does affect the transmission power of FM ?

A. Frequency of carrier

B. Frequency of information signal

C. Amplitude of carrier

D. Amplitude of information signal

- A message signal s(t) = cos2000πt + 4cos4000πt modulates the carriers c(t) = cos2πfct where fc = 1MHz to produce an AM signal. For demodulating the generated AM signal using an envelope detector, the time constant RC of the detector circuit should satisfy

A. 0.5 ms < RC < 1 ms

B. 1 µs << RC < 0.5 ms

C. RC << 1 µs

D. RC >> 0.5 ms

- Which of the following is true about quantization error in a PCM system?

A. It increases with increase in number of quantization levels

B. It decreases with increase in number of quantization levels

C. It is independent of quantization levels

D. It depends only on sampling frequency

- Vestigial Sideband (VSB) modulation is particularly suitable for:

A. Radio broadcasting

B. Television video transmission

C. Audio transmission

D. Satellite telemetry

- The primary function of Automatic Gain Control (AGC) in a receiver is to:

A. Maintain a constant output signal level despite variations in input signal strength

B. Automatically tune the local oscillator frequency

C. Reduce image frequency interference

D. Demodulate the carrier

- Which one of the following is a primary advantage of Angle Modulation (FM/PM) over Amplitude Modulation (AM)?

A. Reduced bandwidth requirements

B. Simple receiver circuit design

C. Better discrimination against noise and interference

D. Lower power consumption at the transmitter

- A band-limited signal (𝑡) has a maximum frequency component of 𝑊 Hz. According to the Nyquist sampling theorem, what is the minimum sampling rate required to avoid aliasing?

A. W samples/sec

B. 2W samples/sec

C.W/2samples/sec

D. 4W samples/sec

- Which of the following receiver performance metrics measures the ability of a communication system to detect weak signals just above the noise floor?

A. Selectivity

B. Fidelity

C. Sensitivity

D. Noise Figure

- Compare PAM, PWM, and PPM in terms of noise immunity. Which statement is correct?

A. PAM has the best noise immunity because amplitude is easy to detect

B. PWM and PPM have better noise immunity than PAM because information is in pulse timing, not amplitude

C. All three have identical noise immunity

D. PPM has the worst noise immunity

- In a superheterodyne receiver, why is the incoming RF signal combined with the Local Oscillator (LO) signal in the mixer stage?

A. To increase the signal bandwidth before it reaches the detector.

B. To allow for consistent gain and selectivity at a fixed frequency.

C. To eliminate the need for an RF front-end amplifier stage.

D. To directly convert the modulated signal into an audio output for the speaker.

- Compare the bandwidth efficiency and implementation complexity of ASK, FSK, and PSK.Which statement accurately reflects their trade-offs?

A. PSK has the best bandwidth efficiency but the most complex demodulation

B. ASK has the best bandwidth efficiency and is the most complex

C. FSK has the best bandwidth efficiency but is the simplest

D. All three have identical bandwidth efficiency

- Which of the following is an advantage of analog communication systems over digital communication systems?

A. Less transmission bandwidth is required

B. Allows integration of voice, video, and data within a single system

C. Hardware implementation is flexible

D. provides better noise immunity and allows signal regeneration

- In a large interconnected power system, AC transmission is still widely used despite the advantages of HVDC. Which of the following BEST justifies this choice?

A. AC systems inherently have lower transmission losses than HVDC

B. AC voltage can be easily stepped up or down using transformers, enabling flexible system

integration

C. AC systems do not require reactive power compensation

D. AC transmission eliminates stability issues

- In an interconnected power system, economic operation is mainly achieved due to:

A. Elimination of all reserve requirements across interconnected areas

B. Simultaneous peak demand occurring in all areas

C. Sharing of load and generation, reducing the need for installed reserve capacity in each

area

D. Reduction of transmission line impedance

- Consider the parameters of an overhead transmission line. Which of the following statements is MOST accurate?

A. Line resistance is mainly influenced by conductor spacing and geometry

B. Inductance depends only on conductor material and temperature

C. Shunt conductance arises due to leakage currents over insulators and atmospheric  conditions

D. Capacitance depends only on conductor cross-sectional area

- A transmission line section passes through a river crossing and is subjected to high mechanical tension and angular deviation. Which insulator type is MOST suitable?

A. Pin insulator

B. Suspension insulator

C. Shackle insulator

D. Strain insulator

- Which of the following statements is INCORRECT regarding transmission line performance?

A. In medium transmission lines, shunt capacitance is modeled as lumped elements

B. Voltage regulation is affected by load power factor

C. Inductive loads operating at leading power factor can cause negative voltage regulation

D. Transmission efficiency is mainly affected by I²R losses

- Corona has both advantages and disadvantages in transmission lines. Which option BEST identifies a beneficial effect of corona?

A. Reduction of transmission efficiency

B. Attenuation of high-frequency surges and lightning effects

C. Increase in audible noise and radio interference

D. Increase in power losses

- During transmission line design, a sag template is primarily used to:

A. Ensure zero sag between supports

B. Determine optimal tower locations considering clearance, terrain, and conductor sag

C. Measure mechanical strength of conductors

D. Eliminate temperature effects on conductors

- If the height of transmission towers is increased while keeping conductor spacing constant, what is the MOST likely effect on line parameters?

A. Capacitance increases due to higher electric field

B. Capacitance decreases due to increased distance from ground

C. Inductance decreases significantly

D. Resistance increases due to longer conductor length

- Corona loss in transmission lines is influenced by multiple factors. Which condition will MOST likely increase corona loss?

A. Large conductor diameter and low frequency

B. Small conductor diameter and high operating voltage

C. Low voltage and large conductor spacing

D. High conductor diameter and smooth surface

- Which statement correctly describes power behavior in AC circuits?

A. In a purely resistive circuit, current leads voltage by 90°

B. In a purely inductive circuit, current leads voltage by 90°

C. A capacitor absorbs real power continuously with non-zero average

D. In a purely capacitive circuit, current leads voltage by 90°

- For the same power transfer, increasing transmission voltage by 2 times will reduce line losses approximately by:

A. 2 times

B. 4 times

C. 8 times

D. No change

- A transmission line delivers 100 MW at 0.8 lagging power factor. Improving power factor to unity will:

A. Increase current

B. Decrease current and losses

C. Increase losses

D. Not affect system performance

- Which method is MOST effective for improving voltage profile in long transmission lines?

A. Series resistance addition

B. Shunt compensation

C. Increasing load

D. Reducing conductor size

- The Ferranti effect becomes significant when:

A. Line is heavily loaded

B. Line is short

C. Line is long and lightly loaded

D. Frequency is zero

- Which of the following BEST explains skin effect?

A. Current flows uniformly across conductor

B. Current concentrates at center

C. Current concentrates near surface at high frequency

D. Resistance decreases with frequency

- A power system engineer wants to reduce transmission losses and improve stability for long-distance bulk power. The BEST choice is:

A. Low voltage AC

B. High current AC

C. HVDC transmission

D. Increase conductor resistance

- Load factor is an indicator of:

A. Peak demand only

B. Efficiency of utilization of installed capacity

C. Voltage level

D. Frequency variation

- Which parameter mainly determines surge impedance loading (SIL)?

A. Resistance

B. Inductance and capacitance

C. Voltage only

D. Frequency only

- Increasing conductor spacing will:

A. Increase capacitance

B. Decrease inductance

C. Decrease capacitance

D. Increase resistance

- Which factor has the LEAST effect on transmission line resistance?

A. Conductor material

B. Temperature

C. Length of line

D. Distance between conductors

Answer Key

| 1 | B | 11 | B | 21 | A | 31 | B | 41 | B | 51 | A | 61 | B | 71 | C | 81 | B | 91 | D | 101 | B | 111 | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | C | 12 | C | 22 | C | 32 | C | 42 | B | 52 | B | 62 | B | 72 | B | 82 | C | 92 | B | 102 | C | 112 | B |
| 3 | B | 13 | A | 23 | A | 33 | A | 43 | A | 53 | B | 63 | B | 73 | A | 83 | A | 93 | B | 103 | D | 113 | B |
| 4 | B | 14 | B | 24 | B | 34 | B | 44 | B | 54 | C | 64 | B | 74 | C | 84 | D | 94 | C | 104 | D | 114 | A |
| 5 | B | 15 | A | 25 | C | 35 | B | 45 | B | 55 | B | 65 | B | 75 | C | 85 | C | 95 | C | 105 | D | 115 | B |
| 6 | B | 16 | A | 26 | D | 36 | C | 46 | B | 56 | B | 66 | B | 76 | B | 86 | B | 96 | C | 106 | C | 116 | C |
| 7 | A | 17 | A | 27 | B | 37 | B | 47 | B | 57 | B | 67 | C | 77 | B | 87 | C | 97 | B | 107 | D | 117 | C |
| 8 | B | 18 | B | 28 | B | 38 | C | 48 | C | 58 | A | 68 | C | 78 | C | 88 | B | 98 | B | 108 | B | 118 | D |
| 9 | B | 19 | A | 29 | D | 39 | D | 49 | B | 59 | B | 69 | B | 79 | B | 89 | C | 99 | C | 109 | B | 119 | B |
| 10 | B | 20 | A | 30 | B | 40 | C | 50 | B | 60 | C | 70 | C | 80 | B | 90 | A | 100 | D | 110 | D | 120 | B |

| 121 | C | 131 | B | 141 | B | 151 | B | 161 | B | 171 | C | 181 | B | 191 | B | 201 | B | 211 | B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 122 | C | 132 | C | 142 | C | 152 | A | 162 | C | 172 | C | 182 | C | 192 | B | 202 | C | 212 | B |
| 123 | C | 133 | B | 143 | B | 153 | B | 163 | B | 173 | B | 183 | D | 193 | A | 203 | C | 213 | B |
| 124 | A | 134 | C | 144 | B | 154 | A | 164 | D | 174 | C | 184 | B | 194 | C | 204 | D | 214 | C |
| 125 | A | 135 | A | 145 | C | 155 | C | 165 | C | 175 | B | 185 | B | 195 | B | 205 | C | 215 | C |
| 126 | C | 136 | C | 146 | C | 156 | B | 166 | A | 176 | C | 186 | C | 196 | C | 206 | B | 216 | C |
| 127 | B | 137 | C | 147 | B | 157 | C | 167 | A | 177 | B | 187 | B | 197 | B | 207 | B | 217 | B |
| 128 | B | 138 | C | 148 | C | 158 | D | 168 | B | 178 | B | 188 | D | 198 | B | 208 | B | 118 | B |
| 129 | D | 139 | B | 149 | B | 159 | C | 169 | A | 179 | C | 189 | D | 199 | A | 209 | B | 219 | C |
| 130 | A | 140 | C | 150 | B | 160 | B | 170 | B | 180 | D | 190 | B | 200 | C | 210 | D | 220 | D |